3GPP TSG-RAN WG2 Meeting #120	R2-2211594

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	PDCP initialisation

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

Current PDCP specification require that initialRX-DELIV is always provided at MRB setup. Also for AM MRB, it is not possible to update the initialRX-DELIV later. This paper discusses these unnecessary restrictions and potential changes to RAN2 specifications in order to allow update of the PDCP state variables during PDCP re-establishment also for AM MRB. Then the MRB(s) could be configured with any value for initialRX-DELIV and later be updated. The required changes to RAN2 spec are discussed in this contribution.

It should be noted though that these RAN2 discussions are orthogonal to the issue which is currently being discussed in RAN3.

The MRB PDCP initialisation issue being discussed in RAN3 is in case when a first UE joins an MBS session in a gNB where no MBS packets have yet been received from CN. When a gNB wants to set up MRB(s) for a UE, current RRC and PDCP specs require that initialRX-DELIV is provided to the UE even if the gNB does not know which value to give.

Therefore, currently gNB has to wait that it receives the first MBS packets before configuring MRB(s) to the UE, especially for AM MRBs. If multiple UEs have joined the MBS session in the gNB during the deactivated duration, the activation of the MBS session will trigger RRC reconfiguration to all UEs to setup the MRB(s) at same time which is not good.  Even if RAN2 agrees that initial values can be further updated at activation time as proposed in this paper, this will still need to reconfigure all UEs at activation time which therefore doesn’t solve RAN3 issue.

RAN3 has discussed CP and UP based solutions to provide the initial PDCP COUNT to gNB in case the MBS session is not active and thus no MBS data is sent to the gNB. The benefit of these RAN3 solutions is that MRB(s) can be configured to the UEs already during the inactive period of the MBS session and there is no need to reconfigure the UEs when the MBS session is being activated.

# 2	Discussion

The following agreements have been made in RAN2 related to PDCP initialisation:

- RAN2#119 multicastHFN-AndRefSN is renamed to initialRXDELIV and update the corresponding description to ‘Indicates an initial value of RX_DELIV for multicast MRB PDCP window initialization as specified in TS 38.323 [5].’. [8] [9] [12] Add to section 16.10 (e.g. 16.10.5.3.2) in TS 38.300 a sentence saying: “For MRBs, PDCP can either be re-established or remain as it is.” [8] Allow configuration of initial value of RX_DELIV also when PDCP is re-established for UM MRB. FFS AM MRB, if a fix is needed - RAN2#119bis Do not reset RX_NEXT and RX_DELIV to the initial value when MRB PDCP is suspended unless a serious issue is found. RAN2 confirms no serious issue on “Do not reset RX_NEXT and RX_DELIV to the initial value when MRB PDCP is suspended” There is no need for configuration of initial value of RX_DELIV when PDCP is re-established for AM MRB (no specification change)

If RAN2 allows update of the PDCP state variables during PDCP re-establishment also for AM MRB, the agreement made in the previous RAN2 meeting has to be reverted. However, the initialisation of RX_NEXT and RX_DELIV for AM MRB during PDCP re-establishment can be made optional and configurable such that they are initialised only if initialRX-DELIV is provided by upper layers TS 38.331. Then it is fully up to network whether to follow current agreement (i.e., not initialisation of RX_NEXT and RX_DELIV at PDCP re-establishment for AM MRB) or request initialisation of the state variables.

Observation 1: The initialisation of RX_NEXT and RX_DELIV for AM MRB during PDCP re-establishment can be made optional and configurable such that they are initialised only if initialRX-DELIV is provided by upper layers TS 38.331

The change needed into PDCP spec 38.323 seems to be fairly simple, one bullet point should be added to section 5.1.2:

### 5.1.2 PDCP entity re-establishment When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2. When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall: ------------skipped text----- When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall: ---------------skipped text-------- - for SRBs, UM DRBs and UM MRBs, set RX_NEXT and RX_DELIV to the initial value; - for AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is provided by upper layers TS 38.331 [3]; ------------skipped text-----

And the RRC spec 38.331 would require a change in the condition MRB-Initialization:

| MRB-Initialization | This field is mandatory present in case of multicast MRB setup and PDCP re-establishment for UM multicast MRB. In case of PDCP re-establishment for AM multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |
| --- | --- |

Alternatively, if the initialisation of PDCP receiver state variables is made optional and configurable for AM MRB, it is simple to make them optional and configurable also for UM MRB.

Observation 2: If the initialisation of PDCP receiver state variables is made optional and configurable for AM MRB, it is simple to make them optional and configurable also for UM MRB.

The change needed into PDCP spec 38.323 in this case is also simple:

### 5.1.2 PDCP entity re-establishment When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2. When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall: ------------skipped text----- When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall: ---------------skipped text-------- - for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value; - for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is provided by upper layers TS 38.331 [3]; ------------skipped text-----

And the condition MRB-Initialization in the RRC spec 38.331 would require the following change:

| MRB-Initialization | This field is mandatory present in case of multicast MRB setup. In case of PDCP re-establishment for UM and AM multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |
| --- | --- |

Proposal 1: RAN2 to discuss whether to make initialisation of RX_NEXT and RX_DELIV for AM MRB during PDCP re-establishment optional and configurable such that they are initialised only if initialRX-DELIV is provided by upper layers TS 38.331.

Proposal 2: RAN2 to discuss whether to make it optional and configurable also for UM MRB.

It should be noted that this change of RAN2 spec would not solve the signalling storm issue at MBS session activation. Therefore, RAN3 should have a separate discussion on its solutions regardless of RAN2 decision.

# 3	Conclusion

This documents has made the following observations and proposals:

Observation 1: The initialisation of RX_NEXT and RX_DELIV for AM MRB during PDCP re-establishment can be made optional and configurable such that they are initialised only if initialRX-DELIV is provided by upper layers TS 38.331

Observation 2: If the initialisation of PDCP receiver state variables is made optional and configurable for AM MRB, it is simple to make them optional and configurable also for UM MRB.

Proposal 1: RAN2 to discuss whether to make initialisation of RX_NEXT and RX_DELIV for AM MRB during PDCP re-establishment optional and configurable such that they are initialised only if initialRX-DELIV is provided by upper layers TS 38.331.

Proposal 2: RAN2 to discuss whether to make it optional and configurable also for UM MRB.

It should be noted that this change of RAN2 spec would not solve the signalling storm issue at MBS session activation. Therefore, RAN3 should have a separate discussion on its solutions regardless of RAN2 decision.