3GPP TSG-RAN WG2 Meeting #120	R2-2211601

Toulouse, France, 14 – 18 November 2022	R2-2209559

Agenda item:	8.5.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	XR Capacity Enhancements

WID/SID:	FS_NR_XR_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses several aspects related to capacity enhancements for XR.

# 2	RLC Retransmissions

For capacity hungry services like XR, it is desirable to use an initial first transmission BLER target of 10-1, and if HARQ fails (for instance after one or two retransmissions) to trigger a ARQ retransmission that allows selection a new MCS for the transmission that better fits the experienced SINR conditions at the Rx. Indeed, it may e.g. happen that the selection of the MCS for initial transmission was far off, e.g. due to CQI measurement imperfections, and hence continuing with HARQ would not lead to successful decoding at the RX side.

However, it is difficult to operate RLC with small PDB needed for services like XR: 1. retransmissions are triggered in the transmitter by status reports received from the receiver, and this may not be fast enough; 2.RLC never gives up on retransmitting and if a packet reaches the maximum number of retransmissions, it notifies upper layers, which triggers radio link failure (RLF). If for instance, only one ARQ retransmission is possible (according to the PDB) RLC will never stop retransmitting whenever that one retransmission is not enough, and we will end up with RLF which we are trying to avoid since the maximum number of retransmission is due to PDB rather than channel conditions. Another issue is that those additional retransmissions will clog the transmission path and stop new PDUs from being transmitted, increasing the severity of the issue.

An indication that RLC retransmissions are needed would be the scheduling of a TB after transmitting all RLC PDUs once. This could be used to retransmit RLC PDUs blindly.

Proposal 1: investigate blind retransmissions of RLC PDUs.

# 3	PDCP Concatenation

One possibility to enhance the handling of PDU sets in the radio protocols (and benefit from the potential gains associated to the discard procedures [R2-2209557]) would be to concatenate the PDCP SDUs belonging to the same PDU set.

Proposal 2: investigate the concatenation of PDCP SDUs belonging to the same PDU set at PDCP.

# 4	Multiple CG configurations

During RAN1 #110bis the following agreement was made [2]:

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration

From RAN2 point of view, we would like to confirm that it should be already possible for the UE to select an UL grant which best fits TBS when there are multiple UL grants. Whether (and how) to have an explicit indication of which one is selected  and whether (and how) to increase CG transmission occasions is also up to RAN1.

Allowing the UE to build a smaller TB than allocated resource would introduce unnecessary complexity for the NW decoding. Different TBS could be easily supported with multiple CG configurations. There is no restriction that the multiple CG configurations cannot overlap so far, and they can be activated simultaneously with LCH restrictions and HARQ processes split configured. If RAN1 introduces UCI to indicate which/number of CG occasions are used, it could simplify the NW blind decoding and the NW could reallocate the resource to other UEs. Even if without UCI indication, the NW can also configure different DMRS to correctly decode on which CG the UE has made the UL transmission.

Proposal 3: RAN2 to confirm it is already possible to configure and simultaneous activate multiple overlapping CG configurations.

For unlicenced band, HARQ process sharing is supported with same processes allocated to different CG configurations, while it is not possible for licensed band with the restriction captured in MAC, as multiple CG was introduced for IIoT/URLLC with the assumption that they will be used for different LCH. This restriction could easily be lifted though.

38.321: NOTE 5: If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

Proposal 4: the restriction of no HARQ process sharing for licensed band should be lifted to allow more flexibility for NW configuration.

Proposal 5: The UE may take TBS of the CG and buffered data into account (on top of existing LCP restrictions and LCH prioritization rules) when selecting an UL grant to use when there are multiple grants.

# 5	RRM Measurements

UEs performing RRM measurements does not come for free as there are cases where this imposes scheduling restrictions: intra-frequency measurements (SMTC) and measurement gaps (inter-frequency). For instance, our studies in RAN1 have shown that when accounting for the SMTC measurement restrictions in FR2, we observe a XR capacity reduction of 4 users per cell both for CG and AR/VR applications (i.e., from 10 CG users per cell down to 6 CG users per cell and 7 AR/VR users per cell down to 3 AR/VR users per cell).

This issue is being investigated in RAN1

Observation: keep in mind that RRM measurements might severely impacts XR capacity.

# 6	Conclusion

This contribution has reviewed possible capacity enhancements for XR and has suggested the following:

Proposal 1: investigate blind retransmissions of RLC PDUs.

Proposal 2: investigate the concatenation of PDCP SDUs belonging to the same PDU set at PDCP.

Proposal 3: RAN2 to confirm it is already possible to configure and simultaneous activate multiple overlapping CG configurations.

Proposal 4: the restriction of no HARQ process sharing for licensed band should be lifted to allow more flexibility for NW configuration.

Proposal 5: The UE may take TBS of the CG and buffered data into account (on top of existing LCP restrictions and LCH prioritization rules) when selecting an UL grant to use when there are multiple grants.