3GPP TSG-RAN WG2 Meeting #120          	          R2-2211604

Toulouse, France, 14th Nov – 18th Nov, 2022

Agenda Item:	6.9.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on the update of last used cell

Document for:	Discussion and decision

# Introduction

During previous meetings, it was agreed to reuse the LTE noLastCellUpdate mechanism for NR PEI based on the RAN3 LS [1] which confirmed that there can be a mismatch for ‘last used cell’ between UE and network. A PEI-capable UE maintains or updates the last used cell information in response to the received RRCRelease message. However, there will be an issue with the noLastCellUpdate mechanism in case the AS security is not activated, which causes the mechanism to fail.

In this contribution, we discuss the issue of the noLastCellUpdate mechanism in case the AS security is not activated and provide our suggestion for addressing it.

# Discussion

## Issue analysis

According to the discussion in RAN2 for LTE WUS [2], the identified issue is that the UE assumes the ‘last used cell’ is a cell through which the last connection was released, but the CN is actually not informed to update the ‘last used cell’ to that cell since the eNB releases the UE without establishing/releasing the S1 connection (e.g. in case of CN overload or MME reset).

The same issue exists in NR that the gNB may release a UE without establishing the N1/NG connection with the CN. Therefore, an indication (i.e., noLastCellUpdate) is introduced in RRCRelease message for indicating the UE not to update its last used cell information in this case.

However, in such a case, the AS security cannot be activated since the UE is not yet connected to the CN and does not complete the prerequisite NAS security procedure. Then as highlighted below, based on the current procedure for RRC release, if the AS security is not activated, the UE will ignore all fields (except waitTime) in the RRCRelease message, which means that even if the gNB sends the noLastCellUpdate field, this field is ignored by the UE and the UE will accordingly update its last used cell information. This is an undesired behaviour which makes the UE fail to receive the paging through the PEI mechanism due to unaligned last cell information between the network and the UE.

### 5.3.8 RRC connection release #### 5.3.8.3 Reception of the RRCRelease by the UE The UE shall: 1> delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier; 1> stop timer T380, if running; 1> stop timer T320, if running; 1> if timer T316 is running; 2> stop timer T316; 2> clear the information included in VarRLF-Report, if any; 1> stop timer T350, if running; 1> stop timer T346g, if running; 1> if the AS security is not activated: 2> ignore any field included in RRCRelease message except waitTime; 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends; ……

Observation 1: If the gNB releases the UE without establishing the N1/NG connection, the AS security cannot be activated in this case due to the fact that the UE is not connected to the CN and the prerequisite NAS security procedure is not completed.

Observation 2: The noLastCellUpdate field in the RRCRelease message is ignored by the UE in case the AS security is not activated, which results in the UE undesirably updating its last used cell to the current cell and thus failing to receive paging through the PEI mechanism due to unaligned last cell information between the network and the UE.

## Possible options for clarifying the UE behaviour

We believe there are two options for clarifying the UE behaviour in the specs.

### Option 1: UE does not to ignore the noLastCellUpdate in case AS security is not activated

Based on the above analysis, the issue is caused by ignoring the noLastCellUpdate field, thus a straightforward way is to let the UE interpret this field even if the AS security is not activated. Then the UE can decide how to maintain its last used cell information based on the value of the noLastCellUpdate field.

In this way, the intention of letting the UE not update the last used cell when the N1/NG connection is not established can be achieved by the introduced noLastCellUpdate mechanism.

A possible change in procedure text for this option is shown below.

### 5.3.8 RRC connection release #### 5.3.8.3 Reception of the RRCRelease by the UE The UE shall: …… 1> if the AS security is not activated: 2> ignore any field included in RRCRelease message except waitTime and noLastCellUpdate if included; 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends; ……

### Option 2: UE does not update the last used cell information in case AS security is not activated

In addition to the above case related to N1/NG connection establishment, another case for the AS security not being activated by the network is related to the false/fake base station. If the gNB is an illegal base station, the AS security may also not be activated by such a base station.

Therefore, taking the false/fake base station scenario into account, a better way will be that we directly specify the UE not to update its last used cell in case the AS security is not activated, regardless of the noLastCellUpdate value set by the network.

In this way, the UE will not update its last used cell to the cell where the N1/NG connection is not established and also to the cell controlled by a false/fake base station.

An example of the spec change for this option is given below.

#### – RRCRelease The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection. Signalling radio bearer: SRB1 RLC-SAP: AM Logical channel: DCCH Direction: Network to UE RRCRelease message -- ASN1START -- TAG-RRCRELEASE-START ...... RRCRelease-v1710-IEs ::= SEQUENCE { noLastCellUpdate-r17 ENUMERATED {true} OPTIONAL, -- Need S nonCriticalExtension SEQUENCE {} OPTIONAL } ...... -- TAG-RRCRELEASE-STOP -- ASN1STOP noLastCellUpdate Presence of the field indicates that the last used cell for PEI shall not be updated. When the field is absent, the UE shall update its last used cell with the current cell. The UE shall not update its last used cell with the current cell if the AS security is not activated..

Proposal 1: We propose RAN2 to discuss and specify a solution to solve the issue of UE incorrectly updating the last used cell information in case the AS security is not activated. Possible solutions are:

- Option 1: Clarify that the UE does not ignore the noLastCellUpdate field in RRCRelease in case the AS security is not activated.

- Option 2: Clarify that the UE does not update its last used cell in case the AS security is not activated, regardless of whether the noLastCellUpdate field is included in RRCRelease.

Proposal 2: Considering the fact that Option 2 in proposal 1 is more secure from security prospective as it will prevent the false/fake base station from changing the noLastCellUpdate value for the UEs, we would like to propose RAN2 to agree to include the clarification suggested by Option 2 in RRC specification.

# Conclusion

In this paper, we discuss UE behaviour for updating its last used cell in case the AS security is not activated. Observations and proposals are summarized as follows.

Observation 1: If the gNB releases the UE without establishing the N1/NG connection, the AS security cannot be activated in this case due to the fact that the UE is not connected to the CN and the prerequisite NAS security procedure is not completed.

Observation 2: The noLastCellUpdate field in the RRCRelease message is ignored by the UE in case the AS security is not activated, which results in the UE undesirably updating its last used cell to the current cell and thus failing to receive paging through the PEI mechanism due to unaligned last cell information between the network and the UE.

Proposal 1: We propose RAN2 to discuss and specify a solution to solve the issue of UE incorrectly updating the last used cell information in case the AS security is not activated. Possible solutions are:

- Option 1: Clarify that the UE does not ignore the noLastCellUpdate field in RRCRelease in case the AS security is not activated.

- Option 2: Clarify that the UE does not update its last used cell in case the AS security is not activated, regardless of whether the noLastCellUpdate field is included in RRCRelease.

Proposal 2: Considering the fact that Option 2 in proposal 1 is more secure from security prospective as it will prevent the false/fake base station from changing the noLastCellUpdate value for the UEs, we would like to propose RAN2 to agree to include the clarification suggested by Option 2 in RRC specification.

# References

- R2-2206932, Reply LS on PEI and UE Subgrouping (R3-224004; contact: ZTE).

- R2-2005985, LS on system support for WUS.