3GPP TSG-RAN WG2 Meeting #120	R2-2211605

Online, 14th – 18th Nov, 2022

Agenda Item:	6.7.3

Source: 	Huawei, HiSilicon

Title: 	Clarification on MAC filtering for discovery message

Document for:	Discussion and Decision

# Introduction

In this paper, we  discuss the MAC filtering issue for the discovery message. In the relay discovery procedure, a remote UE may receive a unicasted discovery message but cannot recognize the source Layer-2 ID contained in it, as discussed  in section 2.1. However, the current MAC filtering process in the specification does not consider this case, thus clarification and modification are needed.

# Discussion

## 2.1 Cast type for discovery message

We have two discovery models, Model A and Model B, for relay discovery. In model A, the relay UE broadcasts the Announcement message with a self-assigned source Layer-2 ID and a default destination layer-2 for sending and receiving the initial UE-to-network relay discovery signaling. When remote UE receives the Announcement message and decides to establishment the PC5 connection with the relay UE, remote UE will unicast the DCR message with a self-assigned source Layer-2 ID and a destination Layer-2 ID filled with the source Layer-2 ID in the Announcement message. Relay UE then responses the DCA message to complete the PC5 establishment.

Figure 1. Discovery Model A

In Model B, remote UE broadcasts the Solicitation message with a self-assigned source Layer-2 ID and a default destination layer-2 for discovery signaling. When relay UE receives the Solicitation message and decides to provide relay service for the remote UE, the relay UE will response a Response message with a self-assigned source Layer-2 ID and a destination filled with the source Layer-2 ID in the Solicitation message. That means the Response message is transmitted in the unicast mode. When remote UE receives the Response message from the relay UE, it initiates the PC5 connection establishment procedures with the relay UE.

Figure 2. Discovery Model B

2.2 MAC filtering for discovery message

As it can be seen from the discovery model B in section 2.1, the Response message from relay UE to remote UE is transmit in the unicast mode. The destination Layer-2 ID related to this message is the source Layer-2 ID assigned by the remote UE for sending the Solicitation message. However, since the related source Layer-2 ID is self-assigned by the relay UE, it can be received by the remote UE for the first time and in this case the remote UE cannot recognize the source Layer-2 ID contained in the Response message. This case is similar to the scenario 2/3 discussed in R2-2202361[1]. In RAN2-118e meeting, we have the following agreement on MAC filtering procedure for scenario 2/3. Similar, we need to update the MAC filtering procedure to consider the mentioned discovery procedure as well.

Proposal 5: RAN2 agree to add one note in MAC spec to solve the mac filtering issue for at least scenario2/3. The below content can be further discussed during phase-III and submitted one CR to the incoming RAN2 meeting. (38.321)NOTE: If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

Proposal 1. Update the MAC filtering procedure in the MAC specs as suggested in the TP in the Annex to handle the case where a remote UE receives the Discovery Response message via unicast but is not aware of the relay UE’s source layer-2 ID contained in it.

The corresponding corrections for TS 38.321 is shown in the Annex.

# Conclusion

Proposal 1. Update the MAC filtering procedure in the MAC specs as suggested in the TP in the Annex to handle the case where a remote UE receives the Discovery Response message via unicast but is not aware of the relay UE’s source layer-2 ID contained in it.

# Reference

[1] 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification ". V17.2.0.

[2] R2-2202361, Summary [POST116-e][710][V2X/SL] PDCP/RLC Entity Maintenance for SL-SRBs (CATT) RAN 2-117e.

# Annex

-------------------------------------------------Change for TS 38.321------------------------------------------------------

##### 5.22.2.2.2	Sidelink process

For each PSSCH duration where a transmission takes place for the Sidelink process, one TB and the associated HARQ information is received from the Sidelink HARQ Entity.

For each received TB and associated Sidelink transmission information, the Sidelink process shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB:

3>	if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or

3>	if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:

4>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

NOTE:	If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0, 1, or 58, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair.

2>	consider the Sidelink process as unoccupied.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

1>	if HARQ feedback is enabled by the SCI:

2>	if negative-only acknowledgement is indicated by the SCI according to clause 8.4.1 of TS 38.212 [9]:

3>	if UE's location information is available and distance beteween UE's location and the central location of the nearest zone that is calculated based on the Zone_id in the SCI and the value of sl-ZoneLength corresponding to the communication range requirement in the SCI as specified in TS 38.331 [5] is smaller or equal to the communication range requirement in the SCI; or

3>	if none of Zone_id and communication range requirement is indicated by the SCI; or

3>	if UE's location information is not available:

4>	if the data which the MAC entity attempted to decode was not successfully decoded for this TB and the data for this TB was not successfully decoded before:

5>	instruct the physical layer to generate a negative acknowledgement of the data in this TB.

2>	if negative-positive acknowledgement or unicast is indicated by the SCI according to clause 8.4.1 of TS 38.212 [9]:

3>	if the data which the MAC entity attempted to decode was successfully decoded for this TB or the data for this TB was successfully decoded before:

4>	instruct the physical layer to generate a positive acknowledgement of the data in this TB.

3>	else:

4>	instruct the physical layer to generate a negative acknowledgement of the data in this TB.

---------------------------------------------------Change ends--------------------------------------------------------------