3GPP TSG-RAN WG2 Meeting #120	R2-2211607

Online, 14th – 18th Nov, 2022

Agenda Item:	8.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity

Document for:	Discussion and Decision

# Introduction

In last meeting, RAN2 has reached the following agreements for R18 service continuity enhancements.

Agreements: Proposal 1 (modified) For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2. FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE. Proposal 3 For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching. Proposal 4 (modified) For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity. Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available). Proposal 5 For i2d path switch scenario, re-use the existing T304 timer Proposal 6 For d2i and i2i path switch scenarios, re-use the existing T420 timer.

In this paper, we will continue discussing the solutions for the scenarios in Figure 1. We will first recall and check the RAN3 progress, then consider the stage 2 procedures which take the R17 mechanisms as the baseline and introduce the additional elements needed on top of these baseline procedures.

Figure 1. Path switching scenarios in R18: A/B. Inter-gNB direct-to-indirect/indirect-to-direct path switching; C: intra-gNB indirect-to-indirect path switching; and D: inter-gNB indirect-to-indirect path switching

# Discussion

## 2.1 path type/ target relay UE selection

We have reached some agreements on the measurement event, sidelink measurement and the timer design in last RAN2 meeting, but the two key issues below are not decided yet, which is left to RAN3.

- Issue 1: Which node should decide on the new path type i.e., direct or indirect path?

- Issue 2: Which node selects the target U2N relay UE?

For the first issue, majority companies preferred that the source gNB decides the new path type in last RAN3 meeting, so RAN3 agreed the working assumption that source gNB selects the target path type (direct or indirect). This working assumption is reasonable from RAN2 perspective, since allowing source gNB select the target path type is more aligned with the legacy HO procedure wherein source gNB decides the target gNB and the target cell. Therefore, RAN2 can inform RAN3 that we agree with the working assumption above and will take it as the baseline for service continuity procedures design.

Proposal 1: To progress this it is proposed that RAN 2 confirms the RAN3 WA and inform RAN3 that RAN2 agrees with the working assumption for inter-gNB path switching that source gNB selects the target path type (direct or indirect), and will take this WA as the baseline for service continuity procedures design.

For issue 2, there are three option 3 for discussion:

- Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

- Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

In last RAN3 meeting, 7 out of 11 companies preferred (or OK with) Option 1, and 4 out of 11 companies did not support it. We think Option 1 is more aligned with the legacy HO procedure, and the only impact is to carry the target relay UE ID in the Xn signalling from source gNB to target gNB. Option 2/3 might allow target gNB to choose a Relay UE with better Uu link other than only considering sidelink measurement results reported by the remote UE, but it is only applicable in case the Relay UE is in RRC_CONNECTED state. Besides, multiple candidate relay UE information should be carried in the HO request message which will increase the signalling overhead. Considering the simplicity and signalling efficiency, we slightly prefer to choose Option 1.

Proposal 2: For inter-gNB path switching, when the source gNB decides to switch the remote UE to an indirect path, it can further select the target relay UE.

## 2.2 service continuity procedures

In the following subsection, based on proposals 1 and 2, we describe the how the stage-2 procedures would work for:

- inter-gNB indirect-to-direct path switching,

- inter-gNB direct-to-indirect path switching,

- intra-gNB indirect-to-indirect path switching, and

- inter-gNB indirect-to-indirect path switching.

We will also provide the relevant proposals to support these procedures.

## 2.2.1 Inter-gNB indirect-to-direct path switching

Figure 2 shows the procedure for the inter-gNB indirect-to-direct path switching case which is extended from Rel-17 intra-gNB indirect-to-direct path switching procedure as defined in clause 16.12.6.1 in TS 38.300.

Remote UE

Relay UE

S-gNB

9. sidelink release

T-gNB

3. HO request

4. HO ACK

6. RA to target gNB

8. RRC Reconfiguration

UL and DL data

5. RRC Reconfiguration message

1. Measurement configuration and reporting

2. HO decision

7. RRC Reconfiguration complete message

Figure 2.  Inter-gNB indirect-to-direct path switching procedure

Step 1. Remote UE triggers the measurements report when the sidelink measurement quantity between Relay UE and Remote UE indicates deteriorated link quality. The triggering Event X1 and Event X2 can be reused in the measurement configuration. The Uu measurements of neighbour cells can be included in the measurements report.

- Event X1: Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2

- Event X2: Serving L2 U2N Relay UE becomes worse than threshold

Step 2. Source gNB decides to switch the Remote UE to a target cell in the target gNB based on the reporting results, based on proposal 1.

Step 3. Source gNB sends the HANDOVER REQUEST message to target gNB over the Xn interface/NG interface, which contains target cell ID, Remote UE’s UE context, and other necessary information. (Here we take Xn-based path switching as an example).

Step 4. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

Step 5. Source gNB sends the RRC configuration to Remote UE.

Step 6. The UE (Remote UE in previous steps) synchronizes with the target gNB and performs Random Access.

Step 7. The UE sends the RRC Reconfiguration complete message to target gNB via the direct path. From this step, the UE uses the RRC connection via the direct path to the gNB.

Step 8. Source gNB sends RRC Reconfiguration message to the Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB.

Step 9. Sidelink between Remote UE and Relay UE is released, which can be triggered by either Relay UE or Remote UE.

In the above procedures, we can see the HO preparation between S-gNB and T-gNB in step 3 and step 4 are needed to enable inter-gNB path switching, other than that the whole procedure of intra-gNB path switching is applicable to inter-gNB path case. The information/configuration contained in the messages in step 3 and step 4 are the same as those used for a normal UE’s handover.

Proposal 3a: In inter-gNB indirect-to-direct path switching, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

Proposal 3b: Agree the above procedure in Figure 2 as a baseline for Inter-gNB indirect-to-direct path switching.

## 2.2.2 Inter-gNB direct-to-indirect path switching

Remote UE

Target Relay UE

S-gNB

8. sidelink establishment if not exist

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

3. HO decision

2. Measurement configuration and reporting

4. HO request

5. HO ACK

9. RRC Reconfiguration complete message

6. RRC Reconfiguration message

UL and DL data

Figure 3 shows the inter-gNB direct-to-indirect path switching procedure, which is extended from Rel-17 procedure for L2 U2N Remote UE switching to indirect path as defined in clause 16.12.6.2 in TS 38.300.

Figure 3.  Inter-gNB direct-to-indirect path switching procedure

Steps 1 and 2: Remote UE reports the candidate Relay UE(s) to the source gNB, which is discovered by discovery procedure in step 1. The triggering Event Y1 and Event Y2 can be reused in the measurement configuration. The reported results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

- Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

- Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

Step 3: Based on measurement results, the source gNB is aware that HO/path switching is needed. Based on proposal 1 and 2, the source gNB decides to switch the UE to an indirect path, and selects a target relay under a target cell in the target gNB.

Step 4: To perform HO preparation, source gNB sends the HANDOVER REQUEST message to target gNB via Xn interface/NG interface. (Here we take Xn-based path switching as an example). In this message, the target relay UE ID should be contained together with the target cell ID.

Proposal 4a: For inter-gNB direct-to-indirect path switching, source gNB indicates the target Relay UE ID in HO request message to target gNB.

Proposal 4b: Agree the above procedure in Figure 3 as a baseline for Inter-gNB direct-to-indirect path switching.

Step 5. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACK to the source gNB.

Step 6. Source gNB sends the RRC configuration to Remote UE.

Step 7. Target gNB sends the relaying reconfigurations to the Relay UE.

Step 8. Remote UE establishes the sidelink RRC connection with the target Relay UE if it does not exist.

Step 9. Remote UE sends the RRC complete message to gNB via the Relay UE.

Regarding the measurement configuration, it would be beneficial if gNB can also indicate the Allowed-List and Block-List to restrict the Relay UE’s serving cell. Then Remote UE would not report the Relay UE that is camped on the Block-List cells or contained in the Block-List for Relay UE.

Proposal 5: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

## 2.2.3 Intra-gNB indirect-to-indirect path switching

The indirect-to-indirect path switch type has not been discussed in R17 SL relay, but some legacy procedures and principle can be reused as well with some modification. Combining the procedure of direct to indirect path switching with the one for indirect to direct path switching, Figure 4 shows the intra-gNB indirect-to-indirect path switching procedure. Before the path switching, the Remote UE is connected with the same gNB via a Relay UE. After the path switching, the Remote UE established unicast link with a new Relay UE (i.e. target Relay UE), and the unicast between the Remote UE and the old Relay UE is be released (as in step 9).  Meanwhile, the gNB sends RRC message to the old Relay UE to release/reconfigure the relaying configuration associated the Remote UE (as in step8). These procedures are similar to the indirect-to-direct path switching case in R17.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

9. sidelink release

8. RRC Reconfiguration

5. RRC Reconfiguration for Remote UE

2. Measurement configuration and reporting, contain the target Relay UE info

UL and DL data

4. RRC Reconfiguration message

6. sidelink establishment if not exist

7. RRC Reconfiguration complete message

Figure 4.  Intra-gNB indirect-to-indirect path switching procedure

Step 1 and Step 2. Remote UE triggers to report the candidate Relay UE(s), when the sidelink measurement quantity between Relay UE and Remote UE shows deteriorated link quality. The reporting results includes the at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

Step 3. The gNB decides to switch the Remote UE to a target Relay UE in the target cell of the same gNB based on the reporting results.

Step 4. The gNB sends an RRC configuration message to the Remote UE. The RRC message include at least L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s).

Step 5. The gNB sends an RRC configuration message to the target Relay UE, which includes at least the Remote UE's L2 ID and local ID, Uu and PC5 Relay RLC channel configuration for relay traffic and bearer mapping configuration.

Step 6. Remote UE establishes the sidelink RRC connection with the target Relay UE if not exist.

Step 7. Remote UE sends the RRC complete message to gNB via the target Relay UE.

Step 8. The gNB sends an RRC message to the source Relay UE to reconfigure the connection between the source Relay UE and the gNB.

Step 9. Sidelink between Remote UE and the source Relay UE is released.

Proposal 6: Agree the above procedure in Figure 4 as a baseline for intra-gNB indirect-to-indirect path switching.

For indirect-to-indirect path switch, the new measurement events are needed to compare the RSRP of candidate Relay UEs with the serving Relay UE as described below:

- Event1: Candidate Relay UE becomes offset better than the serving Relay UE

- Event2: Serving Relay UE becomes worse than threshold 1 and candidate Relay UE becomes better than threshold 2

In last meeting, Event1 is agreed to be used for indirect-to-indirect path switch, while Event2 is FFS. We think Event2 should be supported in principle, since it is similar to the legacy Event A3.

Proposal 7: For indirect-to-indirect path switching, agree the new measurement event that candidate Relay UE becomes offset better than the serving Relay UE

## 2.2.4 Inter-gNB indirect-to-indirect path switching

Figure 5 shows the inter-gNB indirect-to-indirect path switching procedure, which reuses the procedures for intra-gNB indirect-to-indirect case and the procedures over the Xn interface is the same as the inter-gNB direct-to-indirect path switching case.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

4. HO request

5. HO ACK

11. sidelink release

10. RRC Reconfiguration

UL and DL data

2. Measurement configuration and reporting, contain the target Relay UE info

6. RRC Reconfiguration message

8. sidelink establishment if not exist

9. RRC Reconfiguration complete message

Figure 5.  Inter-gNB indirect-to-indirect path switching procedure

We believe to support Rel-18 inter-gNB indirect-to-indirect path switching on top of Rel-18 intra-gNB path switching from indirect to indirect path, the HO preparation between source gNB and target-gNB can be performed in the same way as Rel-18 inter-gNB path switching from direct to indirect path, without additional RAN2/RAN3 spec impact. Hence we can consider the above procedure as the baseline for inter-gNB indirect-to-indirect path switching

Proposal 8: Agree the above procedure in Figure 5 as a baseline for inter-gNB indirect-to-indirect path switching.

## 2.3 Data Delivery for Inter-gNB cases

    - 2.3.1 DL data delivery

In Rel17, where the intra-gNB path switch is supported, if downlink data packets are not transmitted from the relay UE to the remote UE in the source indirect link before remote UE receiving path switch command, gNB can retransmit the data packets according to the PDCP status report sending in the target direct link. Lossless DL data transmission can be realized when gNB configures PDCP status report.

Observation 1: In intra-gNB path switch case, data retransmission in the target gNB via PDCP status report can avoid UE DL data loss without any spec impact.

For Inter-gNB path switch from indirect to direct, the DL data needs to be forwarded to the T-gNB from the S-gNB. In the DL data forwarding for the inter-gNB cases, same as the handover, the S-gNB forwards the unacknowledged PDCP SDU to the T-gNB for AM DRB as follows.

As long as data forwarding of DL user data packets takes place, the source NG-RAN node shall forward user data in the same forwarding tunnel, i.e. - for any QoS flow accepted for data forwarding by the target NG-RAN node and for which a DRB DL forwarding tunnel was established for a DRB to which this QoS flow was mapped at the source NG-RAN node, any fresh packets of this QoS flow shall be forwarded as PDCP SDUs via the mapped DRB DL forwarding tunnel. - for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE. NOTE: The SN of forwarded PDCP SDUs is carried in the "PDCP PDU number" field of the GTP-U extension header. - for any QoS flow accepted for data forwarding by the target NG-RAN node for which a DL PDU session forwarding tunnel was established, the source NG-RAN node forwards SDAP SDUs as received on NG-U from the UPF.

For the cases where path switch from indirect path,  the S-gNB may successfully transmit the data packet to the relay UE and obtain the RLC ACK, while the relay UE has not obtained the PC5 RLC ACK from the remote UE. For example, S-gNB has transmitted packets to the relay UE with SN {1,2,3,4,5,6,7,8} , the next packet associated SN will be 9. S-gNB only get the ACK of packets with SN {1,3,5,6,8} from the relay UE. The relay UE also send the {1,3,5,6,8} to the remote UE, but none of them get ACK from the remote UE. Then the remote UE receives the path switch command and switch to the T-gNB for connection establishment.

According to the legacy data forwarding procedure, where only the unacknowledged data packets are forwarded, the remote UE will suffer from data loss, i.e., packets {1,3,5,6,8} are lost.

Observation 2: In inter-gNB path switch case, data retransmission in the target gNB via PDCP status report cannot avoid UE DL data loss due to legacy data forwarding procedure, i.e., the target gNB may not have all the unacknowledged data in PDCP status report.

Based on the analysis above, the problem results from the lack of accurate information of the receiving status of UE in the S-gNB. So the following options can be used to solve the problem.

Option 1: Send PDCP status report to the S-gNB after receiving the path switch command.

Option 2: Enhance the inter-gNB data forwarding for T-gNB for inter-gNB path switch.

Option 1 can limit the impact in RAN2 and Option 2 needs some changes together with RAN3.

Proposal 8: Discuss avoiding DL data loss during the inter-gNB path switch with following two options.

Option 1: Send PDCP status report to the S-gNB after receiving the path switch command.

Option 2: Enhance the inter-gNB data forwarding for T-gNB for inter-gNB path switch.

    - 2.3.2 UL data delivery

For UL data delivery during the path switch, some data packets are received by the relay UE but has not been transmitted to the gNB before the path switch. When the remote UE path switch to the direct link, the remote UE may be indicated to do PDCP reestablishment or PDCP data recovery to retransmit the data which has been confirmed by RLC. However, the confirmation from RLC only indicates the data PDU is received by the relay UE and the data may not be received by the gNB.  Therefore, there may also be data loss during UL data delivery.

According to the UL data forwarding procedure during HO, the transmission status of the UL transmission will be transmitted to the T-gNB via SN STATUS TRANSFER. Therefore, the T-gNB will know which data packets is not delivered.

However, based on the legacy retransmission procedure of the UE, these data may not be retransmitted to the T-gNB due to these packets are acknowledged by the relay UE.

Observation 3: in both intra-gNB and inter-gNB path switch case, if the source gNB cannot obtain all the data packets at the relay UE after release of the RRC connection of the remote UE, UL data will be lost due to the retransmission during the legacy PDCP reestablishment.

To solve the problem, one way is to make the UE obtain the receiving status of the UL data when receiving the path switch command from the S-gNB. Another way is to introduce new PDCP status report, which can trigger PDCP retransmission of the UE.

Proposal 9: Discuss to avoid UL data loss during the intra-gNB/inter-gNB path switch via letting UE retransmit the data packet according to the received status of the UL data during path switch.

# Conclusion

Based on above discussion, we have the following proposal.

path type/ target relay UE selection

Proposal 1: To progress this it is proposed that RAN 2 confirms the RAN3 WA and inform RAN3 that RAN2 agrees with the working assumption for inter-gNB path switching that source gNB selects the target path type (direct or indirect), and will take this WA as the baseline for service continuity procedures design.

Proposal 2: For inter-gNB path switching, when the source gNB decides to switch the remote UE to an indirect path, it can further select the target relay UE.

Service Continuity Procedures

Inter-gNB indirect-to-direct path switching

Proposal 3: In inter-gNB indirect-to-direct path switching, the HO preparation between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN2/RAN3 spec impact.

Proposal 3b: Agree the above procedure in Figure 2 as a baseline for Inter-gNB indirect-to-direct path switching.

Inter-gNB direct-to-indirect path switching

Proposal 4a: For inter-gNB direct-to-indirect path switching, source gNB indicates the target Relay UE ID in HO request message to target gNB.

Proposal 4b: Agree the above procedure in Figure 3 as a baseline for Inter-gNB direct-to-indirect path switching.

Proposal 5: Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell.

Intra-gNB indirect-to-indirect path switching

Proposal 6: Agree the above procedure in Figure 4 as a baseline for intra-gNB indirect-to-indirect path switching.

Proposal 7: For indirect-to-indirect path switching, agree the new measurement event that candidate Relay UE becomes offset better than the serving Relay UE

Inter-gNB indirect-to-indirect path switching

Proposal 8: Agree the above procedure in Figure 5 as a baseline for inter-gNB indirect-to-indirect path switching.

Data Delivery for Inter-gNB cases

Observation 1: In intra-gNB path switch case, data retransmission in the target gNB via PDCP status report can avoid UE DL data loss without any spec impact.

Observation 2: In inter-gNB path switch case, data retransmission in the target gNB via PDCP status report cannot avoid UE DL data loss due to legacy data forwarding procedure, i.e., the target gNB may not have all the unacknowledged data in PDCP status report.

Observation 3: in both intra-gNB and inter-gNB path switch case, if the source gNB cannot obtain all the data packets at the relay UE after release of the RRC connection of the remote UE, UL data will be lost due to the retransmission during the legacy PDCP reestablishment.

Proposal 8: Discuss avoiding DL data loss during the inter-gNB path switch with following two options.

Option 1: Send PDCP status report to the S-gNB after receiving the path switch command.

Option 2: Enhance the inter-gNB data forwarding for T-gNB for inter-gNB path switch.

Proposal 9: Discuss to avoid UL data loss during the intra-gNB/inter-gNB path switch via letting UE retransmit the data packet according to the received status of the UL data during path switch.

# Reference

[1] RP-221262, Revised WID on NR sidelink relay enhancements, LG Electronics