3GPP TSG-RAN WG2 Meeting #120	 R2-2211612

Toulouse France, 14 Nov – 18 Nov, 2022

Agenda Item:	8.11.4

Source: 	NEC

Title:  	Discussion on RAN sharing scenarios for eMBS

Document for:	Discussion and Decision

- 1 Introduction

In the objective of this WID, RAN sharing scenarios for eMBS is described as following:

- Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

It seems RAN2 would not touch this topic since it was regarded as a RAN3 only topic in RAN perspective. However, companies are invited to share the views on the LS from RAN3 (R3-226084) [1], thus in this paper, we give our analysis and make the answer.

- 2 Discussion

- 2.1 Background

Network sharing is a common practice to reduce network CAPEX. With RAN sharing deployment, if the same Multicast/Broadcast service is provided by two (or more) operators separately, this service would be recognized as separate TMGIs resulting in duplicated PTM radio resources consumption in the same cell for transmission of the same content. This justifies resource efficiency improvement in the RAN sharing scenario.

- 2.2 Analysis on LS question

Question 1: Does RRC support in Rel-17 configuration of an MBS broadcast session, which is associated with multiple TMGIs?

In Rel-17, broadcast configuration is provided by MCCH information (i.e., MBSBroadcastConfiguration) which includes mbs-SessionInfoList. Based on the current RRC spec, the mbs-SessionId (i.e., TMGI) is configured per MBS-SessionInfo, which means each session can only be configured by one TMGI.

Answer 1: Based on the current RRC spec, each MBS broadcast session can only be configured by one TMGI.

Question 2: RRC supports the indication whether a neighbour cell provides the broadcast service on MTCH. Given the size of the mbs-NeighbourCellList to which the mtch-NeighbourCell in each MBS-SessionInfo item refers to, it is possible that not all neighbour cells can be indicated. This size-limitation would presumably be more acute in RAN sharing scenarios, at the border between a shared area and a non-shared area or similar. Can it be assumed that service continuity is also supported towards a neighbour cell not indicated in the mbs-NeighbourCellList?

Based on RRC spec, currently RAN2 support at most 8 neighbour cells’ broadcast information, and mtch-NeighbourCell within MBS-SessionInfo can indicate whether the service is provided or not in neighbour cells within mbs-NeighbourCellList. Furthermore, if the field mtch-NeighbourCell is absent, UE can not determine whether the service is provided or not in neighbour cells. In our understanding, this field is an optional optimization to let UE be aware of the broadcast information in advance during cell reselection procedure (e.g., re-connect with network for unicast if the service is not provided). Even if a neighbour cell is not indicated in the mbs-NeighbourCellList, as long as this neighbour cell can provide the same service, then service continuity is also supported.

On the other hand, for MBS cell reselection optimization, SIB21 which contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI can also be considered.

Answer 2: The neighbour cell information is an optional optimization to let UE be aware of the broadcast information in advance during cell reselection procedure (e.g., UE can re-connect with network for requesting unicast (DRB) if the broadcast service is not provided). Even if a neighbour cell is not indicated in the mbs-NeighbourCellList, as long as this neighbour cell can provide the same service, then service continuity is also supported. Furthermore, SIB1 which contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI can also be considered.

Question 3: Is there any significant limitation from RRC point of view if the TMGI as received by the 5GC contains a PLMN/SNPN ID not broadcast in SIB1?

Based on the current RRC spec, TMGI includes PLMN ID but not includes SNPN ID. For PLMN case, if the TMGI as received by the 5GC contains a PLMN ID not broadcast in SIB1, then the PLMN index can not be used as this is a per-cell configuration. For SNPN case, since there is no NID specified in TMGI field, it is not supported for identifying a TMGI from RAN2 point of view at least for now.

Answer 3: For PLMN case, if the TMGI as received by the 5GC contains a PLMN ID not broadcast in SIB1, then the PLMN index can not be used as this is a per-cell configuration. For SNPN case, since there is no NID specified in TMGI field, it is not supported for identifying a TMGI from RAN2 point of view at least for now.

- 3 Conclusions

Answer 1: Based on the current RRC spec, each MBS broadcast session can only be configured by one TMGI.

Answer 2: The neighbour cell information is an optional optimization to let UE be aware of the broadcast information in advance during cell reselection procedure (e.g., UE can re-connect with network for requesting unicast (DRB) if the broadcast service is not provided). Even if a neighbour cell is not indicated in the mbs-NeighbourCellList, as long as this neighbour cell can provide the same service, then service continuity is also supported. Furthermore, SIB1 which contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI can also be considered.

Answer 3: For PLMN case, if the TMGI as received by the 5GC contains a PLMN ID not broadcast in SIB1, then the PLMN index can not be used as this is a per-cell configuration. For SNPN case, since there is no NID specified in TMGI field, it is not supported for identifying a TMGI from RAN2 point of view at least for now.

- 4 References

[1] R3-226084 LS on resource efficiency for MBS reception in RAN sharing scenario