3GPP TSG-RAN WG2 Meeting #120                                                      R2-2211613

Toulouse, France, 14th - 18th November 2022

Source:	CATT

Title:	Discussion on inter-RAT SHR and SPR

Agenda Item:	8.13.4

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, SPR related issues were discussed based on offline discussion [1] and achieved some agreements [2] to make some progress on SPR.

After RAN2#119bis-e meeting, RAN2 receives the LS from RAN3 [3] in which some issues about inter-RAT SHR and SPR are proposed by RAN3.

In this contribution, we will continue to discuss some FFS issues about SPR and also to discuss the inter-RAT SHR based on the issues proposed in RAN3 LS.

Discussion

SPR

- RAN3 LS aspects

In RAN3 LS [3], RAN3 has the following questions to RAN2 related to SPR:

- Successful PSCell Change Report (SPCR) - RAN3 also discussed different aspects related to SPCR and have the following questions to RAN2: - Q6. Whether the SPCR can be stored at the UE and sent later to the gNB or is sent immediately after the successful PSCell change or addition? - Q7. Which node (MN or SN) retrieves the SPCR from the UE? - Q8. Which node (MN or SN) sends the Successful PSCell Change/Addition configuration to the UE?

For Q6, in last meeting, RAN2 agreed that SPR is fetched via UE Information Request/Response procedure which means the UE can store SPR and send later to the gNB. Furthermore, RAN2 also agreed that SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. In R17 SHR, SHR can be stored at the UE 48 hours after the last successful handover information is added to the VarSuccessHO-Report. Similarly, SPR can also be stored at the UE 48 hours after the last successful PSCell addition/PSCell change report is stored.

Proposal 1: UE can store SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE.

For Q7, RAN3 asks RAN2 that which node (MN or SN) retrieves the SPR from the UE. According to Q6, the UE can store SPR and send later to the gNB. If supporting SN retrieves the SPR from UE, it is limited as the UE only can report SPR to SN in DC scenario. Furthermore, if supporting SN retrieves the SPR from UE, the UE information request/response message will need to be extended to SN side as RAN2 agreed that SPR is fetched via UE Information Request/Response procedure. Therefore, it seems to be more suitable for MN to retrieves the SPR from UE, then, the SPR could be forwarded to corresponding SN (source SN or target SN during PSCell addition/PSCell change) to analyze and optimize.

Proposal 2: It is MN to retrieves the SPR from UE.

For Q8, RAN2 has agreed in last meeting that SPR configuration is configured by network through otherConfig, however, which node configures the triggering condition is FFS. Similarly as R17 SHR, the source PSCell can configure T310 and T312 thresholds, and target PSCell can configure T304 threshold to UE for successful PSCell change. For successful PSCell addition, the target PSCell can configure the T304 threshold to UE. The key issue is whether the MN also can configure SPR configuration to UE. From our perspective, we think that only support SN to configure SPR configuration is enough, it can be the SPR is forwarded to SN, and the SN identifies it is too late PSCell change, then the SN can also send the SPR to MN to optimize.  In addition, supporting MN configures the SPR configuration to UE could need to introduce extra SPR configuration IE in MN otherConfig IE to distinguish from SHR configuration which introduces unnecessary signaling overhead. Therefore, we prefer only SN configures the SPR configuration to the UE.

Proposal 3: The SPR configuration is configured only by SN, i.e. source SN configures T310 and T312 thresholds, target SN configures T304 threshold.

In addition, for which node (MN or SN) sends the Successful PSCell Change/Addition configuration to the UE which is proposed by RAN3, if it is the source SN to configure T310 and T312 thresholds, source SN could send the SPR configuration to UE by SRB3 directly. If it is the target SN to configure T304 threshold, it could need to be sent by MN (for inter-SN PSCell change and PSCell addition) or source/target SN (for intra-SN PSCell change) to UE.

Proposal 4: It is source SN to send the T310 and T312 thresholds of SPR configuration to UE, and MN (for inter-SN PSCell change and PSCell addition) or source/target SN (for intra-SN PSCell change) to send T304 threshold of SPR configuration to UE.

- Other aspects

Information in SPR

In last meeting, RAN2 discussed SPR contents and agreed some information (e.g. Source PSCell info, Target PSCell info, Neighbour Cells info etc.) need to be included in SPR. However there are the following three contents are still FFS whether they should or how they can be included in SPR.

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell. 7b: FFS on whether to include or on conditional inclusion of random access related information. 7c: FFS on Location Information

In R17 SHR, a CHO candidate cell flag was introduced in neighbor cell measurement to indicate whether the neighbor cell is CHO candidate cell or not. For R18 SPR, the similar CPAC candidate cell flag also can be considered to introduce to indicate whether the neighbor cell is CPAC candidate cell or not. CPAC candidate cell flag can reuse CHO candidate cell flag, no need to introduce a new flag.

Proposal 5: CPAC candidate cell flag can reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

In R17 SHR, random access related information is included only when the SHR is triggered due to T304 exceeds the configured threshold. Similarly as SHR, random access related information is included in SPR only when the SPR is triggered due to T304 exceeds the configured threshold.

Proposal 6: Random access related information is included in SPR only when the SPR is triggered due to T304 exceeds the configured threshold.

For location information, we think it is useful to include it in SPR to indicate UE location.

Proposal 7: Location information is included in SPR.

Signalling design for SPR

Another import issue is the signalling design for SPR. There are three signalling methods could be considered. One is that reuse PCell SHR for PSCell SPR. Another, the UE can record the SPR embedded in PCell SHR. Besides, the UE also can record the PCell SHR and PSCell SPR separately.

Observation 1: There are following three signaling designs for SPR:

Option1: Reuse PCell SHR;

Option2: PSCell SPR embedded in PCell SHR;

Option3: PCell SHR and PSCell SPR are separate.

For option1, if PSCell SPR reuses PCell SHR, PSCell SPR will override previously recorded PCell SHR which is not we expected. For option2, the PCell SHR and PSCell SPR will be reported together, the network cannot fetch them separately which has low flexibility. In addition, if only PSCell SPR is triggered and PCell SHR is not triggered, how the UE logs the PSCell SPR should be discussed. Option 3 allows the UE records and reports the PCell SHR and PSCell SPR separately, which provides more flexibility. Therefore, we prefer option3.

Proposal 8: UE records/reports PCell SHR and PSCell SPR separately.

Inter-RAT SHR

In RAN3 LS [3], some issues about inter-RAT SHR are proposed by RAN3 to RAN2, as follows.

- Q1. Is RAN2 planning to impact LTE specifications to support inter-RAT SHR? - Q2. Whether T304 trigger for inter-RAT SHR from NR to LTE is to be supported? - Q3. If yes to Q2, whether the inter-RAT SHR is always encoded in source RAT format or can be encoded based on the RAT format which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR encoded in NR format for T310/T312 triggers and in LTE format for T304 triggers for inter-RAT HO from NR to LTE)? - Q4. If yes to Q2, and if inter-RAT SHR is collected due to T304 triggers (configured by target LTE node), what is RAN2’s preference on the following two options? - Option 1: It is sufficient for UE to report the inter-RAT SHR once UE is back to NR - Option 2: The LTE node should have the capability to retrieve the inter-RAT SHR - RAN3 further discussed on the potential contents for inter-RAT successful handover and agreed on the following: - Source NR cell information - Target LTE cell information - Measurement results for source, target and neighbours - Cause to indicate which inter-RAT SHR triggering condition was met - UE location Information - Considering there might be parallel discussion in RAN2, RAN3 would like to check the following: - Q5: Can RAN2 confirm the support for above parameters for inter-RAT SHR from NR to LTE? Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

For Q1, R17 SHR is focus on intra-NR, for R18 inter-RAT SHR, we do not expect to affect the LTE spec, and i.e. only NR spec will be updated for inter-RAT SHR.

Proposal 9: RAN2 discuss to avoid LTE spec impact.

For Q2, RAN3 agreed that HO from NR to LTE will be treated first for intra-system inter-RAT, based on proposal 8, in order to avoid affecting LTE spec, T310 and T312 trigger conditions can be support and configured by NR cell, but T304 trigger condition which is configured by LTE cell should be avoided, i.e. not support T304 trigger condition for HO from NR to LTE.

Proposal 10: T304 trigger condition is not supported for HO from NR to LTE.

Based on P9 and P10, in case of HO from NR to LTE, it is reasonable that the inter-RAT SHR is always encoded in source RAT format, i.e. NR format, and should be reported to NR cell when the UE is back to NR.

Proposal 11: The inter-RAT SHR is always encoded in source RAT format, i.e. NR format, and is reported to NR cell when the UE is back to NR in case of HO from NR to LTE.

About Q5, all the potential contents for inter-RAT successful handover listed in RAN3 LS [3] are aligned with intra-NR SHR and can be supported.  From RAN2 perspective, the current intra-NR SHR should be extended to include LTE target cell identity and measurement results.

Proposal 12: Introduce LTE target cell identity and measurement results in SHR to support inter-RAT SHR.

Conclusion

According to the discussion in section 2, we propose:

Proposal 1: UE can store SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE.

Proposal 2: It is MN to retrieves the SPR from UE.

Proposal 3: The SPR configuration is configured only by SN, i.e. source SN configures T310 and T312 thresholds, target SN configures T304 threshold.

Proposal 4: It is source SN to send the T310 and T312 thresholds of SPR configuration to UE, and MN (for inter-SN PSCell change and PSCell addition) or source/target SN (for intra-SN PSCell change) to send T304 threshold of SPR configuration to UE.

Proposal 5: CPAC candidate cell flag can reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

Proposal 6: Random access related information is included in SPR only when the SPR is triggered due to T304 exceeds the configured threshold.

Proposal 7: Location information is included in SPR.

Observation 1: There are following three signaling designs for SPR:

Option1: Reuse PCell SHR;

Option2: PSCell SPR embedded in PCell SHR;

Option3: PCell SHR and PSCell SPR are separate.

Proposal 8: UE records/reports PCell SHR and PSCell SPR separately.

Proposal 9: RAN2 discuss to avoid LTE spec impact.

Proposal 10: T304 trigger condition is not supported for HO from NR to LTE.

Proposal 11: The inter-RAT SHR is always encoded in source RAT format, i.e. NR format, and is reported to NR cell when the UE is back to NR in case of HO from NR to LTE.

Proposal 12: Introduce LTE target cell identity and measurement results in SHR to support inter-RAT SHR.

Reference

- R2-2210986	[AT119bis-e][802][R18 SON/MDT] SHR and SPR (Ericsson)

- RAN2-119-e-SONMDT-HU _2022-10-19 0535 UTC

- R3-226003 LS on inter-RAT SHR and SPCR