3GPP TSG-RAN WG2 Meeting #120	R2-2211618

# Toulouse, France, 14 – 18 November 2022

Agenda item:		8.10.2

Source:	Intel Corporation

Title:	Enhanced FDM solution for IDC

Document for:		Discussion and Decision

# Introduction

In RAN2#119-e meeting, following was agreed regarding enhanced FDM solutions for IDC:

⇒ The Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18. ⇒ The Adjacent channel interference between SN (NR) of MR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18. ⇒ NE-DC is not considered; We will work on NR freq as SA NR case. ⇒ We will not consider the enhancements on E-UTRA freq for EN-DC scenario. FFS, on signalling details; ⇒ The IMD interference from simultaneous Tx in EN-DC to non-3GPP should be considered for the FDM enhancement in Rel.18. ⇒ The IMD interference from simultaneous Tx in NR-DC to non-3GPP should be considered for the FDM enhancement in Rel.18. Note: the solution (on freq granularity) for adjacent can be reused for IMD, we will not invent new solution on freq granularity for IMD. FFS on signalling details. ⇒ Granular indications of the affected NR frequency reported for IDC issue needs to consider both serving and non-serving frequency as in the legacy FDM solution.

Email discussion “[Post119-e][650][IDC] Comparison of FDM solutions (Ericsson)” [1] was held to analyze and compare FDM solutions.

In this contribution, we discuss enhanced FDM solution for IDC.

# Discussion

## Comparison of FDM solutions

In email discussion “[Post119-e][650][IDC] Comparison of FDM solutions (Ericsson)”, following solutions were discussed for enhanced FDM solution of NR frequencies:

Option 1: Central frequency + Bandwidth of the actual affected frequency range.

Option 2: Starting frequency + Ending frequency of the actual affected frequency range.

Option 2a: Starting frequency + Bandwidth of the actual affected frequency range.

Option 3: BWP-based reporting using BWP ID.

Option 4: BWP-based reporting using BWP ID + PRB index.

Option 6: Resource Block Group (RBG) based reporting.

Option 1/2/2a/4/6 are all based on reporting a frequency region, with the difference mainly about indication method and granularity. The key problem of Option 3 is limited information / granularity. There can be a maximum of 4 BWPs configured per serving cell and 4 BWPs cannot provide much information regarding IDC interference. Since IDC interference is related to various factors e.g. channel conditions in both RAT as well as UE implementation, gNB might not have good knowledge to configure BWPs which are related to IDC interference.

Observation 1: Using BWP for frequency resource indication is not suitable due to less information from 4 BWPs and inherent lack of knowledge of IDC interference when gNB configures BWPs.

Proposal 1: Affected frequency region (instead of BWP ID) is indicated as enhanced IDC FDM solution.

Option 1/2/2a/4/6 are compared below on various aspects.

A. The pair of values for the frequency region:

1) Center frequency + bandwidth: Option 1,

2) Start frequency + end frequency: Option 2/4

3) Start frequency + bandwidth: Option 2a/6

In this aspect, the differences between these options are more or less a matter of taste, except that in option 2, both the start and end frequency are indicated in ARFCN-ValueNR.

B. How to indicate frequency:

1) ARFCN-ValueNR: Option 1/2/2a

2) Frequency index + PRB index: Option 4

3) Frequency index + RBG index: Option 6

Since PRB and RBG are defined in BWP, both Option 4 and 6 are based on BWP configuration. For non-serving cells, gNB can configure a BWP so that UE can report PRB / RBG information. For Option 1/2/2a, absolute frequency is indicated. Although it seems that no need to provide additional information for non-serving cells (in addition to carrier frequency) for Option 1/2/2a, there is a need that UE knows the bandwidth of the non-serving cell so that it can assess whether / which affected frequency resource to be reported. Otherwise UE may report IDC interference for frequency resources that gNB is not interested in, which may introduce unnecessary UL signalling. The reason is that maximum channel bandwidth can be as large as 100 MHz in FR1 (as in TS 38.101-1, which is copied below), while the actual channel bandwidth might be much smaller.

TS 38.101-1 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 121 | 135 |

Proposal 2: Option 1/2/2a/4/6 need information related to bandwidth to be configured for non-serving cell.

C. How to indicate bandwidth:

a) PRB: Option 4

b) RBG: Option 5

c) FFS: Option 1/2a

D. Comparison of indication granularity:

There are granularity differences for indicating frequency and bandwidth. For frequency, there are mainly three granularity levels: RBG, PRB, and ARFCN-ValueNR.

In TS 38.331, definition of ARFCN-ValueNR is as follows:

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::=               INTEGER (0..maxNARFCN)

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

Relevant part in TS 38.101-1 clause 5.4.2.1 is copied below:

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

It can be seen that ARFCN-ValueNR has the granularity of 5 kHz and 15 kHz in FR1. PRB consists of 12 subcarriers, and has the granularity of 180/360/720 kHz for 15/30/60 kHz SCS respectively. RBG contains 2 to 16 PRBs, therefore the granularity is coarser than that of PRB. Note that although ARFCN-ValueNR has the finest granularity, from gNB scheduling perspective, the basic scheduling unit is PRB or RBG, therefore the finer granularity of ARFCN-ValueNR does not provide much help to gNB scheduler.

Observation 2: ARFCN-ValueNR has the finest granularity, but might be an overkill for gNB scheduling.

E. Comparison of signalling overhead:

Since the range of ARFCN-ValueNR is 0..3279165, 22 bits are needed to indicate ARFCN-ValueNR.

For RBG, 5 and 4 bits are needed for Configuration 1 and 2, respectively. For PRB, 9 bits are needed according to ASN.1 definition for PRB-Id below:

PRB-Id ::=                          INTEGER (0..maxNrofPhysicalResourceBlocks-1)

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

For Option 4 and 6, UE needs to indicate carrier frequency index among candidateServingFreqListNR, which is configured by gNB (with ASN.1 code shown below) and has the size of 128. Therefore 7 bits are needed to indicate the carrier frequency.

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

maxFreqIDC-r16                     INTEGER ::= 128     -- Max number of frequencies for IDC indication.

For Option 1 and 2a, there are no details on how bandwidth is indicated. To match the frequency granularity of 5 and 15 kHz of ARFCN-ValueNR, it might be desirable to use the same granularity for bandwidth indication. To make the analysis simple, it is assumed that PRB is used to indicate the bandwidth, which is favorable to Option 1 and 2a from signalling perspective.

Signalling overhead is analyzed in Table 1 below.

Table 1: Signalling overhead comparison for FDM solutions

| Option | Signalling component (with # of bits in bracket) | Overhead (bit) |
| --- | --- | --- |
| 1 | ARFCN-ValueNR (22) + # of PRBs (9) | 31 |
| 2 | ARFCN-ValueNR (22) + ARFCN-ValueNR (22) | 44 |
| 2a | ARFCN-ValueNR (22) + # of PRBs (9) | 31 |
| 4 | Frequency index (7) + PRB index (9) + PRB index (9) | 25 |
| 6 | Frequency index (7) + RBG index (5) + # of RBGs (5) | 17 |

It is obvious that the signalling overhead is largest for option 2 since end frequency is also indicated in ARFCN-ValueNR, but the start and end frequency should be relatively close in terms of ARFCN-ValueNR. Option 4 and 6 has less signalling overhead thanks to the signalling saving of frequency indication (the signalling overhead of combined frequency index + PRB / RBG index is less than ARFCN-ValueNR) as well as bandwidth indication. Given that fine granularity provided by ARFCN-ValueNR is not helpful to gNB scheduler (as in Observation 2), it is proposed that frequency indication for IDC FDM solution is based on frequency index + PRB or RBG index.

Proposal 3: Frequency indication for IDC FDM solution is based on frequency index + PRB or RBG index.

## Details for RBG based solution (Option 6)

As discussed in previous section, it is more efficient that UE reports IDC issue in the unit of RBG and PRB. Comparing RBG vs. PRB, RBG has the benefit of less signaling overhead, and is also flexible in size since there are 2 configurations, as defined in TS 38.214 clause 5.1.2.2.1 (DL) and 6.1.2.2.1 (UL), with the relevant table copied below for reference.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

Considering above discussion, it is proposed that the unit for enhanced frequency resource indication is RBG.

Proposal 4: The unit for enhanced frequency resource indication for IDC FDM solution is RBG.

There can be different approaches to indicate affected frequency resource, e.g. bitmap or start-length. Start-length pair approach has less signalling overhead, and is sufficient for IDC purpose. Start-length pair can reuse the signalling-efficient approach used in RAN1 DCI signalling, i.e. resource indication value (RIV) approach as defined in TS 38.214 clause 5.1.2.2.2, as copied below:

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block (RBstart) and a length in terms of contiguously allocated resource blocksLRBs. The resource indication value is defined by

if (LRBs-1)≤NBWPsize/2 then

RIV=NBWPsize(LRBs-1)+RBstart

else

RIV=NBWPsize(NBWPsize-LRBs+1)+(NBWPsize-1-RBstart)

where 1 and shall not exceed NBWPsize-RBstart.

Proposal 5: Start-length pair is used to indicate frequency resource affected by IDC problem. Signalling efficient approach like resource indication value (RIV) used by RAN1 DCI can be considered.

As discussed in previous section, to apply enhanced FDM solution for non-serving frequencies, additional information (e.g. bandwidth) should be provided for non-serving cell. To make RBG solution common for serving and non-serving frequency, a BWP can be configured (e.g. using existing IE BWP, as copied below) for non-serving frequency for IDC reporting purposes. RBG configuration (1 or 2) can be either fixed (e.g. configuration 2) or explicitly configured.

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }        OPTIONAL    -- Need R

}

Proposal 6: For RBG based indication in non-serving cell, a BWP can be configured, and RBG configuration can be either fixed or explicitly configured.

## Indication for DL and UL

When there is an IDC problem for a serving frequency, either DL, UL, or both DL/UL can be affected. This is reflected by IE interferenceDirection-r16, as from related ASN.1 and field description copied below.

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

UEAssistanceInformation field descriptions

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

Therefore, enhanced frequency resource indication should be signalled for DL and UL independently, to allow flexible and efficient scheduling.

Proposal 7: Enhanced frequency resource indication is signalled for DL and UL independently.

# Conclusion

In this contribution, we discuss enhanced FDM solution for IDC, and have the following observations:

Observation 1: Using BWP for frequency resource indication is not suitable due to less information from 4 BWPs and inherent lack of knowledge of IDC interference when gNB configures BWPs.

Observation 2: ARFCN-ValueNR has the finest granularity, but might be an overkill for gNB scheduling.

We propose the following:

Proposal 1: Affected frequency region (instead of BWP ID) is indicated as enhanced IDC FDM solution.

Proposal 2: Option 1/2/2a/4/6 need information related to bandwidth to be configured for non-serving cell.

Proposal 3: Frequency indication for IDC FDM solution is based on frequency index + PRB or RBG index.

Proposal 4: The unit for enhanced frequency resource indication for IDC FDM solution is RBG.

Proposal 5: Start-length pair is used to indicate frequency resource affected by IDC problem. Signalling efficient approach like resource indication value (RIV) used by RAN1 DCI can be considered.

Proposal 6: For RBG based indication in non-serving cell, a BWP can be configured, and RBG configuration can be either fixed or explicitly configured.

Proposal 7: Enhanced frequency resource indication is signalled for DL and UL independently.

# References

[1] R2-2212420, Ericsson, "Report from [Post119-e][650][IDC] Comparison of FDM solutions (Ericsson)"