3GPP TSG-RAN2 Meeting #120	R2-2211627

# Toulouse, France, November 14th - 18th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3623 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on when to consider SDT procedure is not ongoing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | R17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#119 e-meeting, as the handling of T319a is ambiguous when the start of the timer is delayed, the sentences T319a running and not running are replaced with SDT ongoing and not ongoing respectively.Currently, when SDT procedure is successful or unsuccessful, it is clearly specified in some cases that the UE stops T319a while “consider SDT procedure is not going”. These cases includes entering to RRC_CONNECTED (via RRCResume or RRCSetup), continuing in RRC_INACTIVE (via RRCRelease or RRCReject), expiry of T319a, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission. But in some cases, e.g. the UE enters RRC_IDLE upon reception of RRCRelease message, or the UE enters RRC_IDLE due to cell re-selection during SDT is ongoing, there is no explicit behavior that the UE considers SDT procedure is not going. Although in the above two cases the UE will perform the actions upon going to RRC_IDLE as specified in 5.3.11, there is no explicit behavior that the UE considers SDT procedure is not going either. Then it is not clear if the UE considers SDT procedure is not going in the above two cases. With the unclear behavior, some mistake may be led. For example: In T1, the UE is performing SDT procedure. Then in T2, the UE enters RRC_IDLE upon reception of RRCRelease message or upon cell reselection. And the UE still considers SDT procedure is going. After a while, the UE enters RRC_CONNECTED in T3 and it enters RRC_INACTIVE after a later time (e.g. in T4). If in the subsequent time (e.g. T5), the UE performs normal cell reselection in RRC_INACTIVE, the UE will enter RRC_IDLE according to 5.3.13.6 as shown below as the UE considers SDT procedure is ongoing. But it is not the desired UE behaviour. 5.3.13.6 Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured The UE shall: 1> if cell reselection occurs while T319 or T302 is running or while SDT procedure is ongoing; or 1> if relay reselection occurs while T319 is running; or 1> if cell changes due to relay reselection while T302 is running: 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure';. |
| Summary of change: |  | Add a clarification in 5.3.11 (UE actions upon going to RRC_IDLE) that the UE considers SDT procedure is not ongoing if T319a is running. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: SDT Inter-operability: - If the UE is implemented according to this CR and the network is not then there is no interoperability issue. - If the network is implemented according t this CR but the UE is not then there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may still consider SDT procedure is going when entering RRC_IDLE upon reception of RRCRelease message or when entering RRC_IDLE due to cell re-selection during SDT is ongoing. Some mistake may be led. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

1>	reset MAC;

1>	set the variable pendingRNA-Update to false, if that is set to true;

1>	if going to RRC_IDLE was triggered by reception of the RRCRelease message including a waitTime:

2>	if T302 is running:

3>	stop timer T302;

2>	start timer T302 with the value set to the waitTime;

2>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2'.

1>	else:

2>	if T302 is running:

3>	stop timer T302;

3>	perform the actions as specified in 5.3.14.4;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the UE is leaving RRC_INACTIVE:

2>	if going to RRC_IDLE was not triggered by reception of the RRCRelease message:

3>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities;

3>	stop the timer T320, if running;

2> if T319a is running:

3> stop T319a;

3> consider SDT procedure is not ongoing;

1>	stop all timers that are running except T302, T320, T325, T330, T331 and T400;

1>	discard the UE Inactive AS context, if any;

1>	release the suspendConfig, if configured;

1>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

1>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

2>	for the associated reportConfigId:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

3>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

1>	discard the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if any;

1>	release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	inform upper layers about the release of all application layer measurement configurations;

1>	discard any application layer measurement reports which were not yet submitted to lower layers for transmission;

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running or when selecting an E-UTRA cell for EPS fallback for IMS voice as specified in 5.4.3.5:

2>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

End of change