3GPP TSG-RAN2 WG2 Meeting #120	R2-2211653

# Toulouse, France,  14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3626 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SRS Release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | From TS38.321, upon SR failure, MAC notifies RRC to release SRS for all Serving Cells. In RRC specification, it has been described to only release SRS-Resource instances configured in SRS-Config, but there is no description on the set of SRS-Resources, i.e. SRS-ResourceSet. Since SRS-Resources composing SRS-ResourceSet are released, the SRS-ResourceSet has to be released as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the procedual text in clause 5.3.12 “1> release SRS-Reseources instances configured in SRS-Config.” to “1> release SRS-ReseourceSet and SRS-Resource instances configured in SRS-Config.” Impact Analysis: Impacted 5G architecture options: All architectures Impacted functionality: SRS release Interoperability issue: - if the network is implemented according to the CR and the UE is not, as the network configures new SRS-Resource(s) and SRS-ResourceSet(s), UE may misunderstand it due to the old SRS-ResourceSet(s). - if the UE is implemented according to the CR and the network is not, as the network may request to release SRS-ResourceSet(s) which UE have already released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior on SRS release remains ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== The unchanged skipped =====

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-ReseourceSet and SRS-Resource instances configured in SRS-Config.

Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall:

1>	release the configured srs-PosRRC-Inactive.

===== The unchanged skipped =====