3GPP TSG-RAN WG2#120	R2-2211661

Toulouse, France, 14-18 November 2022

Agenda Item:	8.2.2

Source: 	MediaTek Inc.

Title:  	Server UE functions

Document for:	Discussion, decision

# 1	Introduction

In [1], SA2 pose questions about sidelink positioning to RAN WGs. This document addresses which positioning server UE functions are “reasonable” from the RAN2 point of view.

It is understood in SA2 that a positioning server UE can perform positioning calculation. The LS lists the following additional functions that could be considered:

- SL positioning/ranging method determination

- Operation coordination

- Resource coordination and scheduling

We discuss these functions separately. We assume that “reasonable” comprises two questions: (1) Is the function feasible from RAN2 perspective? and (2) Does the function offer some benefit from RAN2 perspective?

(We use the name RSPP for the protocol over PC5 between target/anchor/server UEs, since SA2 seem to have converged on it. Note that SLPP is already used in the LCS context for Subscriber LCS Privacy Profile.)

# 2	Discussion

## 2.1	Positioning method determination

On Uu, of course, the LMF determines the positioning method(s). We understand this should also be the case for hybrid Uu+PC5 positioning and for PC5-only positioning when the LMF is involved (it would be very strange to expropriate this function from an LMF). The open question is about PC5-only positioning where a server UE is involved.

In case the server UE has been discovered and selected to do the position calculation, the entity that selects the positioning method needs to know the following information:

- Capabilities of the server UE;

- For absolute positioning, whether the server UE knows (or can determine) the locations of the involved anchor UE(s).

It is clear how this works if the server UE selects the method. For another UE (target or anchor) to select the method, that UE needs to receive the server capabilities (introducing a new RSPP procedure: server capability transfer to target/anchor) and to somehow discover the server UE’s knowledge of anchor UE locations (introducing an entirely new class of RSPP procedure). In addition, the selected method needs to be indicated to the server (another new RSPP procedure), and only then can the server request the measurements it needs to compute the location estimate.

Observation 1a: If a target or anchor UE selects the positioning method when a server UE computes the location estimate, the target/anchor UE must know the server UE’s capabilities and (at least for absolute positioning) which anchor UEs the server UE knows the location of, and the selected method needs to be indicated to the server.

On the other hand, if the server selects its own method, it immediately knows its own capabilities and anchor UE awareness. In such a case, the server can behave like an LMF to select a suitable positioning method. This selection involves no over-the-air signalling and can be viewed from RAN2 perspective as an autonomous function of the server UE. There is no spec impact for RAN2 to enable this functionality.

Observation 1b: If the server UE selects the positioning method when it computes the location estimate, it knows its own capabilities and anchor UE awareness without additional procedures or RAN2 impact.

It is not clear why positioning method selection should take place at a node that does not automatically know the capability of the position calculation function. Observation 1a above suggests that it might be technically possible to enable this, but it has complexity and latency costs and no benefits.

Observation 1c: Positioning method selection by the server UE is reasonable from RAN2 perspective, at least for the case where the server UE computes the location estimate.

## 2.2	Operation coordination I: Anchor UE selection

The LS is not explicit about what “operation coordination” means, but we understand that it comprises such functions as anchor UE selection and SL-PRS configuration (as described in [2], the configuration of parameters such as sequence and comb that are not directly related to resource coordination and scheduling). The equivalent functions on Uu belong to the LMF. This section examines anchor UE selection; section 2.3 below considers SL-PRS configuration.

Anchor UE selection presents some challenges for any solution:

- If the server UE selects the anchor UEs, it needs to determine which anchors are (likely to be) visible to the target UE.

- If the target UE selects the anchor UEs, it needs to determine the capabilities of the candidate anchor UEs, so that it does not, for example, select anchor UEs that then cannot transmit SL-PRS with a configuration that will meet the needed accuracy.

- If the anchor UEs offer themselves without coordination, it is not clear how they know what operation they are volunteering for (e.g., if they will be asked to transmit or receive SL-PRS or both, or with what parameters), nor how they know when enough anchor UEs have been found and they can stop volunteering.

Different use cases may have very different populations of candidate anchor UEs. In V2X use cases, absolute positioning might depend on using RSUs as anchors (since they are stationary and may not have privacy concerns about their locations), but ranging between two vehicles would need to use the peer vehicle as the anchor almost by definition. In the first case, it might make sense for the RSUs to advertise their availability as anchor UEs during discovery, so the population of candidates could be known in advance to either the target UE or the server UE. In the second case, especially in a latency-critical case like collision avoidance, it makes sense for the target UE to discover its own anchor UE.

In general, the expected behaviour for anchor UE selection depends on what the criteria for selection are:

- Is any UE authorised to volunteer as an anchor UE without any checking? (Is there a security issue if a malicious UE acts as an anchor and crafts signals intended to result in incorrect location results?)

- Is any target UE authorised to work with any anchor UE that makes itself available?

- Is an anchor UE guaranteed to know its own location, with uncertainty, as an input to the position calculation function for absolute-positioning cases?

- Should a candidate anchor UE have PC5 connectivity to both the target and the server? (Without a connection to the server, an anchor UE might take measurements in a target-to-anchor case, but then be unable to deliver them to the position calculation function. For anchor-to-target cases, if the server UE is responsible for SL-PRS configuration as discussed in the next section, it would not be easily able to configure the reference signals for an anchor UE without a connection to the server.)

- Is the target UE always able to determine from the location request what characteristics would be needed in SL-PRS transmission (even if the target UE does not compute the location estimate)?

Observation 2a: From RAN2 perspective, it is not obvious what the criteria for anchor UE selection should be in different cases, and therefore it is not obvious whether this function is easier to locate at the server UE or somewhere else (e.g., target or anchor UE(s)).

Although observation 2a above applies for the general case, some conclusions can be made from RAN2 perspective about specific cases.

- Authorisation of UEs to act as anchors is outside RAN2 scope, but RAN2 can understand that some node must determine the authorisation, so that only allowed anchor UEs are discovered/used for positioning. This information seems not easily accessible to the target or to other anchor UEs, but if the server marshals the pool of available anchor UEs, it can know in advance what operations the candidate anchor UEs are authorised for.

- An anchor UE with limited capabilities and cost, such as an IoT device, might not have a positioning application in its upper layers and might not know its own location. Whether this limitation affects the selection of such an anchor UE for positioning the target UE depends on the positioning method, QoS criteria, absolute vs. relative positioning, etc., which cannot easily be known to the anchor UE.

- A target UE with limited capabilities and cost might be unduly burdened if it is required to perform intelligent anchor UE selection and configuration to meet the criteria of a location request.

- It may be important for anchor UEs to have sidelink connections (at least to be close enough to establish a connection) to both the target UE and the server UE. However, the importance depends on RAN1 work (will SL-PRS be hearable/measurable for UEs that are out of PC5 connection range?) and SA2 decisions (is the server UE expected to configure SL-PRS for the anchors?).

- If the server UE provides the anchor UE information to the target UE using the assistance data framework, the server UE can select anchor UEs using information embedded in the assistance data request (e.g., about which anchor UEs are visible to the target).

Observation 2b: RAN2 are not ready to take a general decision on which node should select anchor UEs; there are RAN1 and SA2 dependencies. Any alternative—selection by the target UE, the server UE, or the anchor UEs themselves—has challenges that will need to be addressed. However, selection by the server UE, as asked about by SA2, appears reasonable from RAN2 perspective if combined with the assistance data framework.

## 2.3	Operation coordination II: SL-PRS configuration

On Uu, DL-PRS and SRSp configuration are handled between the gNB and the LMF. The LMF knows what signal characteristics are needed to meet the requirements of a positioning request; the gNB knows what resource usage can be supported on the Uu interface. The situation on sidelink is somewhat similar, with the target and/or server knowing the needs of the positioning request and the target and anchor UE(s) having some idea of what can be carried on the PC5 interface (not as complete as a gNB’s knowledge of the Uu interface).

Observation 3a: The anchor UE(s) do not have knowledge of the requirements of a particular positioning request.

Observation 3b: The server UE does not know the state of the PC5 interface between the target UE and the anchor UE(s).

No single entity is perfectly positioned to configure the SL-PRS by itself. The easiest approach to implement, from RAN2 perspective, may be something like DL-PRS configuration for Uu: The server (LMF, resp. server UE) requests from the transmitters (gNBs, resp. anchor UEs and/or target UE) a reference signal configuration that will meet the needs of the positioning method, and each transmitter responds with whether it can comply. This model assumes that the server has foreknowledge of the transmitters’ capabilities, which is straightforward to support with the RSPP capability procedure. It does not assume that the server allocates specific resources, and it leaves to the anchor UEs and/or the target UE the final decision about what can be supported in the PC5 interface.

Observation 3c: Configuration of SL-PRS by the server UE, resembling DL-PRS configuration for Uu positioning (and not including resource allocation), is reasonable from RAN2 perspective.

Configuration by the server UE might not be the only method—SA2 only asked about the reasonableness of the server UE operations, and the RAN2 reply should not be understood to exclude support for other configuration approaches.

Observation 3d: Other approaches to configuration of SL-PRS, such as negotiation directly between the target and the anchor(s), could be considered, with no impact on the reply LS to SA2.

## 2.4	Resource coordination and scheduling

The coordination and scheduling of resources for SL-PRS (which we assume is the intention of this question from the LS) is largely in RAN1 scope, and we understand it is already in discussion in RAN1. However, RAN1 may not have much visibility into the concept of the server UE up to this point.

Centralizing resource coordination and scheduling for SL-PRS might make sense, similar to centralizing control of DL-PRS or UL-SRS at the gNB. A single control point allows coordination of resources to avoid collisions, for example. On the other hand, the joint optimisation problem of coordinating scheduling between a target UE and N anchor UEs may be difficult.

RAN2 may not be the best-positioned group to answer this question, at least until RAN1 have given some further information about their plans for the resource management framework.

Observation 4a: RAN1 need to progress on the management of sidelink radio resources between the target UE and the anchor UE(s) before RAN2 can indicate if resource coordination and scheduling by the server UE would be reasonable.

## 2.5	Example procedures

There are many possible combinations of the operations under consideration. As an example of the tradeoffs, Figures 1 and 2 show a case where an SL-TDOA positioning method with anchor-to-target (a2t) SL-PRS transmission is selected, either by the target (Figure 1) or the server (Figure 2), and the anchor UEs are selected either by the target (Figure 1) or the server (Figure 2). The SL-PRS configuration is also shown as being initiated by the target (Figure 1) or the server (Figure 2). In both cases, the server performs the position calculation.

Note that the figures only show two anchor UEs (for space reasons), and only anchor UE 1 is selected for use. This minimum configuration might be adequate for ranging but would not work for most positioning operations—however, it fits in a legible figure. The generalisation to multiple anchor UEs should be straightforward.

Figure 1: Target UE selects method and anchors, and configures SL-PRS

The procedure of Figure 1 introduces three new RSPP operations (steps 1, 3, and 5). Steps 3 and 7 might conceivably be combined, if all the possible assistance data for all candidate anchor UEs and all candidate positioning methods are provided in one message (step 3b), but it is not clear if this would be feasible without prior knowledge of the method at the server and selection of the anchor UEs.

Considering the need for the server UE to know the population of anchor UEs so that it can provide assistance data in step 7b, the target UE could indicate the list of selected anchor UEs in step 5 or step 7a.

Figure 2: Server UE selects method and anchors, and configures SL-PRS

In general, the server-centric procedure of Figure 2 follows LPP behaviour, and it saves several signalling exchanges compared to the target-centric procedure of Figure 1.

## 2.6	Ownership of the location request

The sidelink equivalent of the LCS framework is outside RAN2 scope, but it is important to understanding the requirements of the positioning system as a whole. Requests can come from a third party (the SL Positioning Client UE), but the pCR in [3] suggests that the request always comes to the target and/or anchor UEs, not the server UE (“communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established…for the transmission of the service request and the result”—in NOTE X under the definition of the SL Positioning Client UE).

If this implication holds, it seems to follow that sidelink positioning resembles an MO-LR operation, in the sense that even if the position calculation function and/or other functionalities are hosted at a server UE, the first interaction with the server UE would be originated by the target UE (“please help position me”), or conceivably by an anchor UE (“please help me position this target UE”). The indication that the result of the positioning operation is delivered to the SL Positioning Client UE by the target/anchor UE(s) also suggests that if the position calculation function is hosted at the server UE, the server UE has to deliver the location to the target or an anchor UE for it to be delivered to the client UE.

Observation 5a: When a location request comes from a SL Positioning Client UE, the related SA2 pCR indicates that the request goes to the target and/or anchor UE(s), not to the server UE, and that the result is returned to the SL Positioning Client UE by the target and/or anchor UE(s), not by the server UE.

This procedure seems potentially complex, and it may be useful for RAN2 to request clarification. Although RAN2 will not specify the LCS-like procedures, they may create requirements on where information is distributed by RSPP.

Observation 5b: It may be helpful for RAN2’s work in specifying RSPP to ask SA2 about the flow of information in the positioning request/result when a SL Positioning Client UE is involved.

# 3	Conclusion

This document promulgated the following observations:

Observation 1a: If a target or anchor UE selects the positioning method when a server UE computes the location estimate, the target/anchor UE must know the server UE’s capabilities and (at least for absolute positioning) which anchor UEs the server UE knows the location of, and the selected method needs to be indicated to the server.

Observation 1b: If the server UE selects the positioning method when it computes the location estimate, it knows its own capabilities and anchor UE awareness without additional procedures or RAN2 impact.

Observation 1c: Positioning method selection by the server UE is reasonable from RAN2 perspective, at least for the case where the server UE computes the location estimate.

Observation 2a: From RAN2 perspective, it is not obvious what the criteria for anchor UE selection should be in different cases, and therefore it is not obvious whether this function is easier to locate at the server UE or somewhere else (e.g., target or anchor UE(s)).

Observation 2b: RAN2 are not ready to take a general decision on which node should select anchor UEs; there are RAN1 and SA2 dependencies. Any alternative—selection by the target UE, the server UE, or the anchor UEs themselves—has challenges that will need to be addressed. However, selection by the server UE, as asked about by SA2, appears reasonable from RAN2 perspective if combined with the assistance data framework.

Observation 3a: The anchor UE(s) do not have knowledge of the requirements of a particular positioning request.

Observation 3b: The server UE does not know the state of the PC5 interface between the target UE and the anchor UE(s).

Observation 3c: Configuration of SL-PRS by the server UE, resembling DL-PRS configuration for Uu positioning (and not including resource allocation), is reasonable from RAN2 perspective.

Observation 3d: Other approaches to configuration of SL-PRS, such as negotiation directly between the target and the anchor(s), could be considered, with no impact on the reply LS to SA2.

Observation 4a: RAN1 need to progress on the management of sidelink radio resources between the target UE and the anchor UE(s) before RAN2 can indicate if resource coordination and scheduling by the server UE would be reasonable.

Observation 5a: When a location request comes from a SL Positioning Client UE, the related SA2 pCR indicates that the request goes to the target and/or anchor UE(s), not to the server UE, and that the result is returned to the SL Positioning Client UE by the target and/or anchor UE(s), not by the server UE.

Observation 5b: It may be helpful for RAN2’s work in specifying RSPP to ask SA2 about the flow of information in the positioning request/result when a SL Positioning Client UE is involved.

We propose to take these observations into account in responding to SA2.

Proposal 1: Respond to SA2 that:

- Positioning method determination by the server UE is reasonable from RAN2 perspective;

- With respect to operation coordination by the server UE:

  - Anchor UE selection by the server UE has RAN1 and SA2 dependencies, but it is reasonable from RAN2 perspective;

  - SL-PRS configuration by the server UE is reasonable from RAN2 perspective, assuming the configuration is negotiated between the server UE and the transmitter (target and/or anchor);

- Resource coordination and scheduling by the server UE should be considered in light of RAN1 decisions on resource management for sidelink positioning, and RAN2 are unsure if it is reasonable without further information;

- RAN2 would benefit from more detailed information about the interaction between the SL Positioning Client UE and the target/anchor/server UEs.

# 4	References

[1]	R2-2211139/S2-2209961, “LS on RAN dependency for Ranging/Sidelink Positioning”, LS from SA2 to RAN1/RAN2/RAN3, RAN2#120

[2]	R2-2209536, “SL-PRS configuration”, MediaTek Inc., RAN2#119bis-e

[3]	S2-2207117, “General: Clarification on Location Server UE and peer-to-peer operation”, MediaTek Inc., SA2#152-e