3GPP TSG-RAN WG2 Meeting #120		R2-2211665

Toulouse, France, 14th Nov – 18th Nov, 2022

Agenda Item:	8.3.3

Source:	vivo

Title:	discussion on SSB_SIB-less_paging

Document for:	Discussion and Decision

Introduction

According to post119bis-e discussion, the following TP for NES Cell without SIB was proposed[1]:

6.1.3.x Higher layer procedures NES cell without SIB is only applicable in multi-carrier scenario, where there is an anchor cell and one or multiple NES cell(s) without SIB in different carrier(s). Anchor cell is a cell where NES-capable UE assumes SSB, system information and paging are transmitted. Editor's note: FFS: The system information transmitted by anchor cell also includes the necessary information for NES-capable UEs to access via an NES cell without SIB. An NES cell without SIB omits the transmission of SIB, or omits the transmission of both SSB and SIB. Editor's note: FFS the details on how to support NES cell without SIB or without SSB and SIB. Editor's note: FFS whether paging enhancements, e.g. no paging transmission, is applied to the scenarios where SSB and/or SIB is not transmitted on the NES cells.

In this contribution, we will discuss about various aspects regarding NES Cell without SIB.

Discussion

Background

Dating back to Rel-13 NB-IOT, considering that the maximum bandwidth of a carrier is 200kHz, and the capacity is limited in a carrier when taking the expenditure of NPSS/NSSS/NPBCH/SIB into account, to support massive NB-IOT equipments accessing the cell, the concept of (non-)anchor carrier was introduced in NB-IOT. In this release, the UE receive paging and perform RACH only on the anchor carrier. The NB-IOT UE can first access the cell on anchor carrier and reconfigured to a non-anchor carrier for data transmission/reception. Once the data transmission/reception is completed, the UE enter RRC_IDLE and camp on the anchor carrier.

Later in Rel-14, paging and RACH are supported to be offloaded to non-anchor carrier to alleviate the load of anchor carrier. SIB1 can also be offloaded for TDD mode, the NB-IOT UE can determine whether it receives SIB1 on anchor carrier or non-anchor carrier by reading the information in MIB [2]:

MasterInformationBlock-TDD-NB

-- ASN1START

MasterInformationBlock-TDD-NB-r15 ::=	SEQUENCE {

systemFrameNumber-MSB-r15				BIT STRING (SIZE (4)),

hyperSFN-LSB-r15						BIT STRING (SIZE (2)),

schedulingInfoSIB1-r15					INTEGER (0..15),

systemInfoValueTag-r15					INTEGER (0..31),

ab-Enabled-r15							BOOLEAN,

operationModeInfo-r15				CHOICE {

inband-SamePCI-r15					Inband-SamePCI-TDD-NB-r15,

inband-DifferentPCI-r15				Inband-DifferentPCI-TDD-NB-r15,

guardband-r15						GuardbandTDD-NB-r15,

standalone-r15						StandaloneTDD-NB-r15

},

sib1-CarrierInfo-r15					ENUMERATED {anchor, non-anchor},

ab-Enabled-5GC-r16						BOOLEAN,

spare									BIT STRING (SIZE (8))

}

...<irrelevant IEs>

-- ASN1STOP

As for FDD mode, SIB1 is still transmitted only on anchor carrier.

- Paging and RACH can be performed either on a anchor carrier or on a non-anchor carrier in NB-IOT.

Since we are discussing NES cell without SIB, the rest parts of this contribution will only be related to paging and RACH. We initiate the following discussion based on the anchor cell definition from [1], i.e. anchor cell is a cell where NES-capable UE assumes SSB, system information and paging are transmitted.

RACH impacts

Before we discuss about RACH procedure for NES cell without SIB, we can first check how it is designed for legacy NB-IOT and see if there is any mechanism that can be reused.

In NB-IOT, the concept of enhanced coverage level (CEL) was introduced, because NB-IOT UEs are deployed in various environment and the UEs may suffer from severe pathloss. CELs are devided into CEL0/CEL1/CEL2 as illustrated Figure.1. When UE performs RACH, it needs to determine its CEL, and CEL2 may indicate that the UE is suffering the worst pathloss:

Figure.1 illustration of enhanced coverage level in NB-IOT

The UE obtains the parameters for NPRACH on (non-)anchor carrier in SIB2-NB [2]. When UE is to perform RACH, the UE measures the RSRP of NSSB on the anchor carrier and determines its enhanced coverage level (CEL). Take CEL0 for example, the UE then uses the NPRACH parameters referring to CEL0 and choose a RACH carrier for with RACH resources is configured using nprach-ProbabilityAnchorList-r14.

- RACH in NB-IOT is based on CEL determination and probability-based carrier selection method.

Now we can have a brief analysis for whether the RACH method in NB-IOT can be reused for NES cell without SIB. First of all, at least for NR UEs operating in FR1, the UEs are usually indoors or outdoors, they may not suffer severe pathloss as that of the NB-IOT UEs which may be deployed underground, e.g. water meters. Therefore, there may not be enough motivation to introduce similar concept of CEL to NR UEs in this scenario. Secondly, when NR UEs are on the edge of the cell, the UEs are likely to perfrom cell reselection instead of still camping on the current serving cell, which means the UEs are not that in need to enhance RACH as that of NB-IOT UEs, e.g. a larger RACH repetition number for CEL2. In sum, we prefer not to reuse the CEL concept for NES cell without SIB.

- RAN2 to discuss whether to reuse the CEL concept in NB-IOT for NES cell without SIB.

As for the probability-based RACH carrier selection method, it was introduced in NB-IOT to balance the capacity of different carriers since an NB-IOT needs to serve massive amount of NB-IOT UEs. When we look back to the network energy saving scenario, we observe that most NES techniques are to be applied when the cell load or traffic load is low. Therefore, it seems there is little need to alleviate the RACH pressure of a NES cell as that of NB-IOT. Yet, we are open to discuss it.

- RAN2 to discuss whether to reuse probability-based RACH carrier selection method for NES cell without SIB.

To support NES cell without SIB, the following questions need to be addressed first for further discussion about RACH:

- Whether the RACH parameters of the non-anchor cell(s) can be transmitted in the SIB from the anchor cell? If yes, FFS which parameters.

- Whether the UE can get synchronization from SSB of the anchor cell? This depends on RAN4 input.

- If the answer to Q2 is no, whether SSB can be transmitted on non-anchor cell?

- If the answer to Q2 is no, and the answer to Q3 is yes, if the SSB on the non-acnhor cell is not transmitted, whether UE WUS can be triggered to require on-demand SSB on non-anchor cell? Note that the UE in this case needs to first wake-up the non-anchor cell to transmit SSB and then perform RACH procedure on it.

Since Q2~Q4 is depending on RAN4 input, we propose to only discuss Q1 in RAN2 for now.

- RAN2 to discuss whether RACH parameters of the non-anchor cell(s) can be transmitted in the SIB from the anchor cell? If yes, FFS which parameters.

Paging for NES Cell without SIB

As mentioned above, the NES techniques are usually applied when the cell load or traffic load is low, the cell can further achieve NES gain by only paging UE on the anchor cell. Although we are open to discuss whether paging can be on a NES cell without SIB, we assume that it may not be necessary to support it. If we are to support paging on a NES cell without SIB, its further discussion also rely on RAN4’s conclusion on whether the UE can get synchronization from SSB of the anchor cell.

- RAN2 to discuss whether to support paging on a NES cell without SIB. If yes, wait for RAN4’s conclusion on whether the UE can get synchronization from SSB of the anchor cell before further discussion about paging on a NES cell without SIB.

Conclusion

Based on the discussion, we have the following observation and proposals:

- Paging and RACH can be performed either on a anchor carrier or on a non-anchor carrier in NB-IOT.

- RACH in NB-IOT is based on CEL determination and probability-based carrier selection method.

- RAN2 to discuss whether to reuse the CEL concept in NB-IOT for NES cell without SIB.

- RAN2 to discuss whether to reuse probability-based RACH carrier selection method for NES cell without SIB.

- RAN2 to discuss whether RACH parameters of the non-anchor cell(s) can be transmitted in the SIB from the anchor cell? If yes, FFS which parameters.

- RAN2 to discuss whether to support paging on a NES cell without SIB. If yes, wait for RAN4’s conclusion on whether the UE can get synchronization from SSB of the anchor cell before further discussion about paging on a NES cell without SIB.

References

- R2-2211427 TP on cell selection／reselection and SSB／SIB-less

- TS 36331, V17.2.0