3GPP TSG-RAN WG2 Meeting #120                	R2-2211666

Toulouse, FR, 14th – 18th Nov 2022

Agenda Item:	8.3.4

Source: 	vivo

Title:         	Discussion on cell selection/reselection

Document for: 	Discussion and Decision

- Introduction

One objective of the NR network energy saving SID [1] is the following.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

During post RAN2#119bis email discussion [2], a TP for cell selection/reselection and SSB/SIB-less was agreed in [3]. In this contribution, we will discuss the remaing issues for cell selection/reselection, including the Editor Notes in the TP, the detailed solutions and the potential RAN2 impact.

- Discussion

From the below description in the TP, NES cells should be able to bar legacy UEs but allow NES-capable UEs, or bar both kind of UEs. Therefore, the possible solutions were proposed.

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to: - Use IntraFreqExcludedCellList/InterFreqExcludedCellList - Use the cellBarred or cell reservation fields in MIB/SIB Editor's note: FFS whether to keep the terminology of “NES cells” and its definition, or change it to “a cell that uses an NES technique”. Editor's note: FFS the exact mechanism and the spec impacts.

For the first Editor Note:

Editor's note: FFS whether to keep the terminology of “NES cells” and its definition, or change it to“a cell that uses an NES technique”.

The terminology of NES cells is mainly used to distinguish it from the legacy cells which do not support Rel-18 NES techniques. Technical discussion can be easily conducted with this terminology, so we prefer to keep “NES cells” terminology in the TP, and the definition of “NES cells” can be simply interpreted as NES-capable cells. After specific NES techniques are agreed, RAN2 can further discuss whether to change the definition from NES-capable cells to specific NES technique-capable cells.

Proposal 1: Keep the terminology of “NES cells” with the definition of NES-capable UE. Can discuss whether to change the definition when specific NES technique is agreed.

For the below Editor Note:

Editor's note: FFS the exact mechanism and the spec impacts.

For solution (Use IntraFreqExcludedCellList/InterFreqExcludedCellList):

To avoid legacy UEs camping on NES Cells, the network can configure the NES neighboring cells in existing IntraFreqExcludedCellList / InterFreqExcludedCellList via SIB3. Since the NES-capable UE has no backward compatibility issue when camping on these NES cells, the NES-capable UE can just ignore IntraFreqExcludedCellList / InterFreqExcludedCellList. However, as in legacy, the network should be also allowed to configure black lists for NES-capable UEs for cell reselection, which means additional information need to be provided to NES-capable UEs via SIB. How to configure black lists for NES-capable UEs can be further discussed. Thus, we have the following proposal:

Proposal 2: For solution (Use IntraFreqExcludedCellList/InterFreqExcludedCellList), capture the below spec impact in the TP:

- NES-capable UE ignores legacy IntraFreqExcludedCellList/InterFreqExcludedCellList;

- Additional information is added to indicate the black lists for NES-capable UEs for cell reselection.

For solution (Use the cellBarred or cell reservation fields in MIB/SIB):

To avoid legacy UEs camping on NES Cells, the network can configure cellBarred in MIB as barred. The NES-capable UE has no backward compatibility issue when camping on the NES cell, the NES-capable UE can just ignore cellBarred in MIB. However, as in legacy, the network should be also allowed to bar NES-capable UEs, which means additional bar is needed for NES-capable UEs via SIB. And RAN2 can discuss if the cellBarred in MIB set to notBarred, whether the NES-capable UE can treat this cell is not barred instead of receiving SIB1. Thus, we have the following proposal:

Proposal 3: For solution (Use the cellBarred or cell reservation fields in MIB/SIB), capture the below spec impact in the TP:

- If cellBarred in MIB is set to barred, NES-capable UE ignores it but uses cell bar in SIB1;

- If cellBarred in MIB is set to notBarred, NES-capable UE considers the cell as not barred or still uses cell bar in SIB1.

Then, we will discuss the below Editor Note:

The NW should be able to configure NES-capable UEs to prioritize/de-prioritize a specific NES cell or NES cells on a specific frequency. Editor's note: FFS whether the existing mechanism for cell prioritization/de-prioritization is enough.

Deprioritize NES cells:

In most cases, the UE’s performance will be impacted by the network energy saving operation. Regardless of whether the UE supports NES techniques, the UE does not prefer to be served by NES cells. To deprioritize NES cells for all UEs, the existing specification allows the network to do the following:

- Option 1: Configure the frequencies of the NES cells as low priority frequency for cell reselection;

- Option 2: For intra-frequency cell reselection and same priority frequency cell reselection, the network can configure proper cell offset to enable all the UEs try to camp on the neighbouring normal cells instead of the current cell who is in NES state.

For option 1, the UEs cannot perform cell reselection to the normal cells on that frequency, which may lead to the situation that the UE is more difficult to find a suitable cell during cell reselection. Moreover, NES will be mainly used for higher frequency with more band resources, so generally speaking, the network may want to set this frequency as high priority frequency for load balance. This option constraints the network implementation. And for high priority frequency cell reselection, it seems the network cannot deprioritize NES cells based on the current specification.

For option 2, if the neighbouring cells want to enter sleep mode, the current specification does not support the network to configure seperate cell offsets for normal neighbouring cells and NES neighbouring cells.

Given above, the potential impact is to introduce separate cell offset for NES cells and normal cells, which can be sent via SIB or dedicated signalling (basically for high priority frequency cell reselection). Then, the following question is how does the UE identify the NES cell and normal cell, then apply different cell offsets. This can be provided in the SIB. Moreover, since the NES-capable UE will read the bar in SIB1, then the network capability (whether the cell supports or uses NES technique) can be indicated in SIB1, then the NES-capable UE can perform cell reselection reordering after receiving SIB1.

Prioritize NES cells:

The motivation of prioritization NES cells is for load balancing. Even if the motivation is doubtful, it seems the solutions for deprioritization NES cells can also be used for prioritize NES cells. That is, if separate cell offsets are used for normal cells and NES cells, it’s up to network how to configure these offset values.

Thus, we have the following proposal:

Proposal 4: for (de)prioritization NES cells, capture the below spec impact:

- Configure separate cell offset for NES cells in SIB or dedicated signalling;

- NES cells are indicated to the UE via SIB or dedicated signalling.

Measurement for intra/inter frequency cell reselection:

When the cell A will enter NES state, the network can let all UEs to start intra/inter-frequency measurement early. However, it seems the NES-capable UE can still camp on this NES cell, the reason is that this NES cell still has high quality and performing measurement will increase UE power consumption. Thus, it is also deserved to consider whether to let the NES-capable UE not to perform intra/inter frequency cell reselection measurement when the legacy condition is fulfilled. But, a new condition can be introduced to enable the NES-capable to start intra/inter frequency cell reselection measurement when the current cell’s quality is bad. Thus, we have the following proposal:

Proposal 5: To allow the NES UE not start intra/inter frequency cell reselection measurement when legacy conditions are fulfilled, when camping on NES cell. Capture the below spec impact in the TP:

- NES-capable UE may not start intra/inter frequency cell reselection measurement when legacy condition is fulfilled.

- Configure new intra/inter frequency cell reselection measurement condition (threshold) for NES-capable UE. NES-capable UE starts intra/inter frequency cell reselection measurement when new condition is fulfilled.

Proposal 6: Consider the TP in the appendix.

- Conclusion

In this paper, the following proposals are given:

Proposal 1: Keep the terminology of “NES cells” with the definition of NES-capable UE. Can discuss whether to change the definition when specific NES technique is agreed.

Proposal 2: For solution (Use IntraFreqExcludedCellList/InterFreqExcludedCellList), capture the below spec impact in the TP:

- NES-capable UE ignores legacy IntraFreqExcludedCellList/InterFreqExcludedCellList;

- Additional information is added to indicate the black lists for NES-capable UEs for cell reselection.

Proposal 3: For solution (Use the cellBarred or cell reservation fields in MIB/SIB), capture the below spec impact in the TP:

- If cellBarred in MIB is set to barred, NES-capable UE ignores it but uses cell bar in SIB1;

- If cellBarred in MIB is set to notBarred, NES-capable UE considers the cell as not barred or still uses cell bar in SIB1.

Proposal 4: for (de)prioritization NES cells, capture the below spec impact:

- Configure separate cell offset for NES cells in SIB or dedicated signalling;

- NES cells are indicated to the UE via SIB or dedicated signalling.

Proposal 5: To allow the NES UE not start intra/inter frequency cell reselection measurement when legacy conditions are fulfilled, when camping on NES cell. Capture the below spec impact in the TP:

- NES-capable UE may not start intra/inter frequency cell reselection measurement when legacy condition is fulfilled.

- Configure new intra/inter frequency cell reselection measurement condition (threshold) for NES-capable UE. NES-capable UE starts intra/inter frequency cell reselection measurement when new condition is fulfilled.

Proposal 6: Consider the TP in the appendix.

- References

[1] RP-213554, Study on network energy savings for NR, Huawei.

[2] Report of [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less (Huawei).

[3] R2-2211427, TP on cell selection/reselection and SSB/SIB-less.

- TP

## 6.2	Techniques in Y domain

## 6.x	Higher layer aspects for network energy savings

Editor's note: This section includes common aspects of higher layers deduced from the above candidate directions.

### 6.X.1	Cell selection/reselection

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

- Use IntraFreqExcludedCellList/InterFreqExcludedCellList

- Use the cellBarred or cell reservation fields in MIB/SIB

- Control intra/inter frequency cell reselection measurement

Editor's note: FFS whether to change the terminology of “NES cells” to “a cell that uses an NES technique” when specific NES technique is agreed.

The NW should be able to configure NES-capable UEs to prioritize/de-prioritize a specific NES cell or NES cells on a specific frequency.

6.X.1.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

For solution (Use IntraFreqExcludedCellList/InterFreqExcludedCellList), potential specification impact are:

- NES-capable UE ignores legacy IntraFreqExcludedCellList/InterFreqExcludedCellList;

- Additional information is added to indicate the black lists for NES-capable UEs for cell reselection.

For solution (Use the cellBarred or cell reservation fields in MIB/SIB), potential specification impact are:

- If cellBarred in MIB is set to barred, NES-capable UE ignores it but uses cell bar in SIB1;

- If cellBarred in MIB is set to notBarred, NES-capable UE considers the cell as not barred or still uses cell bar in SIB1.

For (de)prioritization NES cells, potential specification impact are:

- Configure separate cell offset for NES cells in SIB or dedicated signalling;

- NES cells are indicated to the UE via SIB or dedicated signalling.

For measurement for intra/inter frequency cell reselection, potential specification impact are:

- NES-capable UE may not start intra/inter frequency cell reselection measurement when legacy condition is fulfilled.

- Configure new intra/inter frequency cell reselection measurement condition (threshold) for NES-capable UE. NES-capable UE starts intra/inter frequency cell reselection measurement when new condition is fulfilled.