3GPP TSG-RAN WG2 Meeting #120	R2-2211675

# Toulouse, France, 14th Nov. – 18th Nov. 2022

Agenda item:	8.9.2

Source:	vivo

Title:	Discussion on the common L2/L3 parts for U2U relaying

Document for:		Discussion & Decision

1. Introduction

In RAN2#119bis-e meeting[1], the Discovery and Relay (re)selection for U2U have been discussed and some agreements have been reached on e.g. Protocol stack, discovery pool, triggers of relay (re)selection.

In this contribution, we will further discuss some left issues for discovery and relay (re)selection.

2. Discussion

2.1 Terminology for U2U

In TR 23.752 for Release-17 Prose[2], the terminology for U2N remote UE and U2N relay UE are defined as follows:

5G ProSe UE-to-Network Relay: A UE that provides functionality to support connectivity to the network for Remote UE(s). Remote UE: A 5G ProSe-enabled UE that communicates with a DN via a 5G ProSe UE-to-Network Relay.

In RAN2 specification e.g. TS 38.331, we use the similar terminology as in SA2 that is U2N Relay UE and U2N Remote UE.

However for Prose in Release-18[3], the terms used for U2U relay operation, especially for the ‘remote UE’, are changed as follows:

5G ProSe End UE: A 5G ProSe-enabled UE that connects with another 5G ProSe-enabled UE(s) via a 5G ProSe UE-to-UE Relay. 5G ProSe UE-to-UE Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity between 5G ProSe End UEs.

Meanwhile in RAN2 discussions, the terminology used in U2U relay operation are various in different contributions/discussions, e.g. for ‘remote UE’,

- Remote UE

- Source UE

- Source remote UE

- Target (remote) UE

- Destination UE

- U2U UE

Observation 1: SA2 use the term ‘end UE’ in U2U relay operation to denote the UEs connected via a U2U relay.

Observation 2: In RAN2, various terms are used in U2U relay operation.

We understand it is beneficial for a unified terminology in RAN2 discussion as well as when finalizing the specificaiton. To have more commonality with SA2 and with U2N relay operation, we propose to adopt the term ‘U2U End UE’ and ‘U2U Relay UE’ in RAN2.

Moreover, as there are two end UEs in U2U relay operation, we can call them ‘Source U2U end UE’ and ‘Target U2U end UE’, or, ‘Initiating U2U End UE’ and ‘Peer U2U End UE’, as ‘initiating UE’ and ‘peer UE’ are used in RAN2 specification when talking about two UEs in sidelink in a procedure-wise way (who is initiating a signalling to start a procedure). Therefore we propose:

Proposal 1: RAN2 agrees to use the term ‘U2U End UE’ for remote UE and ‘U2U Relay UE’ for relay UE in discussions/specifications for U2U relay operation.

2.2 Discovery

In RAN2 #119bis-e meeting, it was agreed:

Proposal 2.2:		U2U Relay re-uses SL-SRB4 (with associated PDCP, RLC procedures and configuration) to carry discovery messages

However, in SA2 TR[3], the Discovery integrated into PC5 unicast link establishment procedure is supported.

## 8.1 Key Issue #1: Support of UE-to-UE Relay For Key Issue #1 (Support of UE-to-UE Relay), the followings are taken as conclusions: The following conclusions are common for both Layer-3 UE-to-UE Relay and Layer-2 UE-to-UE Relay: - For UE-to-UE Relay discovery, both Model A and Model B discovery are supported. - Discovery integrated into PC5 unicast link establishment procedure is supported. Sol#1 Alt1 is used as basis for normative phase.

Observation 3: Discovery integrated into PC5 unicast link establishment procedure is supported in SA2.

In that case, the discovery is performed by including a relay_indication in Direct Communication Request message. RAN2 needs to confirm that in this case, the SL-SRB used for DCR message which is SL-SRB0, should be used for discovery.

Proposal 2: RAN2 to confirm that SL-SRB0 is used for discovery which is integrated into PC5 unicast link establishment procedure.

The next issue is about the discovery pool. RAN2 has agreed in RAN2 #119bis-e meeting that:

Proposal 4.1:		Both shared and dedicated resource pool can be used for U2U discovery transmission and Rel-17 pool selection principle is re-used.

However, for the following Proposal, it was discussed but without any consensus:

- Proposal 4.2:		RAN2 discuss whether the dedicated discovery resource pool introduced in Rel-17 for U2N relay discovery is used for U2U relay discovery as well.

Companies were thinking that the pool configuration may have impacts on the co-existence for U2U and U2N. However, in our understanding, there are a number of reasons that a same dedicated discovery pool should be used. First, if separate dedicated pools are used for U2N and U2U discovery, the resource segmentation may lead to worse resource utilization. Second, if different resource pools are configured for U2U and U2N discovery, there are impacts on RAN1 because U2U discovery pool should also be considered in “Resource pool index” in a DCI format 3_0. Third, all the RAN2 procedures which consider the discovery pool configuration, e.g. LCP procedure, would also need to be revised to consider the impact for different discovery pools of U2U and U2N, leading to miscellaneous discussions. Therefore, we propose to use the dedicated discovery pool introduced in Rel17 for for U2U relay discovery as well.

Observation 4: If separate dedicated pools are used for U2N and U2U discovery, the resource segmentation may lead to worse resource utilization.

Observation 5: If different resource pools are configured for U2U and U2N discovery, there are impacts on RAN1 as U2U discovery pool should also be considered in “Resource pool index” in a DCI format 3_0.

Observation 6: If different resource pools are configured for U2U and U2N discovery, a lot of RAN2 procedures e.g. LCP procedure, need to be revised to consider the impact for different dedicated discovery pools.

Proposal 3: The dedicated discovery pool introduced in Rel-17 for U2N relay discovery is re-used for U2U relay discovery as well.

2.3 Relay (re)selection

1. Trigger condition

For U2U relay (re)selection, a number of agreements have been reached in RAN2#119bis-e meeting, as follows:

Proposal 7.1a:       Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions.  RAN2 further discuss details for trigger 2).

Proposal 7.1b (modified):       Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.  RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

First, for relay selection, there are two concepts that needs to be clarified:

- Relay selection trigger condition. It means the U2U end UE or the U2U relay UE decides to perform U2U relay operation because the communication on direct link seems impossible, and the U2U end UE or the U2U relay UE can transmit/respond to U2U relay discovery messages.

- Relay selection criteria. It means once the U2U end UE or the U2U relay UE decides to perform U2U relay operation, how can the U2U end UE selects a suitable U2U relay UE, to ensure the channel quality is good.

In previous discussions, these two concepts are not really clear during the whole discussion in relay selection. In the relay selection trigger discussion, the ‘PC5 signal strength conditions’ mainly means that the PC5 signal strength on direct link between two end UEs, e.g. relay selection can be triggered when PC5 signal strength of the direct PC5 link is below a threshold.

However, if the UEs would take some PC5 RSRP on direct link as the condition to trigger relay selection, it is not clear how it would work in mode-A like discovery because the source U2U end UE may not have available PC5 RSRP considering the target U2U end UE is still undecided, while the source U2U end UE is supposed to perform relay selection. For other cases, i.e. when there is available SD-RSRP or SL-RSRP on the direct link, it is possible to take it as the relay selection trigger. Therefore,

Observation 7: There may not be available PC5 RSRP results on direct link between source U2U end UE and non-decided target U2U end UE in discovery procedure.

Proposal 4: Relay selection is triggered at source U2U end UE when there are available PC5 RSRP results on direct link between source U2U end UE and target U2U end UE and the PC5 RSRP is below a threshold. FFS SD-RSRP and/or SL-RSRP.

For relay reselection, the most of the trigger conditions are inherited from U2N relay scenario. For PC5 signal strength conditions, in U2N, they are used as follows:

#### 3GPP TS 38.331 V17.2.0 (2022-09) #### 5.8.15.3 Selection and reselection of NR sidelink U2N Relay UE A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall: 1> if the UE has no serving cell; or 1> if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-remoteUE-Config: 2> if the UE does not have a selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or <omitted…> 3> perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs; 4> when evaluating the one or more detected NR sidelink U2N Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2N Relay UE ID and using the sl-FilterCoefficientRSRP in SystemInformationBlockType12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the preconfigured sl-FilterCoefficientRSRP as defined in 9.3 (out of coverage), before using the SD-RSRP measurement results; 4> consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria; 4> consider one of the available suitable NR sidelink U2N relay UE(s) can be selected; <omitted…>

Therefore, we propose to follow the same principle as in U2N relay.

Proposal 5: When the SL-RSRP of selected U2U relay UE is available and is below a threshold, relay reselection is triggered at source U2U end UE.

Proposal 6: When the SL-RSRP of selected U2U relay UE is not available and SD-RSRP is below a threshold, relay reselection is triggered at source U2U end UE.

For T400 expiry, as it would trigger sidelink RLF which has already been considered as one of the reselection triggers, it does not need to be agreed as one independent resection trigger.

Proposal 7: T400 expiry is not agreed as one relay reselection trigger condition.

For the reselection trigger conditions that needs to be informed to upper layer instead of directly causing reselection, the same principle from U2N relay operation can also be used. In U2N, relay UE would send NotificationMessageSidelink to remote UE, and remote UE may choose to indicate upper layers to trigger PC5 unicast link release, or maintain the PC5 RRC connection. The relay reselection would only be triggered when upper layers request the release of the PC5-RRC connection. On the other hand, RLF detection and PC5 signal strength conditions are considered at UE in AS layer.

Observation 8: In U2N relay, Uu RLF at relay UE would cause Notification message from relay to remote UE and remote UE only trigger relay reselection when it decides to indicate upper layers to trigger PC5 unicast link release.

Proposal 8: As in U2N relay, when source U2U end UE receives PC5-RLF indication from the U2U relay UE, if source U2U end UE decides to release the PC5 link between it and the U2U relay UE, it would inform upper layers and relay reselection can be triggered rely on upper layer indication.

2. Criteria

The agreement in RAN2 #119bis-e on (re)selection criteria is as follows:

Proposal 5.1:		SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.  FFS when each of the two quantities are used and whether to re-use the criteria in Rel17.

For relay selection criteria about SL-RSRP and SD-RSRP, we think the criteria in Rel-17 used for U2N relay is quite mature after several meeting discussions. We don’t think there is strong motivation to design a different mechanism in U2U relay operation. Therefore, we prefer to reuse the Rel-17 criteria.

Proposal 9: Reuse Rel-17 U2N principle for SL-RSRP/SD-RSRP in relay (re)selection criteria, i.e., use SL-RSRP when there is data transmission and up to UE implementation to use SL-RSRP or SD-RSRP when there is not.

However, one more aspect needs to be clarified is that SL-RSRP/SD-RSRP on which hop should be considered as there are now more than one hops for U2U relay communication. We think a general principle can be agree here that PC5 RSRP on all hops should be considered. How to consider the PC5 RSRP for all hops at the same time could be FFS.

Proposal 10: SL-RSRP/SD-RSRP for all hops should be considered to select a U2U relay UE, i.e. only when the SL-RSRP/SD-RSRP for all hops are higher than a configured signal strength threshold, a U2U Relay UE can be selected as the candidate U2U relay UE.

3. Conclusion

In this contribution, we discussed common part for L2/L3 U2U relay (discovery and relay (re-)selection).

We have the following observations and proposals:

For terminology

Observation 1: SA2 use the term ‘end UE’ in U2U relay operation to denote the UEs connected via a U2U relay.

Observation 2: In RAN2, various terms are used in U2U relay operation.

Proposal 1: RAN2 agrees to use the term ‘U2U End UE’ for remote UE and ‘U2U Relay UE’ for relay UE in discussions/specifications for U2U relay operation.

For discovery

Observation 3: Discovery integrated into PC5 unicast link establishment procedure is supported in SA2.

Observation 4: If separate dedicated pools are used for U2N and U2U discovery, the resource segmentation may lead to worse resource utilization.

Observation 5: If different resource pools are configured for U2U and U2N discovery, there are impacts on RAN1 as U2U discovery pool should also be considered in “Resource pool index” in a DCI format 3_0.

Observation 6: If different resource pools are configured for U2U and U2N discovery, a lot of RAN2 procedures e.g. LCP procedure, need to be revised to consider the impact for different dedicated discovery pools.

Proposal 2: RAN2 to confirm that SL-SRB0 is used for discovery which is integrated into PC5 unicast link establishment procedure.

Proposal 3: The dedicated discovery pool introduced in Rel-17 for U2N relay discovery is re-used for U2U relay discovery as well.

For relay (re)selection

Observation 7: There may not be available PC5 RSRP results on direct link between source remote UE and non-decided target remote UE in discovery procedure.

Observation 8: In U2N relay, Uu RLF at relay UE would cause Notification message from relay to remote UE and remote UE only trigger relay reselection when it decides to indicate upper layers to trigger PC5 unicast link release.

Proposal 4: Relay selection is triggered at source U2U end UE when there are available PC5 RSRP results on direct link between source U2U end UE and target U2U end UE and the PC5 RSRP is below a threshold. FFS SD-RSRP and/or SL-RSRP.

Proposal 5: When the SL-RSRP of selected U2U relay UE is available and is below a threshold, relay reselection is triggered at source U2U end UE.

Proposal 6: When the SL-RSRP of selected U2U relay UE is not available and SD-RSRP is below a threshold, relay reselection is triggered at source U2U end UE.

Proposal 7: T400 expiry is not agreed as one relay reselection trigger condition.

Proposal 8: As in U2N relay, when source U2U end UE receives PC5-RLF indication from the U2U relay UE, if source U2U end UE decides to release the PC5 link between it and the U2U relay UE, it would inform upper layers and relay reselection can be triggered rely on upper layer indication.

Proposal 9: Reuse Rel-17 U2N principle for SL-RSRP/SD-RSRP in relay (re)selection criteria, i.e., use SL-RSRP when there is data transmission and up to UE implementation to use SL-RSRP or SD-RSRP when there is not.

Proposal 10: SL-RSRP/SD-RSRP for all hops should be considered to select a U2U relay UE, i.e. only when the SL-RSRP/SD-RSRP for all hops are higher than a configured signal strength threshold, a U2U Relay UE can be selected as the candidate U2U relay UE.

4. References

RAN2 #119bis-e chairman notes;

3GPP TR 23.752 V17.0.0 (2021-03);

3GPP TR 23.700-33 V1.1.0 (2022-10);