3GPP TSG-RAN WG2 Meeting #120		R2-2211676

Toulouse, France, 14th – 18th November 2022

Source:	vivo

Title:	Remaining issues on service continuity enhancement for L2 U2N relay

Agenda Item:	8.9.3

Document for:	Discussion and Decision

- Introduction

At the last RAN2#119bis-e meeting, RAN2 has reached the following agreements.

Agreements:

Proposal 1 (modified)	For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.  FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE.

Proposal 3	For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching.

Proposal 4 (modified)	For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available).

Proposal 5	For i2d path switch scenario, re-use the existing T304 timer

Proposal 6	For d2i and i2i path switch scenarios, re-use the existing T420 timer.

In this contribution, we will focus on the remaining issues (related to the red-highlighted texts above) and give our views correspondingly.

- Discussion

Issue 1: Revisit the Agreed Proposal 4 as above

At RAN2#116-e meeting, RAN2 has made the agreement for Rel-17 SL relay WID as below:

Agreements:

Proposal 18: RAN2 does not consider the sharing of unicast link between relay service and non-relay service in L2 relay, and the related descriptions are to be removed from stage 2 running CR.

If we follow the above conclusion in Rel-18, it means that there is no case that SL-RSRP can be derived from the remote UE and candidate Relay UE on the PC5 unicast link specific to L2 U2N relay, because even if the Remote UE is communicating with the candidate Relay UE on the non-relay link where it can get SL-RSRP, it cannot get SL-RSRP on the relay link to be established. In other words, the DST L2 IDs for direct link and U2N relay link are always different, and neither can the remote UE even know that the DST L2 ID used for the non-relay link and the DST L2 ID used for the rely link actually correspond to the same relay UE, as per the basic unicast DST L2 ID design since Rel-16 NR SL.

Observation 1 	Since Rel-17 SL Relay, the RAN2 conclusion “RAN2 does not consider the sharing of unicast link between relay service and non-relay service in L2 relay” has been followed throughout the whole L2 path switching design. If this conclusion is not reversed, there is no case that SL-RSRP can be derived from the remote UE and candidate Relay UE on the PC5 unicast link specific to L2 U2N relay.

Based on the observation, the candidate U2N relay UEs would never have SL-RSRP available and only SD-RSRP can be used as the measurement quantity. Therefore, we propose to revise the Agreed Proposal 4 as follows:

Proposal 1	Revise the previous agreement at RAN2#119bis-e meeting as follows:

Proposal 4 (modified) For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available). Towards each candidate L2 U2N relay UE, RAN2 confirms that only SD-RSRP is used as the measurement quantity.

Issue 2: Whether to support Event Z2: Candidate L2 U2N relay UE becomes an offset better than serving L2 U2N relay UE

Given the above Proposal 1, there will be two possible cases for comparing the measurement results:

- Case 1: the L2 U2N remote UE uses SL-RSRP towards the serving L2 U2N relay UE and SD-RSRP towards the candidate L2 U2N relay UE (i.e., when SL-RSRP of the serving U2N relay UEs is available)

- Case 2: the L2 U2N remote UE uses SD-RSRP towards both the serving L2 U2N relay UE and the candidate L2 U2N relay UE (i.e., when SL-RSRP of the serving U2N relay UEs is unavailable)

For the above two cases, it is still open whether we can also use the measurement Event Z2 (i.e., Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE). During RAN2#119bis-e meeting online discussion, some company expressed concerns about the necessity of introducing two different measurement Events for the i2i path switch procedure and commented that there could be confusion about when to use which measurement Event. In our understanding, the measurement Event Z2 is mainly used for the above case 2 with the same measurement quantity. And by directly comparison between serving’s and candidate’s link quality, measurement Event Z2 is beneficial for the NW to switch the L2 U2N remote UE to a better candidate L2 U2N relay UE, and thus effectively lower the risk of service interruption, when the L2 U2N remote UE is soon to lose the connection with the serving Relay (especially for the Remote UE out of Uu coverage).

Therefore, we suggest to also support measurement Event Z2.

Proposal 2	For i2i path switch purpose, RAN2 also support the measurement Event Z2: Candidate L2 U2N Relay UE becomes offset better than the serving L2 U2N Relay UE.

As to when to use which measurement Event, basically the L2 U2N remote UE shall follow the measurement configuration by the NW as legacy. However, since the NW is not aware of the availability of the serving U2N relay UE’s SL-RSRP or SD-RSRP measurement results, whether the configured measurement Event takes effect needs to be checked at the L2 U2N remote UE side. For example, if the measurement Event Z2 is configured by the NW, we think the measurement Event Z2 can be used for evaluation by the L2 U2N remote UE only if the L2 U2N remote UE determines that SD-RSRP is used towards both the serving L2 U2N Relay UE and the candidate L2 U2N Relay. If this situation does not hold, the UE may alternatively use event Z1, if configured by the NW for I2I path switch purpose.

Proposal 3	Only when the L2 U2N remote UE determines to use SD-RSRP towards both the serving L2 U2N Relay UE and the candidate L2 U2N Relay UE as the measurement quantity for i2i path switch, the measurement event Z2 is used.

- Conclusion

This contribution discussed the remaining issues on serving continuity enhancement for L2U2N relay. The contribution concludes with:

Observation 1 	Since Rel-17 SL Relay, the RAN2 conclusion “RAN2 does not consider the sharing of unicast link between relay service and non-relay service in L2 relay” has been followed throughout the whole L2 path switching design. If this conclusion is not reversed, there is no case that SL-RSRP can be derived from the remote UE and candidate Relay UE on the PC5 unicast link specific to L2 U2N relay.

Proposal 1	Revise the previous agreement at RAN2#119bis-e meeting as follows:

Proposal 4 (modified) For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available). Towards each candidate L2 U2N relay UE, RAN2 confirms that only SD-RSRP is used as the measurement quantity.

Proposal 2	For i2i path switch purpose, RAN2 also support the measurement Event Z2: Candidate L2 U2N Relay UE becomes offset better than the serving L2 U2N Relay UE.

Proposal 3	Only when the L2 U2N remote UE determines to use SD-RSRP towards both the serving L2 U2N Relay UE and the candidate L2 U2N Relay UE as the measurement quantity for i2i path switch, the measurement event Z2 is used.

- References

- R2-2210804, Report from session on positioning and sidelink relay, Session Chair (MediaTek).