3GPP TSG-RAN WG2 Meeting #120	R2-2211714

Toulouse, France, 14 November – 18 November 2022

Agenda item:	8.14.4

Source:	Apple

Title:	QoE Reporting in NR-DC

WID/SID:	NR_QoE_enh-Core

Document for:	Discussion / Decision

# Introduction

According to the WID [1], Rel-18 should consider how QoE can be supported in NR-DC scenarios. The following agreements have been made by RAN3:

RAN3 #117e Agreements on QoE support in NR-DC: - MN is responsible to configure the s-based QoE to UE. - For M-based QoE configuration in NR-DC, coordination between MN and SN is needed. Details are FFS. - If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE. - If the M-based QoE configuration is received only by the SN, whether the MN or the SN performs UE selection and sends the QoE configuration to the UE needs to be further discussed. - QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session. Send LS to RAN2. - WA: If QoE reports are received by the SN, SN can forward the QoE reports to MCE directly. - RAN3 should discuss and clarify the scenarios for QoE reporting transmitted over SN. Which SRB can be used for QoE reporting in SN depend on RAN2. - WA: MN and SN can generate RVQoE configurations. - MN and SN should coordinate about configuring a dual-connected UE with RVQoE measurements. The details of the coordination are FFS. - WA: UE can send RVQoE report to MN, MN then forward the RVQoE report to SN if needed, and vice versa. RAN3 #117bis-e Agreements on QoE support in NR-DC: - In DC, the UE switches the reporting leg based on indication from network, FFS on implicit or explicit way. - RAN3 should discuss which node can command the UE to switch the reporting leg. - Turn into an agreement the WA stating that, if QoE reports are received by the SN, the SN can forward the QoE reports to MCE directly. Reporting of QoE - If a node has configured the UE with QoE measurements, and the other node is receiving the QoE reports from the UE and forwarding them directly to the MCE, then: The node that has configured the UE with QoE measurements should indicate the QoE reference to the node that receives the reports and forwards them directly to MCE. Configuration of RVQoE - The MN can generate an RVQoE configuration for a UE. - The SN can generate an RVQoE configuration for a UE. FFS whether MN can modify the SN generated RVQoE configuration - The MN can send an RVQoE configuration to the UE. Reporting of RVQoE - The MN can receive RVQoE reports directly from the UE. - The SN can receive RVQoE reports directly from the UE. - Turn the following WA into an agreement: “UE can send RVQoE report to the MN, the MN then forward the RVQoE report to the SN if needed, and vice versa”. - Agree to ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. FFS on how to ensure. MN-SN coordination procedure The coordination between the MN and the SN should support at least the following (details to be further discussed): - Initiation by either the MN or the SN for m-QoE, by the MN for s-QoE. - Coordination for configuring the UE. - Coordination for establishing the SRB for receiving QoE/RVQoE reports. - Indication about switching the reporting leg.

On the other hand, in RAN2 #119bis-e, we have reached the following agreements:

RAN2 #119bis-e Agreements on QoE support in NR-DC: Observation: Rel-18 QoE configuration may be created by MN or SN. Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination. 1: In NR-DC scenario, both signalling-based and management-based QoE measurement collection shall be supported. Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

# Discussions

Based on the discussions in RAN2 #119bis-e, RAN2 has identified a few different options for QoE reporting toward SN in NR-DC, including:

- Option 1: A new SRB connecting to SN via SCG for QoE reporting (e.g. SRB5)

- Option 2: Extending the purpose of SRB3 to include QoE reporting

- Option 3: Configure SRB4 as a split SRB such that QoE reporting leg can be switched within the same bearer.

By considering the agreements in both RAN2 and RAN3, we have noted the following:

- In NR-DC, the UE should be able to flexibly switch the reporting leg of QoE toward MN and SN. This is based on RAN3 agreement [2]: “QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session.” Thus, the UE should be ready for reporting to either MN or SN at any time while the application is running.

- To decide the uplink bearer for QoE reporting in NR-DC scenarios, SRB4 should be used as a baseline in accordance to RAN2 agreement. From our perspective it means that, when should only consider introducing other types of SRB if SRB4 and/or its extension cannot support QoE reporting to SN.

With these considerations, we provide a comparison among the bearer options:

Table 1 A comparison among different options of SRB for QoE reporting toward SN

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: New SRB | - A radio bearer is dedicated to QoE reporting toward SN, which seems to be a clean approach. | - To support the cases where the UE may need to report QoE to either MN or SN, Rel-17 SRB4 has to be configured together with the new SRB. This means a UE has to be configured with two independent SRBs for QoE reporting concurrently. - From specification point of view, RAN2 needs to introduce a new SRB, and it does not seem to be reasonable to have two out of six SRBs that are dedicated to QoE. |
| Option 2: Reuse SRB3 | - No new SRB has to be introduced for SN reporting. | - To support the cases where the UE may need to report QoE to either MN or SN, Rel-17 SRB4 has to be configured together with SRB3. This means a UE has to be configured with two independent SRBs for QoE reporting concurrently. - From specification point of view, RAN2 needs to examine how to prevent interference of QoE to other higher priority control-plane data in SRB3. |
| Option 3: Split SRB4 | - No new SRB has to be introduced, and this is aligned with RAN2 agreement where SRB4 is used as baseline. - Flexible leg switch for QoE reporting is possible without having to configure two independent SRBs. | - From specification point of view, RAN2 needs to examine how leg switching can be fulfilled with split SRB4. |

Based on the analysis, we think split SRB4 offers the flexibility of QoE reporting leg switching without the need of configuring two independents radio bearers dedicated to QoE at the UE side. In addition, it saves some specification complexity of introducing new SRB. As mentioned above, if RAN2 further introduce a new SRB for QoE reporting, it means two out of the six SRB types are dedicated to QoE reporting, and we do not think it is justifiable to have so many SRBs for an optional feature such as QoE.

Observation 1: It is not justifiable to have two different SRBs (i.e. SRB4 and another new SRB) dedicated to the feature of QoE, considering that extension of SRB4 (which is already agreed as the baseline) may already offer some flexibility of leg switching.

It has been argued that split SRB is only used for duplication, while duplication is not needed for QoE as application layer measurement results do not require low-latency or high reliability, and so Split SRB4 should not be used for QoE reporting to SN. We totally agree that duplication is not needed for QoE, but we would like to point out it is not entirely true that split SRB is used for duplication only. In particular, it is clear that pdcp-duplication in pdcp-config is an optional field, and it does not have to configured even if the PDCP entity is for a split SRB. This is also described in TS 37.340:

## 7.6 Split SRB Split SRB is supported for both SRB1 and SRB2 (split SRB is not supported for SRB0 and SRB3) in all MR-DC cases. RRC PDUs on split SRB are ciphered and integrity protected using NR PDCP. Split SRB can be configured by the MN in Secondary Node Addition and/or Modification procedure, with SN configuration part provided by the SN. A UE can be configured with both split SRB and SRB3 simultaneously. SRB3 and the SCG leg of split SRB can be independently configured. For the split SRB, the selection of transmission path in downlink depends on network implementation. For uplink, the UE is configured via MN RRC signalling whether to use MCG path or duplicate the transmission on both MCG and SCG.

Hence, duplication is an optional feature that can be used on top of split SRB, but it does not mean split SRB is only used for duplication. A split SRB can be configured to use one of the paths only.

Observation 2: Split SRB is not only used for duplication.

The next question is, how to enable leg switching of QoE reporting with split SRB4 ? Based on RAN3 agreement, it is not yet clear what is the trigger of leg switching, but we know it is some type of network indication. In any case, we need a procedure for the UE to switch reporting leg of QoE when certain conditions are met.

We would like to emphasize that, in the existing RRC specification (TS 38.331), we already have some procedures relating to leg switching in split SRB. In particular, when the UE intends to transmit MCGFailureInformation message to SN, the UE may set the primaryPath of the PDCP entity to SCG, and hence the MCGFailureInformation message can be transmitted to SN with split SRB1. This is clearly described as a part of actions related to MCGFailureInformation message transmission in Clause 5.7.3b.4 of TS 38.331:

#### 5.7.3b.4 Actions related to transmission of MCGFailureInformation message …… 1> if SRB1 is configured as split SRB and pdcp-Duplication is not configured: 2> if the primaryPath for the PDCP entity of SRB1 refers to the MCG: 3> set the primaryPath to refer to the SCG. ……

To support QoE reporting using split SRB4, we could re-use the same logic. That is, if the UE intends to transmit an application layer measurement report to SN, it can simply reset the primary path of the PDCP entity to refer to the SCG. Since such UE behavior is already defined in the legacy specification for split SRB1, this is quite straightforward to apply the same procedure to split SRB4 in Rel-18.

Observation 3: The procedure of primary path reset in MCGFailureInformation message transmission can be reused for QoE reporting leg switching in Split SRB4.

Based on the discussions above, we believe split SRB4 is the best option as it does not require RAN2 to define new SRB. Furthermore, split SRB already allows some flexible leg switching mechanism in the current specifications,   which means that specification effort is generally low as we can reuse some existing RRC procedures to enable leg switching. Since split SRB4 is also aligned with RAN2 agreement where SRB4 should be used as the baseline in Rel-18, we think split SRB4 should be used for QoE reporting toward SN.

Proposal : Split SRB4 should be used for QoE reporting toward SN in Rel-18.

# Conclusions

In this contribution, we have discussed about different options for QoE reporting toward SN in NR-DC scenarios. The following observations are made:

Observation 1: It is not justifiable to have two different SRBs (i.e. SRB4 and another new SRB) dedicated to the feature of QoE, considering that extension of SRB4 (which is already agreed as the baseline) may already offer some flexibility of leg switching.

Observation 2: Split SRB is not only used for duplication.

Observation 3: The procedure of primary path reset in MCGFailureInformation message transmission can be reused for QoE reporting leg switching in Split SRB4.

Based on these observations, we conclude with the following proposal:

Proposal : Split SRB4 should be used for QoE reporting toward SN in Rel-18.

# References

[1] RP-221803, WID update for Enhancement on NR QoE, China Unicom, RAN #96, June 2022.

[2] R3-225256, LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC, RAN3#117e, Aug 2022.