3GPP TSG-RAN WG2 Meeting #120	R2-2211716

Toulouse, France, 14 November – 18 November 2022

Agenda item:	8.5.4.1

Source:	Apple

Title:	Considerations for BSR Enhancements

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

During RAN2 #119bis-e, it was quite clear that most companies think there is a need for BSR enhancement. It was mainly motivated by XR characteristics of potentially large packet size, and the benefits of delay-aware scheduling. We have reached the following agreements [1]:

- RAN2 #119bis-e Agreements 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). Delay information consists of at least “remaining time”. 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

Based on the agreements, this paper aims to provide our views on some considerations for BSR enhancements.

# Discussions

## Co-Existence of Multiple BSR Formats

It is expected that new BSR table with finer granularity of buffer size level will be specified in Rel-18. However, a finer granularity BSR table also implies more overhead for MAC CE signaling. From our point of view, a new BSR table is mainly used for XR traffic flows with potential large packet sizes, but there is no need to apply such new BSR table in all cases. For instance, a UE may simultaneously have uplink for video, audio, pose and data. While finer granularity BSR may be more efficient for video/audio, legacy BSR is probably sufficient for other pose and data. Therefore, we think RAN2 can confirm that both legacy BSR and new BSR would co-exist for XR operations.

Proposal 1: RAN2 should confirm that both new and legacy BSR would co-exist in XR operations.

Moreover, according to the agreements, new BSR with delay information may also be introduced. Obviously, delay information is not always needed. For certain traffics with long delay budget, there is no need to increase BSR overhead on delay information. On the other hand, whether to use BSR with delay information may also depend on how long the data has been queueing in the buffer. If the packet has just arrived, inclusion of delay information may not be critical; conversely, if the delivery deadline is approaching, some indication about the urgency can be indicated in the BSR. Thus, in order to have a more flexible and efficient BSR framework, we think the UE may have to consider multiple BSR formats when it intends to trigger and send a BSR. The possible BSR formats may include:

- Legacy BSR without delay information (Rel-17 BSR)

- Legacy BSR with delay information

- New BSR (finer granularity) with delay information

- New BSR (finer granularity) without delay information

Note that the above list is merely an example, RAN2 may end up with even more BSR formats that can co-exist (e.g. multiple BSR tables with different granularity levels, on top of the legacy BSR table). In light of this, we think the UE needs a guideline about how the BSR format should be selected based on e.g. which LCG/LCH have data available, and/or what is the status of the buffered data. That is, certain association between LCGs/LCHs/buffered data status and BSR tables/formats may be provided. From our perspective, the details can be discussed in WI phase, but RAN2 should at least confirm such issue will be addressed.

Proposal 2: RAN2 should specify BSR format selection under different conditions, such as which LCH/LCG has data and/or buffered data status.

## UE-Assistance for Buffer Size Granularity Determination

During RAN2 #119bis-e, it was agreed that RAN2 can further discuss potential enhancements about UE assistance information relating to uplink traffic characteristics, which may facilitate the gNB to schedule configured grants more appropriately. We have the following agreement [1]:

- RAN2 #119bis-e Agreements 2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

It is worth noting that, if a UE can identify the characteristics of UL XR traffics by e.g. interactions with application layer, the UE may also provide certain information about BSR table construction. Alternatively, since both gNB and UE may have a common knowledge about different buffer size granularity that could be supported (e.g. based on UE capability), the UE may suggest which buffer size granularity is the best for the application that is being executed. In particular, if the UE can learn what is the expected UL packet size from an application, the UE can also determine its preference on the buffer size granularity that can improve resource allocation efficiency while optimizing the BSR overhead. This would allow the UE and the gNB to “negotiate” and determine the BSR table to be applied. For instance, the UE can simply provide its preferred buffer size and/or buffer size granularity, and the gNB can further confirm if such UE preference can be accepted by e.g. RRC re-configuration.

Proposal 3: RAN2 can consider BSR table construction and/or buffer size granularity selection based on negotiation between UE and gNB.

## Handling of BSR with Urgency

Based on the status of RAN2 discussion on BSR enhancements, we think in some cases the BSR or the corresponding SR may have to be triggered and transmitted quite urgently. For examples:

- The BSR with delay information should be provided to the gNB urgently once this is triggered, so the gNB can have more time budget to schedule data transmission.

- Both [2] and [3] have proposed that packet discarding can be considered as a new BSR triggering event. If this is the case, we think such BSR should be transmitted as soon as possible such that the gNB can have up-to-date buffer status before allocating excessive radio resource which is unnecessary.

Hence, it is beneficial for RAN2 to consider how to facilitate more rapid BSR/SR triggering or transmission, in order to handle BSR with urgency. As an example, we have observed that in some cases SR is not immediately triggered when the logicalChannelSR-DelayTimer is running, and the UE may not be able to have a UL-SCH resource for BSR transmission in time:

TS 38.321: 2> if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running: 3> if there is no UL-SCH resource available for a new transmission; or 3> if the MAC entity is configured with configured uplink grant(s) and the Regular BSR was triggered for a logical channel for which logicalChannelSR-Mask is set to false; or 3> if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see clause 5.4.3.1) configured for the logical channel that triggered the BSR: 4> trigger a Scheduling Request.

Potentially, we may allow SR triggering when the BSR with urgency is triggered, even if the logicalChannelSR-DelayTimer is running. Overall, we think some enhancements may be introduced to make sure BSR with urgency can be triggered/transmitted more quickly.

Proposal 4: RAN2 can consider introducing mechanisms to facilitate more rapid transmission of urgent BSR.

## Cancellation of Triggered BSR Upon Packet Discarding

According to TS 38.321, the triggered BSR may be cancelled if all pending data in the buffer can be transmitted on a MAC PDU:

TS 38.321: All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader.

Essentially, when the UL grant is able to accommodate all the data, the BSR is no longer needed as the buffer becomes empty. With the similar train of thought, we think there could be a scenario where a BSR is triggered based on UL data availability in a LCH, but then the buffer becomes empty due to packet discarding, even before the triggered BSR is transmitted. In such cases, we think the already-triggered BSR can be cancelled upon packet discarding, as the triggering event of this BSR (i.e. UL data availability) may no longer be valid when packets are discarded.

Proposal 5: RAN2 can consider cancellation of previously triggered-BSR upon packet discarding.

## BSR with Indication of Buffered PDU Set Characteristics

SA2 has been discussing the possibility of delivering to RAN some PDU Set characteristics information such as “importance” levels (e.g. an important PDU set may correspond to an I-Frame of video, while PDU sets representing other video frames such as P-Frame may be considered as “less important”). In our understanding, although PDU sets with different importance could be mapped to different QoS flows, they may still be mapped to the same DRB in some implementations. For example, such mapping may be applied in implementations where the number of DRBs is to desired to be limited.

When PDU sets with different importance levels and reliability targets are mapped to the same DRB, the gNB may not be able to allocate uplink resource appropriately if it does know the importance level of PDU set that has arrived in the uplink buffer at the UE. To be on the safe side, the gNB may always allocate radio resources with higher reliability target (e.g. lower MCS and/or more repetitions) for a DRB to ensure important PDU sets mapped to this DRB can always be delivered, even though most of other PDU sets in the same DRB are less important; this apparently degrades resource efficiency.

From our point of view, it is useful for the UE to indicate some of these characteristics of the PDU set(s) arrived in the uplink buffer, such as PDU Set importance level. Hence, in order to enhance BSR to accommodate XR use cases, we think UE should be allowed to use BSR to indicate certain characteristics of PDU sets that have arrived in the buffer.

Proposal 6: RAN2 can consider BSR enhancement to indicate certain characteristics of PDU Sets such as PDU Set importance.

# Conclusions

This contribution provides some of our views on possible BSR enhancements for Rel-18 XR. In particular, we propose the following:

Proposal 1: RAN2 should confirm that both new and legacy BSR would co-exist in XR operations.

Proposal 2: RAN2 should specify BSR format selection under different conditions, such as which LCH/LCG has data and/or buffered data status.

Proposal 3: RAN2 can consider BSR table construction and/or buffer size granularity selection based on negotiation between UE and gNB.

Proposal 4: RAN2 can consider introducing mechanisms to facilitate more rapid transmission of urgent BSR.

Proposal 5: RAN2 can consider cancellation of previously triggered-BSR upon packet discarding.

Proposal 6: RAN2 can consider BSR enhancement to indicate certain characteristics of PDU Sets such as PDU Set importance.

# References

- R2-2210802, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE, Vice Chairman (Nokia), RAN2#119bis-e, Online, October 2022.

- R2-2209558, BSR for XR, Nokia, Nokia Shanghai Bell, RAN2 #119bis-e, October 2022.

- R2-2209636, Enhancements to Buffer Status Reporting for XR Traffic, Intel, RAN2 #119bis-e, October 2022.