3GPP TSG-RAN WG2 Meeting #120	R2-2211768

Toulouse, France, November 14 – November 18, 2022

Agenda Item		:	8.7.4.1

Source		:	LG Electronics Inc.

Title		:	Discussion on NTN-TN cell reselection enhancements

Document for		:	Discussion and Decision

1. Introduction

In RAN2#119bis-e meeting, following agreements are made.

RAN2#119bis-e agreements: 1. To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available. 2. UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

In this contribution, we discuss measurement relaxation on TN frequency when UE camps on an area only covered by NTN cell.

2. Discussion

As agreed in the last meeting, NTN cell should broadcast information to UEs on whether the UEs are able to not perform TN frequency measurement. The NTN cell may convey the information via SIB19 because TN cell measurement relaxation is NTN-specific feature.

Observation 1	NTN cell broadcasts information to UEs whether the UEs are able to not perform TN frequency measurement via SIB19.

The options how to represent the information were proposed in RAN2#119bis-e email discussion [117].

Proposal 8: Regarding the Assistance information for UE to identify an area where TN network is available, the following options can be further discussed: - The cell center and cell radius of TN neighbour cells, or in other terms, the reference location and a distance threshold of TN neighbour cells - The boundary line between TN area and NTN area - For quasi-earth fixed cells, TN coverage is described by a distance range from the cell center and an angle range based on a reference direction - An indication could be included in system information to indicate NTN cell’s coverage overlaps with terrestrial TN cell’s coverage - NTN cell can be divided to several virtual areas based on certain criteria. The virtual areas and the corresponding TN frequency information are broadcast as assistance information to help UE perform more accurate TN measurements. - Introduce a parameter using the polygon shape captured in TS 23.032 to describe the coverage area of a TN neighbour cell.

The companies exposed their views on this, and most supported option 1. The assistance information is intuitive to UE, i.e., if UE locates outside of the coverage, UE is able to not perform measurement on TN neighbor cells. However, there are concerns about broadcasting direct TN coverage. It may cause a security issue, as pointed out in the email discussion. Accurate cell coverage is confident for mobile network operators. Therefore, we propose to revise option 1 as “The reference location and a distance threshold. The assistance information indicates an area only covered by NTN.”

Regarding option 5, second majority, RAN2 has to discuss a representation of virtual area, in which the discussion is duplicated. We think that the simplest assistance information is revised option 1.

Observation 2	Broadcasting direct TN coverage may cause security issue, in which accurate cell coverage information is confidential for mobile network operator.

Proposal 1	The NTN cell provides an assistance information indicating an area only covered by NTN. The assistance information consists of the reference location and distance threshold.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	NTN cell broadcasts information to UEs whether the UEs are able to not perform TN frequency measurement via SIB19.

Observation 2	Broadcasting direct TN coverage may cause security issue, in which accurate cell coverage information is confidential for mobile network operator.

Proposal 1	The NTN cell provides an assistance information indicating an area only covered by NTN. The assistance information consists of the reference location and distance threshold.

4. References

[1] R2-2210860, R2-2210860 Report of [AT119bis-e][117][NR NTN Enh] cell reselection enhancements (Intel), Intel