3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211786

Toulouse, 14th - 18th November, 2022

Source:	Qualcomm Incorporated

Title:	Open issue on service continuity for UE-to-Network relay

Agenda Item:	8.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for REl-18 Relay enhancement.  This contribution discusses open issues based on the RAN2 agreements.

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

# Discussion

## 2.1 New event between indirect and indirect service continuity

In the last RAN2 meeting, RAN2 discussed the measurement event for serving Relay and candicate Relay camparision for indirect-to-indirect path.

Agreements:

Proposal 1 (modified)	For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.  FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE.

Proposal 3	For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching.

Proposal 4 (modified)	For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available).

Proposal 5	For i2d path switch scenario, re-use the existing T304 timer

Proposal 6	For d2i and i2i path switch scenarios, re-use the existing T420 timer.

There is FFS issue as highlighted in the agreements. One benefit of introducing Z2 is always to select the Relay UE with the best PC5 link quality, which is same as today’s A3 event. However, usually, only the same measurement quantity can be compared directly. So if a new event Z2 is introduced, the same measurement quantity should be used to be compared directly.

Proposal 1: If a new event Z2 is determined to be introduced, only the same measurement quantity is used for event evaluation.

For the serving Relay UE, the Remote UE uses SL-RSRP measurements for event trigger evaluation when there is data transmission from Relay UE to U2N Remote UE. UE can by implementation use SL-RSRP or SD-RSRP for event trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE.

For the neighbor Relay UEs, whether the SL-RSRP can be available the sidelink measurement quantity depends on whether the L2 IDs for U2N and U2U Relay can be same. If these IDs can be the same, then SL-RSRP could be available to be measured when there is already SL communication towards to the neighbor Relay UE for U2N Relay.

Observation 1: Whether SL-RSRP could be available to be measured on the neighbor Relay UE depends on whether the L2 IDs for U2N and U2U Relay can be same.

Then, it should be reasonable to wait or ask SA2 whether the L2 IDs for U2N and U2U Relay can be same, and then determine whether SL-RSRP or SD-RSRP could be used for Z2 evaluation.

Proposal 2: If a new event Z2 is determined to be introduced, send an LS to ask SA2 whether the L2 IDs for U2N and U2U Relay can be same.

## 2.3 Target Relay UE and path type selection

Target Relay UE and path type selection was discussed in last RAN3 meeting and initially in RAN2 meeting. This issue is mainly about which RAN node selects the target Relay UE and path type, and RAN3 should be the suitable WG to discuss it. Then from work split point of view, it is proposed to leave RAN3 to discuss which RAN node determines the target Relay UE/path type and the exchange information between the source RAN node and taret RAN node.

Proposal 3: Follow the work split: It is left to RAN3 to discuss which RAN node determines the target Relay UE/path type and the exchange information between the source RAN node and taret RAN node.

## 2.4 Other enhancements

In the last RAN2 meetings, some contributions propose further enhancements in Rel-18 on the following aspects :

- CHO-like and DAPS like solutions

- Lossless path switching

- Prolonged path switching preparation time due to reconfiguration of the target relay UE for remote UE’s path switching

- Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell

Based on current WID scope, only the four scenarios (i.e. A,B,C,D in clause 1) are considered in Rel-18, that means we only need to discuss the four scenarios specific issues compared to the scenarios supported in Rel-17. For the further enhancements listed above, they are common enhancements for the scenarios supported in Rel-17 and Rel-18, and out of Rel-18 scope.

Observation 2: The following enhancements are common for the scenarios supported in Rel-17 and Rel-18, and out of Rel-18 scope.

- CHO-like and DAPS like solutions

- Lossless path switching

- Prolonged path switching preparation time due to reconfiguration of the target relay UE for remote UE’s path switching

- Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell

And then it is proposed not to pursue these enhancements in Rel-18.

Proposal 4: RAN2 does not pursue the following enhancement in Rel-18.

- CHO-like and DAPS like solutions

- Lossless path switching

- Prolonged path switching preparation time due to reconfiguration of the target relay UE for remote UE’s path switching

- Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell

# Conclusion

This contribution discusses the potential aspects on service continuity of U2N relay, and provides the following observations and proposals.

Observation 1: Whether SL-RSRP could be available to be measured on the neighbor Relay UE depends on whether the L2 IDs for U2N and U2U Relay can be same.

Observation 2: The following enhancements are common for the scenarios supported in Rel-17 and Rel-18, and out of Rel-18 scope.

- CHO-like and DAPS like solutions

- Lossless path switching

- Prolonged path switching preparation time due to reconfiguration of the target relay UE for remote UE’s path switching

- Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell

Proposal 1: If a new event Z2 is determined to be introduced, only the same measurement quantity is used for event evaluation.

Observation 1: Whether SL-RSRP could be available to be measured on the neighbor Relay UE depends on whether the L2 IDs for U2N and U2U Relay can be same.

Proposal 2: If a new event Z2 is determined to be introduced, send an LS to ask SA2 whether the L2 IDs for U2N and U2U Relay can be same.

Proposal 3: Follow the work split: It is left to RAN3 to discuss which RAN node determines the target Relay UE/path type and the exchange information between the source RAN node and taret RAN node.

Proposal 4: RAN2 does not pursue the following enhancement in Rel-18.

- CHO-like and DAPS like solutions

- Lossless path switching

- Prolonged path switching preparation time due to reconfiguration of the target relay UE for remote UE’s path switching

- Introduce the Allowed-List/Block-List to restrict candidate relay UE’s serving cell

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    R2-2211101 RAN2#119bis-e Meeting Report	MCC