3GPP TSG-RAN WG2 Meeting #120	R2-2211792

Nov 14th – 18th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on remaining issues for CPAC

Agenda item:		6.2.2

Document for:		Discussion and Decision

- Introduction

At last meeting, RAN2 discussed CPAC and postponed some issues related to conditional measurements [1]:

Proposal 1: RAN2 to discuss and confirm which understanding is right on the restriction that UE is not required to perform the measurement on measIds that are configured for conditional reconfiguration, but are not linked with any candidate cell: - Understanding 1: the restriction applies to SN initiated inter-SN CPC only; - Understanding 2: the restriction applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC; - Postponed. Consider until next meeting whether this is a transitory issue or not. - 4: Can continue to discuss in the November meeting how to solve the situation that, unlike Rel-17 UEs, Rel-16 UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration, and the Rel-17 network is not aware of this. Should provide inter-operability analysis and reasons why something could go wrong. RAN2 can consider minuting the expected behaviour in chair notes even if no CR is agreed.

In this contribution, we discussed the remaining issues for CPAC.

- Discussion

At last meeting, RAN2 discussed how to understand the restriction for conditional measurements as below. But no consensus was reached.

Proposal 1: RAN2 to discuss and confirm which understanding is right on the restriction that UE is not required to perform the measurement on measIds that are configured for conditional reconfiguration, but are not linked with any candidate cell: - Understanding 1: the restriction applies to SN initiated inter-SN CPC only; - Understanding 2: the restriction applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC;

In the AT-meeting email discussion [209], RAN2 agreed to clarify the condition for the UE to perform measurements for conditional events. And the current NR RRC CR in [2] is proceeded based on the understanding 2. Namely, the restriction applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC.

### 5.5.3 Performing measurements #### 5.5.3.1 General *// skip unrelated part //* 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG measConfig and is indicated in the condExecutionCond associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA or MN-initiated inter-SN CPC in NR-DC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC in NR-DC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC):

Observation 1: The current RRC CR is proceeded based on the understanding 2, i.e. the restriction on conditional measurements applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC.

Although the initial intention is to introduce the restriction on conditional measurements for SN initiated inter-SN CPC only (i.e. in case that the execution conditions are pre-configured by the source SN but the candidate SN may not select all suggested candidate PSCells), it may be difficult to identify which measurements are only configured for SN initiated inter-SN CPC from the UE perspective. So it seems no feasible way to capture the understanding 1 in the RRC spec. Thus, it’s preferred to go for understanding 2. In this way, no change to the CR in [1] is required.

Proposal 1: RAN2 confirms to go for understanding 2, i.e. the restriction on conditional measurements applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC.

According to the Rel-16 RRC specs, the UE is required to perform conditional measurements regardless whether there is an associated conditional reconfiguration. But in Rel-17 RRC specs, the UE is not required to perform conditional measurements for which there is no corresponding conditional reconfiguration. Thus, some companies concerned there may have some inter-operability issue when Rel-16 UE connecting with Rel-17 NW. For example, the Rel-17 NW configures some conditional measurements but without associated conditional reconfiguration, which are not required to be measured by the Rel-17 UE. So the NW may assume those conditional measurements shall not consume the UE capability. However, the Rel-16 UE will measure all configured conditional measurements regardless whether there is an associated conditional reconfiguration, which may exceed the maximum UE capability for measurements and cause RRC re-establishment.

Observation 2: If the Rel-17 NW configures the conditional measurements but without associated conditional reconfiguration and assumes those measurements shall not consume the UE capability, but the Rel-16 UE measures all configured measurements (i.e. consuming the UE capability), it shall cause the inter-operability issue, e.g. exceeding the applicable UE capability.

In order to avoid such mismatch between the NW and the UE, we think the NW can simply follow the principle that all configured measurements shall consume the UE capability, regardless whether the UE is required to perform the measurements or not. And we think the smart NW implementation will remove the useless conditional measurements in time. Anyway, it can be up to the NW implementation to ensure the total configured measurements does not exceed the applicable UE capabilities. And no change to the current specs is required.

Proposal 2: It can be up to the NW implementation to ensure that the total configured measurements do not exceed the applicable UE capabilities, regardless whether the UE is required to perform the measurements or not. No change to the current specs is required.

- Conclusion and proposals

In this contribution, we discussed the remaining issues for CPAC with the following observations and proposals:

Observation 1: The current RRC CR is proceeded based on the understanding 2, i.e. the restriction on conditional measurements applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC.

Proposal 1: RAN2 confirms to go for understanding 2, i.e. the restriction on conditional measurements applies to all kind of conditional reconfiguration, i.e., CHO, CPA, intra-SN CPC, MN initiated inter-SN CPC, and SN initiated inter-SN CPC.

Observation 2: If the Rel-17 NW configures the conditional measurements but without associated conditional reconfiguration and assumes those measurements shall not consume the UE capability, but the Rel-16 UE measures all configured measurements (i.e. consuming the UE capability), it shall cause the inter-operability issue, e.g. exceeding the applicable UE capability.

Proposal 2: It can be up to the NW implementation to ensure that the total configured measurements do not exceed the applicable UE capabilities, regardless whether the UE is required to perform the measurements or not. No change to the current specs is required.

- Reference

- RAN2#119bis-e Chair Notes

- R2-2210828	Corrections for further MR-DC enhancements	Huawei, HiSilicon