3GPP TSG-RAN WG2 Meeting #120	R2-2211810

Toulouse, France, November 14-18, 2022

Agenda Item:	8.4.2.3

Source: 	ASUSTeK

Title:  	Discussion on L1L2-triggered mobility

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meeting, some agreements were made regarding model for L1L2-triggered mobility (LTM) and some assumptions were also made related to solutions for dynamic switching:

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - a. Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - b. Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration […] RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. […]

In this contribution, we discuss further details on possible solutions for dynamic switching in L1 L2 mobility procedure based on previous discussions and R1/R2 status.

- Discussion

Support of both RACH-less and RACH-based procedure for LTM

In previous meeting, a RAN2 assumption was made that both RACH-based and RACH-less procedures can be supported for L1 L2-triggered mobility, while the feasibility of RACH-less may be depending on RAN1. According to the RAN1 agreements made in the previous meeting [1]:

Agreement Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2). From Oct 19th GTW session Agreement On mechanism to acquire TA of the candidate cells, the following solutions can be further studied: - RACH-based solutions - e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd - RACH-less solutions - e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell) Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

TA acquisition of candidate Cell is supported before cell switch command is received in L1/L2-triggered mobility. While the actual procedure for how to acquire TA is still under study in RAN1 (e.g. explicit or implicit association between TA/TAG and candidate Cell), RAN2 can first confirm the previous assumption that both RACH-less and RACH-based solution can be supported in LTM. In addition, how does the NW provide the TA information to UE is also pending on the outcome of RAN1 study on TA acquisition.

Proposal 1: 	Confirm RAN2 assumption that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1/L2-triggered mobility can be supported. Details on providing TA information are to be studied after further RAN1 progress.

Completion of RACH-less and RACH-based procedure for LTM

In NR reconfiguration with sync, the UE generates and transmits a RRC reconfiguration complete message to the target Cell. To notify the target Cell about the arrival of the UE, the UE should also generate and transmit a LTM complete message to the target Cell. Similar to the legacy handover procedures, the LTM complete message could be an RRC message as a baseline. Alternatively, L1/L2 signaling (e.g. MAC CE) can be used as the LTM complete message if the content to be carried by the message is limited.

Proposal 2: 	During the LTM procedure, the UE generates and transmits a LTM complete message to target Cell. FFS the content of the message and FFS whether the message is a RRC message or a L1/L2 signaling.

In NR reconfiguration with sync, the procedure is successfully completed when the random access procedure towards the target Cell is completed. The same behaviour can be reused for RACH-based LTM. However, since there’s no random access procedure in RACH-less LTM, completion of the procedure should be defined. In LTE RACH-less handover procedure, the completion of the procedure is defined as below:

-	if the MAC entity is configured with rach-Skip or rach-SkipSCG and a UE Contention Resolution Identity MAC control element for this TTI has been received on the PDSCH indicated by the PDCCH of the SpCell addressed to the C-RNTI:

-	indicate to upper layer the successful reception of a PDCCH transmission addressed to the C-RNTI.

The UE Contention Resolution Identity MAC control element indicated by the PDCCH of the SpCell addressed to the C-RNTI implies that the target cell receives the RRC reconfiguration complete message successfully. To adopt the behaviour in RACH-less LTM, the UE can consider the LTM successful when an acknowledgement for successful reception of the LTM complete message by the target Cell is received by the UE. Whether the acknowledgement is a HARQ ACK, PDCCH, MAC CE, a RRC message, or something else could be further discussed.

Proposal 3: 	A RACH-based LTM is successful when a random access procedure initiated for the LTM is successfully completed.

Proposal 4:	A RACH-less LTM is successful when the UE receives an acknowledgement from the target Cell. FFS on what the acknowledgement is.

Failure handling for LTM

In NR reconfiguration with sync, a T304 timer is maintained for failure detection of the procedure. The timer is started when the UE is indicated with reconfiguration with sync. The timer is stopped when the procedure is successfully complete. When the timer expires, the UE performs RRC connection re-establishment. In LTM, a timer for failure detection should also be adopted. Whether to use T304, a new RRC timer, a new MAC timer, or something else could be further discussed. To adopt reconfiguration with sync behaviour, the timer is started when receiving the trigger (e.g. MAC CE or DCI) for LTM. The timer is stopped when the LTM is successfully completed. When the timer expires, the LTM procedure is considered to be failed.

Proposal 5: 	A timer could be used to detect failure for RACH-based and RACH-less LTM:

- the timer is started when receiving a triggering signal (MAC CE or DCI) for LTM

- the timer is stopped when the LTM is successfully completed

- LTM fails when the timer expires

For RACH-based LTM, the procedure is not likely to succeed in the target Cell when the timer expires. RRC connection re-establishment can be used in order to select a suitable Cell and re-establish RRC connection with NW. For RACH-less LTM, missing of the LTM complete message or the acknowledgement may be due to beam misalignment or inaccurate UL power control. Performing RRC connection re-establishment directly without trying to fall back to RACH in the target Cell may be an overkill.

Proposal 6: For RACH-based LTM, the UE initiates RRC connection re-establishment when LTM fails.

Proposal 7: For RACH-less LTM, the UE fall back to RACH in the target Cell when LTM fails.

- Conclusion

We have the following proposals for solutions for LTM:

Proposal 1: 	Confirm RAN2 assumption that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1/L2-triggered mobility can be supported. Details on providing TA information are to be studied after further RAN1 progress.

Proposal 2: 	During the LTM procedure, the UE generates and transmits a LTM complete message to target Cell. FFS the content of the message and FFS whether the message is a RRC message or a L1/L2 signaling.

Proposal 3: 	A RACH-based LTM is successful when a random access procedure initiated for the LTM is successfully completed.

Proposal 4:	A RACH-less LTM is successful when the UE receives an acknowledgement from the target Cell. FFS on what the acknowledgement is.

Proposal 5: 	A timer could be used to detect failure for RACH-based and RACH-less LTM:

- the timer is started when receiving a triggering signal (MAC CE or DCI) for LTM

- the timer is stopped when the LTM is successfully completed

- LTM fails when the timer expires

Proposal 6: For RACH-based LTM, the UE initiates RRC connection re-establishment when LTM fails.

Proposal 7: For RACH-less LTM, the UE fall back to RACH in the target Cell when LTM fails.

- Reference

[1] Final Report of 3GPP TSG RAN WG1 #107bis-e v1.0.0