3GPP TSG RAN WG2 #120                                 R2-2211850

Toulouse, France, November 14th – 18th, 2022

Agenda item:	8.16.2

Source:	Fujitsu

Title:	Discussion on the AI/ML methods for general aspects of AI/ML via air interface

Document for:	Discussion/Decision

1. Introduction

The AI/ML for Air Interface study item has been setup in RAN plenary meeting #95 and the corresponding SID [1] indicated the work splitting among the WGs, the details can be found in the paragraphs below:

2) Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: o PHY layer aspects, e.g., (RAN1)  Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases  Use case and collaboration level specific specification impact, such as new signaling, means for training and validation data assistance, assistance information, measurement, and feedback o Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1  Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input  Collaboration level specific specification impact per use case o Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2  Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable  Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

From the above SID description, it is noticed that RAN2 will not participate in the discussion of AI/ML model performance evaluation, AI/ML sub use cases selection and AI/ML terminology. Generally speaking, the study scope of RAN2 will be focused on the lifecycle management and framework design for both overall and use case specific aspects. In this contribution, we propose our views on the observation of the detailed RAN2 study scope and how to group different aspects for study in the future.

# 2. Discussion

There are multiple topics related to the AI/ML method, based on the contents of the SID and multiple discussion agreements made in RAN1 below, we share our views with regarding to the two following aspects:

- Work splitting and boundary between RAN1 and RAN2.

- Lifecycle management functionalities

## 2.1 Work split and boundary between RAN1 and RAN2

As a RAN1-led study item, general aspects of framework (RAN1 9.2.1) and evaluation and other aspects of three use cases (RAN1 9.2.2 – 9.2.4) had been studied in previous three RAN1 meetings. Before further digging into the details of the study details in RAN2, it is necessary to first split the work scope between RAN1 and RAN2.

RAN2 will focus on high layer mechanism and signaling study of the procedures in model life cycle management, such as data collection, model monitoring and model switching issues according to the conclusions and agreements achieved in each sub agenda of three use cases in RAN1.

Proposal 1 Study RAN2 signaling and procedures to support the collaboration between network and UE per lifecycle management functionality such as model training, model monitoring, model registration, model ID and UE capabilities.

Proposal 2 It is suggested that RAN2 only study aspects which has clear definitions or decisions given by RAN1, in order to avoid overlapping and conflictions.

## 2.2 Lifecycle management functionalities

The general lifecycle management functionalities are described in the agreement of RAN1 below:

Agreement (R1-110) Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

According to the agreement above, we select the following aspects as starting point for RAN2 study:

- Data collection

- Model training

- Model monitoring

  - Model selection, activation, deactivation, switching, and fallback operation

- Model registration

- UE capability report

- Identification of Models (Model ID)

Please notice that in the above agreement, the items labelled with square brackets are pending for further discussions and decisions in RAN1.

### 2.2.1 Data collection

The following conclusion regarding the data collection is given from RAN1:

Conclusion (R1-110bis-e) Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

It can be seen from the above conclusion that data collection can serve multiple LCM functions, different aspects (e.g., the data volume, format, dimension and related configuration and reporting) of the data collected via the air interface can play different roles per LCM function. Based on the RAN1 conclusion, we have the following proposal for data collection:

Proposal 3 Study necessity, feasibility, procedures and potential specification impact for data collection for AI/ML model training/inference/update/monitoring, taking the following aspects into account:

- Assistance signalling for UE’s data collection

- Assistance signalling for gNB’s data collection

- Assistance signalling for label collection and quality check.

- Assistance signalling for data requirements per LCM function.

- Delivery/transfer of the datasets.

### 2.2.2 Model training

It has been agreed in RAN1 session 9.2.1 that there will be one-sided model and two-sided model, the definitions can be found below:

| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| --- | --- |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |

Both of the two types of models require the support from RAN2 signaling. For two-sided models, RAN2 signaling is needed to complete different training collaboration types between two sides (e.g., UE and gNB) which the model deploys, and for one-sided models, RAN2 signaling is required to indicate the training details such as model capabilities, data and label collection and model output requirements. For model training we have the following proposal:

Proposal 4 Study procedures and signalling for model training, taking the following aspects into account:

- Model capabilities reporting related to model training.

- Data requirements and collection for model training.

- Assistance signalling for different model training types of two-sided models.

### 2.2.3 Model monitoring

The following agreements regarding the model monitoring is given from RAN1:

Agreement(R1-110bis-e) Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). - FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement(R1-110bis-e) Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

Regarding model monitoring, there are two aspects should be studied. The one is per use case-based monitoring label/metric and method should be studied. The other one is the decisions upon monitoring results and following-up mechanisms, such as model activation, deactivation, selection, switching, fallback, and update (including re-training). We think the monitoring label, metric and method is based on RAN1 decision, while the monitoring and decision method with protocol impacts, following-up mechanisms should be studied in RAN1.

Proposal 5 Regarding model monitoring and follow-up actions including model activation, deactivation, selection, switching, fallback, and update (including re-training), RAN2 study mechanism and protocol impacts for above mentioned actions based on monitoring results.

### 2.2.4 Model registration

Model registration is under discussing in RAN1, its terminology, purpose, the relationship with UE capability report was suggested to be clarified in RAN1#111 ([2] RAN1 9.2.1). According to our understanding, model registration is mainly related to RAN2 protocols. For example, in the case of two-sided model, a model part at UE-side needs to confirm the support of its paired model part at network side. In the case of UE-side model, network side may assign different model ID to a set of models with the same functionality owned by the UE. RAN2 can study its procedure and potential signaling upon RAN1 progress and agreement in RAN1#111 meeting.

Proposal 6 Regarding model registration, RAN2’s study on procedure and potential signaling impacts can be started upon RAN1’s agreement on the definition of its terminology and clarifications on its roles in LCM.

### 2.2.5 UE Capability report

The relationship between UE capability report and model registration will be clarified in RAN1#111. According to our understanding, UE capability report is for a UE to report its AI/ML relevant capability for a specific functionality to NW. While model registration is mainly for model awareness and model identification to facilitate model effective management. Since UE capabilities related to AI/ML have not been concluded in RAN1, its report related protocol impacts can be discussed later.

Proposal 7 Regarding UE capability report, RAN2 wait for RAN1’s progress and agreement.

### 2.2.6 Identification of Models (Model ID)

Agreement (R1-110bis-e) Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models - FFS: Detailed discussion of model ID with associated information and/or model functionality. - FFS: usage of model ID with associated information and/or model functionality-based LCM procedure - FFS: whether support of model ID - FFS: the detailed applicable AI/ML operations

According to RAN1 discussions and agreement, model ID can be taken as a key factor in the procedures of model’s LCM, such as model registration, model monitoring, model selection, model switching, model update, model inference and model training procedures. The necessity of having model ID lies in the identification of different models with the same functionality for both level y and level z models. The only exception case is level x model, where the model existence is transparent to the network side.

In RAN1, the design of model ID such as its format and design principle has not been discussed. For example, model ID’s role and related procedures, whether the model ID is assigned by network or is assigned by its model developer? The model ID is a unified ID in network or a temporal/localized model ID? Model version number is included in model ID or not, with the consideration of model update. ID for a model group and ID for its group member (a model group means a set of models with the same functionality for different scenario/configuration/site). Besides, if unified model ID is supported, model-ID based model performance record and sharing can be considered.

We suggest RAN2 clarify these issues first, then study model ID design and related protocol impacts.

Proposal 8 Regarding to model ID, at least the following issues are expected to be clarified in RAN2:

- model ID’s role and related procedures

- model ID is assigned by network or by model developer

- model ID is unified in network or a temporal/localized model ID

- model version number is included in model ID or not

- model ID(s) for a set of models with the same functionality

- model-ID based model performance sharing

# 3. Conclusion

Proposal 1 Study RAN2 signaling and procedures to support the collaboration between network and UE per lifecycle management functionality such as model training, model monitoring, model registration, model ID and UE capabilities.

Proposal 2 It is suggested that RAN2 only study aspects which has clear definitions or decisions given by RAN1, in order to avoid overlapping and conflictions.

Proposal 3 Study necessity, feasibility, procedures and potential specification impact for data collection for AI/ML model training/inference/update/monitoring, taking the following aspects into account:

- Assistance signalling for UE’s data collection

- Assistance signalling for gNB’s data collection

- Assistance signalling for label collection and quality check.

- Assistance signalling for data requirements per LCM function.

- Delivery/transfer of the datasets.

Proposal 4 Study procedures and signalling for model training, taking the following aspects into account:

- Model capabilities reporting related to model training.

- Data requirements and collection for model training.

- Assistance signalling for different model training types of two-sided models.

Proposal 5 Regarding model monitoring and follow-up actions including model activation, deactivation, selection, switching, fallback, and update (including re-training), RAN2 study mechanism and protocol impacts for above mentioned actions based on monitoring results.

Proposal 6 Regarding model registration, RAN2’s study on procedure and potential signaling impacts can be started upon RAN1’s agreement on the definition of its terminology and clarifications on its roles in LCM.

Proposal 7 Regarding UE capability report, RAN2 wait for RAN1’s progress and agreement.

Proposal 8 Regarding to model ID, at least the following issues are expected to be clarified in RAN2:

- model ID’s role and related procedures

- model ID is assigned by network or by model developer

- model ID is unified in network or a temporal/localized model ID

- model version number is included in model ID or not

- model ID(s) for a set of models with the same functionality

- model-ID based model performance sharing

# 4. Reference

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] R1-2210708 Summary#5_9.2.1, 3GPP TSG RAN WG1 Meeting #110-Bis e-meeting, October 10th – 19th, 2022