3GPP TSG-RAN WG2 Meeting #120	R2-2211852

Toulouse, France, Nov 14th~18th ,2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3650 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SUI initiation and PDCCH repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The reporting initiation of the sidelink DRX related SUI message have been added for the UE configured to perform NR sidelink reception in the latest specification, and following general condition are also included: - 1) SUI not transmitted before; or: 2) SUI transmitted but the UE undergo an incapable gNB; or: 3) SUI content change. - but the above three general cases are missed for TX UE to report DRX related information in SUI message. 2. According to the LS(R1-2210735) from RAN1, PDCCH repetition is not supported for sidelink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.8.3.2, for the reporting initiation of the sidelink DRX assistance information or the sidelink DRX configuration reject information, add the similar conditions of 1) SUI not transmitted before; or: 2) SUI transmitted but the UE undergo an incapable gNB; or: 3) SUI content change. - Modify the field description of search space link ID to avoid network configure PDCCH repetition for sidelink. Impact analysis Impacted functionality Inter-operability: - For change-1, if the network implements the change but not the UE, there is no inter-operability since the result is just more or fewer SUI message initiation. For change-2, there is no inter-operability issue. - 2. For change-1, if the UE implements the change but not the network, there is no inter-operability since it is just that UE initiates more or fewer SUI messages than network expected. For change-2, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The TX UE may not report DRX related information via SUI message in following three cases: - 1) SUI not transmitted before; or: 2) SUI transmitted but the UE undergo an incapable gNB; or: 3) SUI content change. - And without the restriction, TX UE may transmit the SUI redundantly. - 2. Network may configure PDCCH repetition to sidelink UE, which is not supported in PHY layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.2, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

5.8.3.2	Initiation

A UE capable of NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation that is in RRC_CONNECTED may initiate the procedure to indicate it is (interested in) receiving or transmitting NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation in several cases including upon successful connection establishment or resuming, upon change of interest, upon changing QoS profiles, upon receiving UECapabilityInformationSidelink from the associated peer UE, upon RLC mode information updated from the associated peer UE or upon change to a PCell providing SIB12 including sl-ConfigCommonNR. A UE capable of NR sidelink communication may initiate the procedure to request assignment of dedicated sidelink DRB configuration and transmission resources for NR sidelink communication transmission. A UE capable of NR sidelink communication may initiate the procedure to report to the network that a sidelink radio link failure or sidelink RRC reconfiguration failure has been declared. A UE capable of NR sidelink discovery may initiate the procedure to request assignment of dedicated resources for NR sidelink discovery transmission or NR sidelink discovery reception. A UE capable of U2N relay operation may initiate the procedure to report/update parameters for acting as U2N Relay UE or U2N Remote UE (including L2 Remote UE's source L2 ID).

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception, upon accepting the sidelink DRX configuration from the associated peer UE. A UE capable of NR sidelink communication that is in RRC_CONNECTED and is performing sidelink unicast transmission with resource allocation mode 1 may initiate the procedure to report the sidelink DRX assistance information or the sidelink DRX configuration reject information received from the associated peer UE, upon receiving either of them from the associated peer UE. A UE capable of NR sidelink communication that is performing sidelink groupcast transmission may initiate the procedure to report the sidelink DRX on/off indication for the associated Destination Layer-2 ID.

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the Destination Layer-2 ID and QoS profile associated with its interested services that sidelink DRX is applied, for NR sidelink groupcast or broadcast reception.

Upon initiating this procedure, the UE shall:

1>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

2>	ensure having a valid version of SIB12 for the PCell;

2>	if configured by upper layers to receive NR sidelink communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqList; or if the frequency configured by upper layers to receive NR sidelink communication on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink communication reception in accordance with 5.8.3.3;

2>	if configured by upper layers to transmit non-relay NR sidelink communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqList; or if the information carried by the sl-TxResourceReqList has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink communication transmission resources in accordance with 5.8.3.3.

2>	if configured by upper layer to receive NR sidelink non-relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-NonRelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-NonRelayDiscovery; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqListDisc; or if the frequency configured by upper layers to receive NR sidelink L2 U2N relay discovery messages or NR sidelink L3 U2N relay discovery messages on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink discovery reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink discovery messages reception in accordance with 5.8.3.3;

2>	if configured by upper layer to receive NR sidelink L2 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to receive NR sidelink L3 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqListDisc; or if the frequency configured by upper layers to receive NR sidelink discovery messages on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	if the UE is capable of U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR relay sidelink discovery reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR relay sidelink discovery messages reception in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink non-relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-NonRelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-NonRelayDiscovery; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqListDisc; or if the information carried by the sl-TxResourceReqListDisc has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink non-relay discovery messages resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink non-relay discovery messages resources in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink L2 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to transmit NR sidelink L3 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case L2 U2N relay operation or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqListDisc; or if the information carried by the sl-TxResourceReqListDisc has changed since the last transmission of the SidelinkUEInformationNR message:

4>	if the UE is capable of U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay discovery messages resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink relay discovery messages resources in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink L2 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay or if configured by upper layer to transmit NR sidelink L3 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case L2 U2N relay operation or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2U2N-Relay; or if the information carried by the sl-TxResourceReqL2U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL3U2N-Relay; or if the information carried by the sl-TxResourceReqL3U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message:

4>	if the UE is capable of U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay communication transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqL2U2N-Relay or sl-TxResourceReqL3U2N-Relay:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink relay communication transmission resources in accordance with 5.8.3.3;

2>	if configured by upper layers to perform NR sidelink reception on the frequency included in sl-FreqInfoList in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

3>	if the UE received a sidelink DRX configuration in the RRCReconfigurationSidelink message for NR sidelink unicast reception from the associated peer UE and the UE accepted the sidelink DRX configuration:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxDRX-ReportList; or if the information carried by sl-RxDRX-ReportList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX configuration in accordance with 5.8.3.3;

3>	else:

4>	if the last transmission of the SidelinkUEInformationNR message included sl-RxDRX-ReportList:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the sidelink DRX configuration is no longer used in accordance with 5.8.3.3;

3>	if the UE is performing NR sidelink groupcast or broadcast reception and is interested in a service that sidelink DRX is applied:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedGC-BC-DestList; or if the information carried by sl-RxInterestedGC-BC-DestList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the Destination Layer-2 ID and QoS profile associated with the service in accordance with 5.8.3.3;

3>	else:

4>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedGC-BC-DestList:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in the service that sidelink DRX is applied in accordance with 5.8.3.3;

2>	if configured by upper layers to perform NR sidelink transmission on the frequency included in sl-FreqInfoList in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs and if the UE is configured with sl-ScheduledConfig:

3>	if the UE received a sidelink DRX assistance information or a sidelink DRX configuration reject information from the associated peer UE for NR sidelink unicast transmission:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-DRX-InfoFromRxList or SL-FailureList; or if the information carried by sl-DRX-InfoFromRxList or SL-FailureList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX assistance information or the sidelink DRX configuration reject information in accordance with 5.8.3.3;

3>	if the UE is performing NR sidelink groupcast transmission:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-DRX-Indication; or if the information carried by sl-DRX-Indication has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report sidelink DRX on/off indication for the corresponding destination in accordance with 5.8.3.3;

Next change

6.3.2	Radio resource control information elements

--------irrelevant text omitted------------

–	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled SCell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions). For a scheduled SpCell in the case of the cross carrier scheduling, if the search space is linked to another search space in the scheduling SCell, all the optional fields of this search space in the scheduled SpCell are absent (regardless of their presence conditions) except for nrofCandidates.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

SearchSpaceExt-v1700 ::=            SEQUENCE {

monitoringSlotPeriodicityAndOffset-v1710 CHOICE {

sl32                                     INTEGER (0..31),

sl64                                     INTEGER (0..63),

sl128                                    INTEGER (0..127),

sl5120                                   INTEGER (0..5119),

sl10240                                  INTEGER (0..10239),

sl20480                                  INTEGER (0..20479)

}                                                                                                   OPTIONAL,   -- Cond Setup5

monitoringSlotsWithinSlotGroup-r17       CHOICE {

slotGroupLength4-r17                     BIT STRING (SIZE (4)),

slotGroupLength8-r17                     BIT STRING (SIZE (8))

}                                                                                                   OPTIONAL,   -- Need R

duration-r17                             INTEGER (4..20476)                                         OPTIONAL,   -- Need R

searchSpaceType-r17             SEQUENCE{

common-r17                      SEQUENCE {

dci-Format4-0-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-2-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-AndFormat4-2-r17  SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-7-r17               SEQUENCE {

nrofCandidates-PEI-r17          SEQUENCE {

aggregationLevel4-r17       ENUMERATED {n0, n1, n2, n3, n4}                         OPTIONAL,   -- Need R

aggregationLevel8-r17       ENUMERATED {n0, n1, n2}                                 OPTIONAL,   -- Need R

aggregationLevel16-r17      ENUMERATED {n0, n1}                                     OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Need R

searchSpaceGroupIdList-r17          SEQUENCE (SIZE (1.. 3)) OF INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)  OPTIONAL,  -- Cond DedicatedOnly

searchSpaceLinkingId-r17            INTEGER (0..maxNrofSearchSpacesLinks-1-r17)                     OPTIONAL    -- Cond DedicatedOnly

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace except commonControlResourceSetExt which is configured by SIB20. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Format2-4 If configured, UE monitors the DCI format 2_4 according to TS 38.213 [13], clause 11.2A.

dci-Format2-5 If configured, IAB-MT monitors the DCI format 2_5 according to TS 38.213 [13], clause 14.

dci-Format2-6 If configured, UE monitors the DCI format 2_6 according to TS 38.213 [13], clause 10.1, 10.3. DCI format 2_6 can only be configured on the SpCell.

dci-Format2-7 If configured, UE monitors the DCI format 2_7 according to TS 38.213 [13], clause 10.1, 10.4A.

dci-Format4-0 If configured, the UE monitors the DCI format 4_0 with CRC scrambled by MCCH-RNTI/G-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-1-AndFormat4-2 If configured, the UE monitors the DCI format 4_1 and 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [11.1].

dci-Format4-1 If configured, the UE monitors the DCI format 4_1 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-2 If configured, the UE monitors the DCI format 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1). This field is not configured for operation with shared spectrum channel access in this release.

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1. If this field is present, the field dci-Formats is ignored and dci-FormatsSL is used.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For SCS 480 kHz and SCS 960 kHz, duration-r17 is used, and the configured duration is restricted to be an integer multiple of L slots and smaller than periodicity, where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17. If duration-r17 is absent, the UE assumes the duration in slots is equal to L. The maximum valid duration is periodicity-L. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set, see TS 38.213, clause 10.1. Each bit in the bitmap corresponds to one RB set, and the leftmost (most significant) bit corresponds to RB set 0 in the BWP. A bit set to 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set.

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. For SCS 15, 30, 60, and 120 kHz and if the UE is configured to monitor: - DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. - DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10). - DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. For SCS 480 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40' are applicable. For SCS 960 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80' are applicable. For SCS 480 kHz and SCS 960 kHz, and the configured periodicity and offset are restricted to be an integer multiple of L slots, where L is the configured length of the bitmap provided by monitoringSlotsWithinSlotGroup-r17, i.e. for a given periodicity, the offset has a range of {0, L, 2*L, …, L*FLOOR(1/L*(periodicity-1))}. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10). If monitoringSlotPeriodicityAndOffset-r17 is present, any previously configured monitoringSlotPeriodicityAndOffset is released, and if monitoringSlotPeriodicityAndOffset is present, any previously configured monitoringSlotPeriodicityAndOffset-r17 is released.

monitoringSlotsWithinSlotGroup Indicates which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. The first (leftmost, most significant) bit represents the first slot in the slot group, the second bit represents the second slot in the slot group, and so on. A bit set to '1' indicates that the corresponding slot is configured for multi-slot PDCCH monitoring (see TS 38.213 [13], clause 10). The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13].

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for (multi-slot) PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE or IAB-MT. For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10. For IAB-MT: For DCI format 2_0 or DCI format 2_5, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-CI The number of PDCCH candidates specifically for format 2-4 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 10.1).

nrofCandidates-PEI The number of PDCCH candidates specifically for format 2-7 for the configured aggregation level.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1). For a search space configured with freqMonitorLocations-r16, only value ′n1′ is valid.

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceGroupIdList-r16, searchSpaceGroupIdList-r17 List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1 if searchSpaceGroupIdList-r16 is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList-r16 is ignored.

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active. For an IAB-MT, the search space defines how/where to search for PDCCH candidates for an IAB-MT where each search space is associated with one ControlResearchSet and for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceBroadcast, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked. Network shall not set same SearchSpaceLinkingId to two search spaces for sidelink.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| DedicatedOnly | In PDCCH-Config, the field is optionally present, Need R. Otherwise it is absent, Need R. |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| Setup2 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt-r16 of the parent IE with the field searchSpaceType-r16 or searchSpaceType-r17 included. Otherwise it is optionally present, Need M. |
| Setup3 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt (without suffix) of the parent IE with the field searchSpaceType (without suffix) included. Otherwise it is optionally present, Need M. |
| Setup4 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset-r17 is not included. It is optionally present, Need M, otherwise. |
| Setup5 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset (without suffix) is not included. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |
| SetupOnly2 | In PDCCH-Config, the field is optionally present upon creation of a new SearchSpace and absent, Need M upon reconfiguration of an existing SearchSpace. In PDCCH-ConfigCommon, the field is absent. |

--------irrelevant text omitted------------

End of the change