3GPP TSG-RAN WG2 Meeting #120	                               R2-2211863

Toulouse, Nov, 2022

Agenda Item:	8.4.2.3

Source: 	OPPO

Title:  	Open issues on dynamic switching for LTM

Document for:	Discussion and Decision

Introduction

In RAN#94 meeting, WID [1] has been agreed to further study mobility enhancement, and one of the objectives is to study L1/L2 triggered mobility, which aiming to reduce the HO latency as well as interruption.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In this contribution, we analyse how UE performs dynamic switch among multiple candidate cells.

Discussion

Currently serving cell change is triggered by L3 signalling, UE needs to perform complete L2 and L1 resets, which leads to relativity long latency. L1/L2 triggered inter-cell mobility aiming to reduce mobility latency especially in intra-CU scenario. In our contribution, we discuss the possible enhancements to achieve dynamic switching for LTM.

Figure 1: Mobility in intra-CU scenario

The principle of L1/L2 triggered mobility enhancements is to enable dynamic switch among candidate serving cells based on L1/L2 signalling. With pre-configured candidate cells, L1/L2 signalling trigged cell switch is much efficiency in contrast to traditional HO preparation, since the propagation delay of cell switch command can be reduced by replacing RRC reconfiguration message with L1/L2 signalling, which avoids HO failure caused by the overdue cell switch command.

Compared with RRC reconfiguration message, L1/L2 signalling reduces the propagation delay of cell switch command.

NW-triggered LTM, i.e. LTM based on received handover command, is considered as high priority.

And as indicated in WID, UE performs dynamic switch among candidate serving cells based on L1/L2 signalling. For L1/L2 signalling, considering MAC CE is relatively easy for extension, and MAC CE can carry more information by comparing to DCI. RAN2 agreed to use MAC CE for conveying cell switch trigger information. While companies have not reach consensus on whether use MAC CE as the actual triggering of cell switching. Companies proposing 2-step LTM trigger prefer to use DCI which indicate TCI state as actual trigger for cell switching. We understand TCI state is further activated by DCI if MAC CE indicates multiple TCI states for UE in current spec. Otherwise, DCI is not required if single TCI state is indicated in the MAC CE. And for LTM, single TCI state indication in cell switch MAC CE is sufficient. There is no need to introduce such 2-step cell switch trigger, that is, single L1/L2 signalling (i.e. cell switch MAC CE) is used for cell switch information indication as well as cell switch triggering.

MAC CE is used for actual cell switch triggering.

Regarding to the content of cell switch MAC CE, we have agreed that at least candidate cell index is included in the MAC CE during last meeting.

RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index.

We would like to provide our further considerations on information that should be conveyed in cell switch MAC CE. As agreed previously, L1 measurements is performed for candidate monitoring. TCI state associated with SSB/CSI-RS of target may be also included in L1/L2 signalling to provide initial beam information for reception and receiving in target serving cell. And in previous meeting, RAN2 reached an assumption on the definition of L1/L2 triggered mobility interruption time as following. The endpoint of HO interruption time is the DL/UL reception/transmission on the indicated beam of the target cell, which means the HO interruption will be reduced if the beam is indicated in HO command.

Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further).

To further minimize the mobility latency due to TA acquisition, RAN1 agreed to study TA acquisition of candidate cell(s) before cell switch command is received in LTM. If TA is pre-obtained by NW, TA info can also be conveyed in cell switch MAC CE for subsequent UL transmission.

Cell switch MAC CE can contain TCI state info and TA info.

In legacy L3 HO command (RRC reconfiguration with sync), protocol handling is explicitly indicated by network in target cell configuration, i.e. PDCP re-establish indication, PDCP data recovery indication, RLC re-establish indication. Similar as L3 HO, RAN2 assumes that whether the UE performs partial or full MAC reset, re-establish RLC, perform data recovery with PDCP is also explicitly controlled by the network for LTM. And it is FFS on how to indicate the PDCP data recovery, RLC re-establish and MAC reset.

RadioBearerConfig information element

As agreed in previous meeting, both intra-DU and intra-CU inter-DU scenarios are supported for L1/L2 triggered mobility, and R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. While for inter-DU case, partial L2 reset may be required due to different configurations. Currently, there is no limitation on whether inter-DU candidate cell and intra-DU candidate cell can be configured to UE simultaneously. As shown in Figure2, there would be a case that two candidate cells are in same DU (e.g., Cell2 and Cell3), and both of them are inter-DU to source cell (e.g., Cell1). If UE moves from Cell1 to Cell2, L2 reset should be performed due to inter-DU protocol handling. Subsequently, if UE moves from Cell2 to Cell3, there is no need to perform L2 reset again since the two cells belong to same DU. It can be observed that the relationship (i.e., inter or intra DU) between source cell and target cell is unfixed. Considering UE has no knowledge of CU/DU deployment, and NW cannot predict UE movement, inclusion of the L2 reset indication in cell switch command is necessary.

Figure 2: An example of consecutive LTM

On the other hand, explicit inclusion of the protocol handling indication in cell switch command can benefit on designing common procedure for intra-DU and inter-DU L1/L2 triggered mobility.

Furthermore, RAN2 discussed two alternatives on RRC modelling for candidate cell configuration, and cellgroupconfig based model is one of the most promising way. While one bottleneck of cellgroupconfig based model is that PDCP data recovery are not included in current cellgroupconfig, and PDCP data recovery may be required for inter-DU scenario for data retransmission. Thus, including L2 reset indication in L1/L2 cell switch command MAC CE is necessary.

L2 reset indication (e.g. PDCP data recovery, RLC re-establish, MAC Reset) is indicated in cell switch MAC CE.

During online discussion last meeting, there are two additional open issues on the content of cell switch command:

FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

Regarding to SCell activation/deactivation indication, currently there are two approaches to inform UE on SCell activation/deactivation, i.e. RRC-based and MAC CE based. For LTM, we see no strong reason to introduce SCell activation/deactivation indication in cell switch MAC CE since LTM aiming to reduce HO interruption time instead of coverage enhancement. And target PCell will anyway be activated to communicate with UE. Additionally, NW may rely on L3 measurement to dynamically control SCell activation/deactivation, while UE will only perform L1 measurement for LTM candidate cells monitoring as agreed in previous meeting. L1 measurement may not be sufficient for SCell activation/deactivation, especially for inter-DU case where L3 measurements on SCell are unavailable.

Thus, there is no need to indicate SCell activation/deactivation simultaneously with cell switch MAC CE.

SCell activation/deactivation is not indicated in cell switch MAC CE.

For BWP information, if source cell includes BWP information, e.g., BWP ID in cell switch command, extra inter-DU interaction may be required on target cell resource allocation. Therefore, we prefer to follow legacy way on BWP information indication, i.e. BWP information is indicated in candidate cell config.

BWP information is not indicated in cell switch MAC CE.

Conclusion

Based on the discussion in section 2 we have following observations:

Observation 1	Compared with RRC reconfiguration message, L1/L2 signalling reduces the propagation delay of cell switch command.

Based on the discussion above, we have the following proposals:

Proposal 1	NW-triggered LTM, i.e. LTM based on received handover command, is considered as high priority.

Proposal 2	MAC CE is used for actual cell switch triggering.

Proposal 3	Cell switch MAC CE can contain TCI state info and TA info.

Proposal 4	L2 reset indication (e.g. PDCP data recovery, RLC re-establish, MAC Reset) is indicated in cell switch MAC CE.

Proposal 5	SCell activation/deactivation is not indicated in cell switch MAC CE.

Proposal 6	BWP information is not indicated in cell switch MAC CE.

Reference

- RP-222332  Further NR Mobility Enhancements    MediaTek Inc