3GPP TSG-RAN WG2 Meeting #120	 R2- 2211877

Toulouse, 14 – 18 Nov, 2022

Agenda Item:	8.16.2

Source:	Xiaomi

Title:	Discussion on AIML for NR air interface

Document for:	Discussion

Introduction

In [1], Artificial Intelligence and machine learning for air interface is proposed. The RAN2 work is as following,

o Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1  Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input  Collaboration level specific specification impact per use case

In this contribution, we discuss the potential impact on RAN2 and give our proposals.

Discussion

To enable the AI/ML utilization in air interface, the basic procedure could comprise data collection, model training, model delivery and inference. Model delivery may not be needed if the model training and inference is performed within the same entity.

RAN1 has reached following working assumption on following terminology,

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

There are various kinds of training manners such as offline training with static data or online training with real-time data. For the three main use cases identified for Release 18 study, all of them can be achieved by offline training. In addition, we think offline training is easier to implement. In addition, the typical use cases and achieved performance gain of online training is not clear yet. Thus, in release 18, we propose to deprioritize the study of online training.

Proposal 1: Deprioritize the study of online training in Rel-18

In current use cases, the inference is done by either UE or gNB. No other entities are considered for inference. Because the radio operation is terminated at UE and gNB for the three use cases. Although UE and gNB may perform the mode training, it’s possible model training is done at other entities. Because model training may require more computation power, which may beyond UE and gNB capability. Also, private model may be provided by third party. In this case, the AI model should be delivered to UE and gNB. Following tables lists all possible combinations entities. The AI model should be delivered from the model training entity in the row to the inference entity in the column.

Table 1. Model delivery option

| From To | Proprietary server | Core network entity | gNB | UE |
| --- | --- | --- | --- | --- |
| gNB | Option 1 | Option 3 | N/A | Option 6 |
| UE | Option 2 | Option 4 | Option 5 | N/A |

If the proprietary server is responsible for either data collection or model training as highlighted in purple, the data set or AI model delivery occurs between 3GPP entity and non-3GPP entity. It can be transmitted as data following existing procedure. Such delivery options seem to have no impact on 3GPP specification and can be left to implementation.

Proposal 2: The AI model delivery between proprietary server and other entities has no impact on 3GPP specification and can be left to implementation.

Among the options without proprietary server involvement, if the core network entity is responsible for either data collection or model training as highlighted in yellow, the data set or AI model delivery occurs between core network entity and RAN entity. If the delivery occurs between core network entity and gNB, it’s likely to use NG interface, which is in scope of RAN3 and SA. If the delivery occurs between core network entity and UE, it’s likely to use NAS, which is in scope of CT and SA. In general, these options are out of RAN2 scope. So, we suggest to not consider such options in RAN2. If any support is identified in future, we can send LS to corresponding WGs.

Proposal 3: The AI model delivery between core network entity and RAN entities, i.e. UE and gNB, is out of RAN2 scope and can be studied by other groups.

Among the options without proprietary server or core network entity involvement as highlighted in green, the delivery occurs between gNB and UE. The delivery would have impact to RAN and is in scope of RAN2. To study the AI model delivery method, some meta info of the model should be clarified. First, the model size could be relatively large, e.g. hundreds of MB. It’s difficult to transmit such large message via lower layer signalling. Higher layer signalling should be used to carry data set.

Proposal 4: AI Model delivery should support large size AI model, e.g. hundreds of MB.

In legacy, the configuration is always from gNB. AI model can be provided by gNB as well. However, in some use case, the AI model may be provided from UE to gNB. For example, in CSI compression, two side AI may be supported. The AI model used by gNB may be provided by UE. Therefore, the AI model delivery from UE to gNB should be supported.

Proposal 5: AI model delivery should be bi-directional between UE and gNB.

Following potential delivery methods are observed:

- The model is delivered in RRC message. RRC message is already defined between UE and gNB, which can be extended. Existing RRC procedure is defined under the assumption of gNB control. However, in model transmission, UE may be the controller if the model is trained at UE. Therefore, new RRC message and procedure may be introduced to support data set transmission.

- The model is delivered in L2 signalling. Current L2 protocol stack between UE and gNB includes SDAP, PDCP, RLC and MAC. The data in payload is invisible to gNB. Although gNB can understand control signalling, e.g. MAC CE or PDCP control PDU, such control signalling may not be able to carry model with large size.

- The model is delivered in new layer. As can be seen above, the AI model delivery is not fit in existing protocol layer. A new layer may be introduced to support model delivery between gNB and UE. The new layer can be defined in a bi-directional way, which fits the requirement of AI model. If open format or 3GPP format AI model is used, it’s better to define a dedicated connection to transmit the AI model. Furthermore, AI model may evolve faster than 3GPP release version. It’s easier to add or modify the AI model in separate protocol layer without impact to other legacy protocol layer.

Proposal 6: RAN2 to study following model delivery method between gNB and UE,

- The model is delivered in RRC message.

- The model is delivered in L2 signalling.

- The model is delivered in new layer signalling.

In last meeting, model ID is assumed. We understand there may be two types of mode ID. If model registration is supported, UE may need to report the AI model to NW for registration. It may be difficult for each UE to report the image of the AI model. If the AI model is common to all UEs, it may be more efficient for UE to report a model ID. Such model ID should be PLMN or globally unique to identify a AI model, so that gNB can perform registration. To achieve the PLMN or globally unique, the model ID should be allocated by NW.

Proposal 7: Model ID for model registration can be PLMN or globally unique, which could be allocated by NW.

The model ID may be long to achieve PLMN or globally unique. For the model switch, model ID can be locally unique within a UE. The model ID can be allocated by gNB or UE, depending which node generates the model. Therefore, the model ID can be much shorter, which is more efficient. During model switch, the target mode ID is indicated.

Proposal 8: Model ID for model switching can be locally unique, which could be allocated by NW or UE.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: Deprioritize the study of online training in Rel-18

Proposal 2: The AI model delivery between proprietary server and other entities has no impact on 3GPP specification and can be left to implementation.

Proposal 3: The AI model delivery between core network entity and RAN entities, i.e. UE and gNB, is out of RAN2 scope and can be studied by other groups.

Proposal 4: AI Model delivery should support large size AI model, e.g. hundreds of MB.

Proposal 5: AI model delivery should be bi-directional between UE and gNB.

Proposal 6: RAN2 to study following model delivery method between gNB and UE,

- The model is delivered in RRC message.

- The model is delivered in L2 signalling.

- The model is delivered in new layer signalling.

Proposal 7: Model ID for model registration can be PLMN or globally unique, which could be allocated by NW.

Proposal 8: Model ID for model switching can be locally unique, which could be allocated by NW or UE.

Reference

[1] RP-213585 New WID on NR sidelink relay enhancements