3GPP TSG-RAN WG2 Meeting #120	R2-2211890

Toulouse, France, November 14-18, 2022

Agenda Item:	8.11.2

Source:	MediaTek Inc.

Title:	Discuss on PTM configuration delivery for multicast in RRC INACTIVE

Document for:	Discussion, Decision

Introduction

Regarding the PTM delivery method for RRC_INACTIVE UE, RAN2 made the following agreements for Rel-18 multicast enhancement in RAN2#119-e:

For PTM configuration delivery, RAN2 further investigates the following solutions: Option 1: Dedicated signalling Option 2: Solution based on SIB+MCCH We do not preclude some “mix” of the options

Then in RAN2#119bis-e, RAN2 further discuss the baseline for these two options of PTM delivery for multicast reception in RRC_INACTIVE state.

In this contribution, we analyse the characteristic of the two options, and then provides our view and preference on the option of PTM configuration delivery.

Discussion

Options of PTM delivery for multicast in RRC_INACTIVE

Option1: Dedicated signalling

In the last meeting, RAN2 agreed the baseline of option 1 for PTM delivery:

The following general description is taken as baseline for PTM configuration delivery Option 1: - (1-a) PTM configuration(s) (i.e., configurations used for multicast reception in RRC_INACTIVE) of one or more multicast sessions for at least one cell are provided via dedicated RRC signaling to a UE. - (1-b) The RRC message for this includes RRCReconfiguration and/or RRCRelease and/or RRCResume (details FFS) - (1-c) UE stores the received configurations while it is in RRC_INACTIVE, and if there is a need to update some or all the configurations, the UE is notified of such changes and may trigger RRC connection resume to obtain the updated configurations. In case of mobility in RRC_INACTIVE, the UE triggers RRC connection resume if the configuration of the session is not available for the new cell.

In Rel-17 multicast, if a UE is interested in a multicast session, the UE shall join in before or after multicast session start. This procedure allow network to confirm that the UE which receive multicast service are authorized, which should be the baseline design principle for multicast (at least for option 1). We suggest this procedure is maintained in Rel-18 multicast enhancement, which is also a baseline for the following discussion.

Proposal 1: At least for option1, UE shall join in the multicast session before receives the multicast in RRC INACTIVE.

The time of PTM configuration

For option 1, the PTM configuration is delivered to UE by dedicated RRC signalling. UE need to first join in the multicast session, then network indicated the configuration by RRC message. According to the baseline for option1, the configuration can be configured when the session is about to start(activate), or it can be pre-configurated to UE before the session start (UE then stores the received configuration).

In the LS to SA2, we assume the RRC_INACTIVE UE can receive MBS data without going back to CONNECTED when session start. This is realized by pre-configure the PTM configuration to UE.

Observation 1: According to the LS to SA2, RAN2 assumes that UE may receive multicast in INACTIVE state without going back to CONNECTED when session start, if the PTM configuration is pre-configured to UE.

Therefore, we suggest to clarify the following cases for PTM configuration delivery before we further design the signaling:

Proposal 2: For option1, the following scenarios are allowed for the PTM configuration delivery of multicast in RRC INACTIVE:

Scenario 1: The PTM configuration can be indicated to UE when the session is about to start, or the session is already started, if UE joins late.

Scenario 2: The PTM configuration can be pre-configured to UE before the session start (and UE stored the configuration until session start). When session start, UE can receive multicast in INACTIVE state by restoring the configuration without going back to RRC_CONNECTED.

If proposal 2 can be agreed, it is clear that the information of whether UE can receive multicast in RRC_INACTIVE should be provided to UE before the PTM configuration is delivered. In order to support both scenario 1 and scenario 2 in proposal 2, the information of whether UE can receive multicast in RRC_INACTIVE should be provided to UE when UE join in the multicast session (before UE is released).

Observation 2: the information of whether UE can receive multicast in RRC_INACTIVE should be provided to UE before the PTM configuration is delivered to UE.

As shown in the following figure, if the information of whether UE can receive multicast in RRC_INACTIVE is provided before session start, there is no need to request RRC resume to obtain such information (and also PTM configuration) when session activate. This prevents the high system load which multiple UEs request PTM configuration simultaneously, and simplifies UE’s behaviour for signalling and state transitions.

Figure 1: Different times of information for whether UE can receive multicast in RRC_INACTIVE is delivered

Observation 3: Pre-config PTM configuration simplifies UE’s signalling and solves the issue when multiple UEs request PTM configuration when session activate.

Therefore, we propose the following:

Proposal 3: If UE joins the multicast session before session activated, the information of whether UE can receive multicast in RRC_INACTIVE is provided to UE when UE joins in the multicast session.

Signaling

The PTM configuration need to be delivered to UE when the first time UE request multicast session (before/when/after session start), and also when UE request configuration update. It is preferred the way of PTM configuration delivery can be unified.

If UE request PTM configuration in RRC INACTIVE, or UE is receiving multicast in RRC INACTIVE and requests PTM configuration update, RRCRelease message with suspendconfig can be used to indicate PTM configuration without letting UE resume to RRC CONNECTED. Also, if a UE is in RRC CONNECTED state, RRCRelease message can be used to indicate both PTM configuration and state transition to RRC INACTIVE. We think this is a general way for RRC CONNECTED and INACTIVE UE to obtain/update PTM configuration.

Observation 4: RRCRelease message (with suspend) is a general way to indicate PTM configuration to UE in RRC_INACTIVE state and UE in RRC CONNECTED (then go to INACTIVE).

Figure 1: Different cases for PTM configuration delivery

Therefore, we propose the following:

Proposal 4: RRCRelease message with suspendconfig is used to indicate PTM configuration and configuration update to UE for multicast reception in RRC INACTIVE state.

Mobility

RAN2 has agreed to support multicast service continuity without resuming RRC connection. In our understanding, the behaviour for mobility service continuity is similar to PTM configuration update (with the assumption that multicast in RRC_INACTIVE is supported in target cell). UE may request PTM configuration update before/after cell reselection without switching back to RRC CONNECTED, the resume cause and the way of PTM configuration delivery can be the same.

Proposal 5: If the multicast service in RRC_INACTIVE is supported in the target cell, the service continuity with mobility is considered as a kind of procedure of UE requesting PTM configuration update and shared the same way of PTM configuration delivery.

The pros and cons for option 1:

Pros:

- Network is aware of UE receiving multicast session. Only UE has been authorized can receive the corresponding multicast session, which is aligned with Rel-17 multicast and should be the baseline of the design principle.

- The PTM configuration is delivered to UE by dedicated signaling (aligned with Rel-17 multicast)

- More compatibility friendly to UE supporting Rel-17 MBS. If a multicast service is provided to UE supporting Rel-17 and Rel-18 MBS, the configuration can be delivered in different way.

Cons:

- UE may need to trigger RRC connection resume to obtain the new configuration when PTM configuration update (e.g., UE moves to another cell with different configuration).

- If the PTM configuration is changed (from the network), there may be high signalling load when a large number of UEs request PTM configuration update simultaneously.

Further optimization for option 1

Since the UE need to request RRC resume to obtain updated PTM configuration, and there may be issues when large number of UEs request simultaneously. From the network perspective, it is beneficial to reduce the frequent PTM configuration update. Since we have discussed the multicast configuration applicable area in the meeting, we proposed:

Proposal 6: Support multicast configuration applicable area to unify the configuration in a certain area. The area may contain a set of cells instead of a single cell.

Option2: MCCH-like channel

In the last meeting, RAN2 agreed the baseline of option 2 for PTM delivery:

The following general description is taken as baseline for PTM configuration delivery Option 2: - (2-a) PTM configurations (i.e., configurations used for multicast reception in RRC_INACTIVE) are provided via an MCCH-like channel (same or different as used for MBS broadcast), and information regarding MCCH scheduling is provided via SIB, FFS dedicated signalling - (2-b) UE can receive such configurations when it is in RRC_INACTIVE, FFS whether it is allowed/needed to also receive when UE is in RRC_CONNECTED - (2-c) If there is a need to update some or all the received configurations, UE does not need to resume RRC connection but is notified of such changes (e.g. via MCCH DCI) and obtains the updated configurations via MCCH.

Meanwhile, many companies show their concerns on the issue for UE to obtain PTM configuration before joining in the multicast session, therefore a further agreement is made:

FFS if there is an issue that a UE can obtain all the PTM configurations for a multicast service via Option 2 without/before joining the multicast session on the condition that security is enabled by service layer. And if yes FFS how to solve the issue (e.g., dedicated configuration + MCCH).

In our understanding, the biggest difference for multicast and broadcast is that multicast is a network-aware system, which should also be the design principle for the multicast service. For multicast, network should be aware of which UEs have the intention to receive the service and decide whether they are authorized to do that. Otherwise, we may use Rel-17 broadcast to deliver the same service instead of multicast enhancement.

Observation 5: The multicast service should be a network-aware service, in which the network is aware of UEs which intend to receive the service and decide whether it is allowed for each UE.

Proposal 7: For option 2, UE should request to join in the multicast service before being able to obtain the PTM configuration

Base on the above discussion, we think the SIB is not a good way to indicate the PTM configuration (i.e. MCCH), since it is open to all UEs. For multicast initialization, dedicate signaling may be a valid option to deliver MCCH, then the following procedure (e.g. configuration update) can be similar with broadcast.

Different with broadcast, the MCCH for multicast should be “service specific” and only contains the configuration for one multicast service. The list of neighbour cells providing the same multicast service may also be provided in this service specific MCCH.

Base on the above discussion, we propose:

Proposal 8: For option2, the multicast MCCH is service specific and contains the PTM configuration for one multicast service. FFS the neighbour cells information for multicast MCCH.

Proposal 9: For option2, dedicate signalling is used to indicate the PTM configuration (i.e. multicast MCCH).

The pros and cons for option 2:

Pros:

- Friendly to high congestion scenario (e.g. when a great number of UEs request configuration update simultaneously), since the configuration is provided by common signalling.

- When UE switch to another cell, UE may obtain the configuration from the neighbour cell information in multicast MCCH and no need to request configuration update (it depends on the detail design for multicast MCCH).

Cons:

- The original SIB+MCCH way go against the design principle that the multicast service should be received after authentication. The network may not be aware of UE receiving multicast.

- Too different from Rel-17 multicast and may be difficult for one multicast service to be supported by both Rel-17 and Rel-18 multicast.

- It seems the Rel-17 broadcast can already work well compared with the original option 2.

Our preference

Although the issue that UE may obtain PTM configuration without authentication (i.e., security issue) can be solved by introducing “dedicated signalling+ MCCH” way, it is preferred that multicast reception in RRC_INACTIVE is provided in a more compatibility friendly way (i.e., option 1), which is more similar to the legacy Rel-17 multicast.

By option1, the network may get more information from UE. E.g., which UE is receiving which multicast service, or tracking UEs at cell level for multicast reception. By option2, the network may not be aware of UE receiving multicast (original option2), or be able to know when UE moved to another cell for multicast reception.

Proposal 10: Adopt option1 (i.e., dedicate signalling) to deliver PTM configuration for multicast configuration in RRC_INACTIVE state.

Conclusion

In this contribution, the following observation is made:

Observation 1: According to the LS to SA2, RAN2 assumes that UE may receive multicast in INACTIVE state without going back to CONNECTED when session start, if the PTM configuration is pre-configured to UE.

Observation 2: the information of whether UE can receive multicast in RRC_INACTIVE should be provided to UE before the PTM configuration is delivered to UE.

Observation 3: Pre-config PTM configuration simplifies UE’s signalling and solves the issue when multiple UEs request PTM configuration when session activate.

Observation 4: RRCRelease message (with suspend) is a general way to indicate PTM configuration to UE in RRC_INACTIVE state and UE in RRC CONNECTED state(then go to INACTIVE).

Observation 5: The multicast service should be a network-aware service, in which the network is aware of UEs which intend to receive the service and decide whether it is allowed for each UE.

Based on the observation, the following proposals are made:

Proposal 1: At least for option1, UE shall join in the multicast session before receives the multicast in RRC INACTIVE.

Proposal 2: For option1, the following scenarios are allowed for the PTM configuration delivery of multicast in RRC INACTIVE:

- Scenario 1: The PTM configuration can be indicated to UE when the session is about to start, or the session is already started, if UE joins late.

  - Scenario 2: The PTM configuration can be pre-configured to UE before the session start (and UE stored the configuration until session start). When session start, UE can receive multicast in INACTIVE state by restoring the configuration without going back to RRC_CONNECTED.

Proposal 3: If UE joins the multicast session before session activated, the information of whether UE can receive multicast in RRC_INACTIVE is provided to UE when UE joins in the multicast session.

Proposal 4: RRCRelease message with suspendconfig is used to indicate PTM configuration and configuration update to UE for multicast reception in RRC INACTIVE state.

Proposal 5: If the multicast service in RRC_INACTIVE is supported in the target cell, the service continuity with mobility is considered as a kind of procedure of UE requesting PTM configuration update and shared the same way of PTM configuration delivery.

Proposal 6: Support multicast configuration applicable area to unify the configuration in a certain area. The area may contain a set of cells instead of a single cell.

Proposal 7: For option 2, UE should request to join in the multicast service before being able to obtain the PTM configuration

Proposal 8: For option2, the multicast MCCH is service specific and contains the PTM configuration for one multicast service. FFS the neighbour cells information for multicast MCCH.

Proposal 9: For option2, dedicate signalling is used to indicate the PTM configuration (i.e. multicast MCCH).

Proposal 10: Adopt option1 (i.e., dedicate signalling) to deliver PTM configuration for multicast configuration in RRC_INACTIVE state.

References

RP-213568 R18 MBS WID: Enhancements of NR Multicast and Broadcast Services

R2_119bis-e_MBS session notes (Dawid)_2022-10-19 EOM_rm

R2-2210880 Report of [AT119bis-e][605][eMBS] PTM configuration for INACTIVE (CATT)