3GPP TSG RAN WG2 Meeting #120               	R2-2211917

# Toulouse, France, 14th 18th November 2022

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on sidelink positioning

Document for:	Discussion & Decision

# Introduction

A new release-18 study item on expanded and improved NR positioning was approved in RANP#94e meeting in December 2021 [1]. The study item to develop sidelink positioning is related to the outcome of the previous study/report,  the study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" which focused on V2X and public safety use cases with the outcome being captured in TR 38.845 [2].

Additionally, SA1 has developed requirements for "Ranging based services" in TS 22.261 [3], and positioning accuracy requirements for IIoT uses cases in out-of-coverage scenarios in TS22.104 [4].

Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

At RA2#119bis e meeting [9], the following agreements were made:

Terminology:

- Proposal 1(20/20): RAN2 adopts the RAN1 definition of ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE. SA2 definition is not conflicted with RAN2 definition, there is no need to ask SA2 to follow RAN2 definition.

- Proposal 2.1 (16/20): RAN2 wait for RAN1 conclusion on the FFS part of anchor UE definition, and adopt whatever agreed by RAN1.

- Proposal 2.2 (16/20): RAN2 does not introduce the definition of located UE of SA2 for now. If there is a need to introduce it later (e.g. in the normative work), we can introduce it then.

- Proposal 2.3 (18/20): RAN2 understands that SL reference UE refers to and is aligned with anchor UE, which can be reflected in the reply LS. SL reference UE can be used in absolute positioning, relative positioning and ranging, no clarification on this from SA2 is needed.

- Proposal 5.1(18/20): RAN2 do not consider RAT-independent SL positioning in this release.

- Proposal 5.2(17/20): RAN2 adopts RAN1 definition on SL positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information. There is no need to ask SA2 to follow RAN2 definition(i.e. introduce reference signal into the definition).

- Proposal 7(20/20) (modified) RAN2 do not introduce SL positioning client UE for now.  If there is a need to introduce it later (e.g. in the normative work), we can introduce it then.

- Proposal 3 (14/20) : RAN2 think the definition of target UE from SA2 is generally aligned with RAN2, but RAN2 stick to the RAN1 definition. There is no need to ask SA2 to follow RAN2’s definition.

- Proposal 8(12/20): RAN2 to wait RAN1 on the definition of relative positioning.

- Proposal 9(13/20): Do not ask SA2 to align the use of “-assisted” with RAN2. RAN2 can tell SA2 about RAN2’s use of “-assisted”, it is up to SA2 to decide whether to align with RAN2 or not.

- Proposal 4(14/20) (modified): RAN2 do not decide to support the role of assistant UE for now.  FFS if there is spec impact in RAN2 from the assistant UE.

- Indicate in the reply to SA2 that RAN2 have not concluded on the server UE functionalities but have agreed to follow SA2 decision on the definition of the server UE, and discussion continues

Sidelink positioning;

Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work.

1. Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF

2. Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP

3. Use of SLPP/RSPP between the UE and the LMF

Proposal 3 (modified): In order to enable sidelink positioning, SLPP/RSPP shall support at least the following functionalities:

1. SL Positioning Capability Transfer

2. SL Positioning Assistance Data exchange

3. SL Location Information Transfer

4. Error handling

5. Abort

This agreement does not imply any specific signalling structure.

Proposal 5: Unicast/one-to-one operation is assumed as baseline for exchange of sidelink positioning signaling.

Proposal 6 (modified): RAN2 shall study applicability of at least the following positioning signaling for groupcast/broadcast (in addition to unicast), including addressing any security aspects (involving SA3 where needed). FFS the specific use case:

SL positioning capability transfer

SL positioning assistance data

FFS SL location information transfer

SLPP/RSPP protocol design

- Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation.  FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless.

- Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs.  RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

In this contribution, we discuss scenarios, protocol aspects and potential solutions for Sidelink positioning related to terminology, Cast types and V2X solutions

# Discussions on Sidelink positioning

## Terminology, Resource allocation, Cast types, in or out of coverage

- According to agreements at RAN2#119bis [9], it was agreed for RAN2 to not support any assistant UE.

- We see reasons where assistant UE may be highly relevant.

- In supporting SL resource allocation for a group of UEs

- To support ranging/distance estimation between two UEs based on angles (see VRU solutions below).

- (A), Supporting SL resource allocation for a group of UEs

- In some scenarios it may be beneficial or relevant (e.g., in case of out of coverage) where a group of UEs would be able to configure and manage resource scheduling/allocation for SL based positioning by themselves within the group of UEs. This could be done e.g., by introducing a central unit or assisting unit to configure and allocate SL resources for the involved UEs.

- The operation could be that Vehicle 1 should perform a positioning activity and request RSU-1 (assistant UE/anchor UE/reference UE) to configure SL-PRS transmissions from nearby UEs (assistant UEs or Reference UEs).

Figure 1. An example of the resource scheduling for SL positioning in mode 2.

- Based on the fact that the group of UEs may have different functionality, it would be relevant to further discuss and align terminology with RAN1 and SA2, where e.g., the anchor UE could both have the function of providing resource configuration as well as acting as a Reference UE transmitting SL-PRS signals.

- Proposal 1: Further discuss and align terminology with RAN1 and SA2, where e.g. the anchor UE could both have the function of providing resource configuration as well as acting as a Reference UE transmitting SL-PRS signals

For cast types for positioning signalling, clearly further discussions are needed, and different scenarios may need separate treatment. We may have to separate in-coverage, partly in-coverage, and out of coverage scenarios. Then uni-cast and groupcast types may be suitable when PC5 connection is established, while broadcast type, should work also for UEs with no PC5 connection. For in-coverage, network-controlled configuration and resource allocation can be done by LMF and gNB. While for out of coverage scenarios, mode 2, or Scheme 2: UE autonomous SL-PRS resource allocation, as agreed by RAN1 [9] is used.

Observation 1: Then uni-cast and groupcast types may be suitable when PC5 connection is established, while broadcast type, should work also for UEs with no PC5 connection.

Proposal 2: Discuss further different scenarios related to in-coverage, partly in-coverage, and out of coverage scenarios.

Proposal 3: Discuss further aspects whether the involved UEs have an PC5 connection established or UE autonomous SL-PRS resource allocation is to be used.

## Solutions for SL positioning

In V2X positioning or any other sidelink positioning use-cases, a UE may need to determine the distance to surrounding UEs. For example, a moving vehicle needs to determine whether there are surrounding VRUs (Vulnerable Road User ) and the relative distance to those VRUs. The determination operation should be performed in a quick manner. Hence, in case one or more VRUs are detected to be very close, the moving vehicle and/or VRUs can perform the necessary action to avoid potential collision. The determination of surrounding UEs (VRUs) can be facilitated by allowing concurrent transmission of SL-PRS from multiple UEs. For example, multiple VRU(s) can transmit “SL-PRS” concurrently and/or within the same set of resources. The transmitted SL-PRS can be received by moving vehicle(s), other VRU(s), or RSU(s). The concurrent/simultaneous transmission of “SL-PRS” can be arranged by allowing multiplexing of SL-RS from multiple UEs within a given set of resources (e.g., time-division multiplexing (TDM), frequency-division multiplexing (FDM), and/or Code-division Multiplexing (CDM))

Proposal 4: Consider supporting multiplexing of sidelink positioning reference signal from several UEs within a given set of resources.

The UE is expected to perform positioning measurement based on the received “SL-PRS”. The relative positioning estimation (i.e, relative distance) is obtained based on the collected positioning measurement. We consider sidelink positioning should consider the legacy positioning measurement, such as received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA). In addition to the relative distance between UEs, other parameters, such as angle of arrival (AoA), angle of departure (AoD), LOS indication, can be obtained based on those positioning measurements. RTT with AoA can also be an option for relative positioning.

Proposal 3: Relative positioning is estimated based on received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA), and/or RTT+AoA.

## Solutions for VRU scenario

One of the challenging scenarios for positioning in V2X use-cases is to handle Vulnerable Road User (VRU). This is one of the scenarios that has been identified by 5GAA as shown in Figure 1.

Figure 1: Illustration of VRU, ref. 5GAA Technical report on C-V2X Use Cases and Service Level Requirements Volume I

In Figure 1, the main purpose is to alert vehicle (Host Vehicle - HV) of VRUs approaching the road or crossing an intersection and warn of any risk of collision. Hence, to provide safety, typically in the road or crossing, there is a road side unit (RSU) that can be in a form of road sign or traffic light. In V2X context, HV, VRU and RSU are equipped with communication module that can support both Uu and PC5 interfaces. In order to avoid the collision, the position of VRU and HV should be known, at least the relative distance. RSU can be used to assist the position of VRU and HV. The positioning estimation can be performed in VRU, HV, RSU, or LMF, depending on the context / purposes.

Observation 2: In V2X scenario, some UE(s) may be stationary (e.g., a UE in RSU). This type of UE can be used to facilitate positioning procedure, such as to perform as reference UE for positioning.

The radio resources for the SL-PRS transmissions are limited. Furthermore, it should also consider the following circumstances:

- The need to perform positioning measurement and estimation in a timely manner. Hence, the transmission, in some cases, may require low latency and high accuracy.

- UEs (VRUs) distribution that can be significantly changed. In one particular time, it can be many UEs (rush hours) and another particular time, it can be a few UEs (midnight time).

The usage of radio resources and the SL-PRS transmission should be controlled (e.g., by gNB) so that sidelink-positioning can be used, particularly to support V2X use-cases (e.g., collision avoidance) where the positioning estimation shall be obtained in a timely manner. Network-controlled radio resources have been supported in legacy sidelink for communication and known as Mode-1. In a junction scenario and during rush hour, the number of VRUs are expected to be high. There should be a mechanism to control which UEs that are allowed to transmit. For example, the RSUs can be configured to transmit SL-PRS and the VRUs are configured to receive SL-PRS. Outside of rush-hours, there could be the opposite case where VRUs are configured to transmit SL-PRS. In this example, UE-types classification for V2X positioning (e.g., RSU, VRU, Car) can be beneficial for V2X positioning. RAN2 can consider sending LS to  SA2 with respect to this.

Proposal 7: Consider to support UE-types classification for V2X positioning (e.g., RSU, VRU, Car).

The concept of region / zone has been commonly used in sidelink for communication. In a given zone, a UE is expected to perform communication using a specific carrier frequency. Similar concept of region / zone can be applied for positioning purpose, such as to adapt positioning procedure based on the region/zone of the UE. For example, if a UE is in a cross-junction, the UE is expected to perform frequent positioning procedure (e.g., positioning measurement / estimation), particularly to detect and avoid potential collision.

Proposal 8: Consider to adapt positioning procedure based on the region/zone of the UE.

Sidelink positioning relies on the interaction, such as transmission and reception, between UEs. For example, a UE-A transmits SL-PRS and another UE-B receives it and performs a positioning measurement. From the positioning measurement, the UE-B is expected to be able to estimate the relative position (distance), such as UE-A is X meters away from UE-B. UE-B would still not be able to know the direction of UE-A relative from UE-B.

Figure 2: Illustration of relative positioning without angle information

Obtaining the relative positioning and relative angle are needed in certain use-cases. One of the use-cases is the use-cases identified in ranging based services technical report [5]. Furthermore, the work in SA2 [6], has identified a key issue on supporting ranging service operation procedure with the assistance of another UE. The assistance of another UE can be used to estimate both relative positioning (distance) and angle as illustrated in Figure 3.

Figure 3: Illustration of ranging between host and target UEs, with help form an assistant UE.

We consider RAN1 should also study this aspect, particularly how the physical layer aspects and the interaction with higher layer can support this procedure.

Proposal 9: Consider supporting positioning procedure with the assistance of another UE for the estimation of relative positioning and relative angle.

# Summary

In this contribution, we have discussed our view on potential positioning enhancements in NR. Our observation and proposals are listed below:

- Proposal 1: Further discuss and align terminology with RAN1 and SA2, where e.g. the anchor UE could both have the function of providing resource configuration as well as acting as a Reference UE transmitting SL-PRS signals

Observation 1: Then uni-cast and groupcast types may be suitable when PC5 connection is established, while broadcast type, should work also for UEs with no PC5 connection.

Proposal 2: Discuss further different scenarios related to in-coverage, partly in-coverage, and out of coverage scenarios.

Proposal 3: Discuss further aspects whether the involved UEs have an PC5 connection established or UE autonomous SL-PRS resource allocation is to be used.

Proposal 4: Consider supporting multiplexing of sidelink positioning reference signal from several UEs within a given set of resources.

Proposal 5: Relative positioning is estimated based on received power measurement (e.g., RSRP) and/or timing measurement (e.g., TDOA), and/or RTT+AoA..

Proposal 6: Support hybrid positioning where the UE receives reference signal for positioning from both direct-link (Uu) and sidelink (PC5) and jointly utilize for positioning estimation purpose.

Observation 2: In V2X scenario, some UE(s) may be stationary (e.g., a UE in RSU). This type of UE can be used to facilitate positioning procedure, such as to perform as reference UE for positioning.

Proposal 7: Consider to support UE-types classification for V2X positioning (e.g., RSU, VRU, Car).

Proposal 8: Consider to adapt positioning procedure based on the region/zone of the UE.

Proposal 9: Consider supporting positioning procedure with the assistance of another UE for the estimation of relative positioning and relative angle.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- TR38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”, Rel-17

- TS22.261, Ranging based services"

- TS22.104, positioning accuracy requirements for IIoT uses cases in out-of-coverage scenarios

- TR 22.855 Study on Ranging-based Services

- TR 23.700-86, XXX

- RAN2#119bis-e meeting