3GPP TSG-RAN WG2 Meeting #120  	                            R2-2211938

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.12.3

Source: 	Sony

Title:	Handling of UE´s in RRC-Inactive mode

Document for:	Discussion

# Introduction

A new work item for Mobile IAB WID [RP-221815] has been agreed for Rel-18, with the following objectives.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2]

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

- Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, we discuss the handling of UE´s in RRC Inactive when it comes to RNA Updates in Rel-18 mobile IAB.

# Discussion on handling of UE´s in RRC Inactive

In the Vehicle Mounted Relay study (FS-VMR) for Rel-18 in SA2 TR 23.700-05, [2] there is the following key Issue: 5.3 Key Issue #3: Efficient mobility and service continuity when served by mobile base station relay

The Key issue above is about when the UE is moving together with the MBSR (Mobile Base Station Relay). Two mobility scenarios are detailed in the key issue description. The UEs traveling along with the MBSR could have different states:

1)	UE is in CM-Idle state and moves together with the MBSR.

2)	UE is in CM-Connected and moves together with the MBSR. This scenario includes both RRC-Connected state and RRC-Inactive state.

When several UEs and the MBSR moves together, it looks tempting to optimize the handling of the UE. The optimization would be either to reduce UE to network signalling or RAN to CN signalling. However, there a few aspects that makes such optimization difficult e.g.:

-	UE mobility within the Registered Area (RA) while the UE is in CM-Idle is not tracked. The current location could be updated when the UE performs SR or periodic registration.

-	The different UEs may belong to different slices, and the handling of them should be separated (slice separation)

-	The different UE may be configured with different Tracking Area List, and have different RA.

The Study in SA2 have discussed when the UE is in RRC-Idle/CM-Idle and RRC-Connected/CM-Connected, but the state RRC-Inactive/CM-Connected have not been discussed. SA2 initial conclusion from the last SA2#153E-mmeting is captured below.

For KI#3, the interim conclusions are as follows: - During MBSR's mobility, the TAC broadcasted by the MBSR is the same as the TAC of the cell where the IAB-UE is located. When the IAB-UE enters to a new TA, the cell broadcasting information is updated accordingly. - The UE's mobility management is performed using the legacy mechanism as defined in the TS 23.501 [2] and TS 23.502 [5]. The UE in CM-Idle shall follow legacy procedure when detecting a TAC which is not in the TA list. - Each UE connected via the MBSR may have different serving AMFs e.g., due to slicing and individual PDU sessions/QoS service flows configured. UE context handling and path switching would be handled per each individual UE. Editor's note: The above interim conclusion and support of group mobility are subject to feedback of RAN WGs and will be revisited as per the feedback. Editor's note: Regarding the alternatives assuming MBSR operates with dedicated TAC(s), it is subject to the feedback of RAN WGs. Editor's note: For UEs in RRC-Connected/CM-Connected state via an MBSR, whether and how to guarantee UE's service continuity is subject to the IAB full migration/mobility in the RAN Rel-18 study. Editor's note: The alternatives assuming MBSR operates with dedicated TAC(s) or TAC from Donor are subject to the feedback of RAN WGs.

Observation 1: Mobility of MBSR and UE when the UE is in RRC-Inactive has not been addressed in SA2.

An RNA can be defined either by an RNA-ID or a list on cells. For the case when the RNA is defined by a RNA-ID, the MBSR could broadcast the same RNA-ID as the donor-gNB. However when the RAN is defined with a list of cells needs some more attention.

When the MBSR (Mobile IAB node) is moving and relocating to another IAB-doner CU, which is belonging to the same RAN Notification Area as the original IAB doner CU. The MBSR may get a new Cell-id/PCI when a MBSR is integrated in the new IAB donor CU. But for a UE that is camping on the MBSR in RRC-Inactive mode, this may lead to an unwanted RNA Update by the UE, since the new cell-id/PCI may not be in the list of cells configured previously when release to RRC-Inactive.

Observation 2: Assigning the MBSR a new cell-id/PCI may trigger unwanted RNA Update

An alternative to dynamic PCI assignment above would be assigning the MBSR with a static cell-id/PCI. In that case the UE can camp on the mobile IAB node as they move along together, but this may lead to that the UE is not triggering an RNA Update, when the MBSR in fact is leaving the defined RAN Notification Area.

Observation 3: In case the MBSR has a fixed cell-id/PCI, they UE may not detect when it moves out of the RNA and will not trigger any RNA Update.

Somehow the UE needs to be configured with a RNA including the support for that the MBSR can get an updated cell-id/PCI, and still the UE should be able to understand whether it is camping and roaming around in the assigned RNA or it is moving outside the RNA and has to trigger an RNA Update.

Proposal 1: Discuss how the UE can be configured with a RNA including the support for that the MBSR can get an updated cell-id/PCI, and still the UE should be able to understand whether the UE is roaming on the assigned RNA or has to trigger an RNA Update

One potential solution could be to define a list of potential Cell-ID/PCI that the MBSR could be assigned with when moving around, changing IAB Doner CU. If the list of cells could be part of the RNA and configured to the UE, this would avoid any risk for the UE to trigger an unwanted RNA Update.

Proposal 2. Define a list of Cell-ID/PCI that can be used in the future for updating the Mobile IAB-nodes/MBSR) as they may change location, and that is part of RNA cell list.

# Conclusion

We propose RAN2 to consider the proposals as follows.

Observation 1: Mobility of MBSR and UE when the UE is in RRC-Inactive has not been addressed

Observation 2: Assigning the MBSR a new cell-id/PCI may trigger unwanted RNA Update

Observation 3: In case the MBSR has a fixed cell-id/PCI, they UE may not detect when it moves out of the RNA and will not trigger any RNA Update.

Proposal 1: Discuss how the UE can be configured with a RNA including the support for that the MBSR can get an updated cell-id/PCI, and still the UE should be able to understand whether is roaming on the assigned RNA or has to trigger an RNA Update

Proposal 2. Define a list of Cell-ID/PCI that can be used in the future for updating the Mobile IAB-nodes/MBSR) as they may change location, and that is part of RNA cell list.

# References

- Mobile IAB WID [RP-221815

- SA2 TR 23.700-05