3GPP TSG-RAN WG2 #120	R2-2211955

Toulouse, France, November 2022

Agenda Item:	8.3.4

Source:	OPPO

Title:	Discussion on cell selection/reselection

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on cell selection/reselection, for which RAN2#119bis-e meeting reached the following agreements.

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

Discussion

In RAN2#119bis-e meeting, RAN2 assumes to consider both frequency level and cell level cell (re)selection (de)prioritization. One open issue is whether the existing mechanism is sufficient.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

RAN2 also considers that the prevention of the legacy UEs from the NES cell is not mandatory. It implies that sometimes the NES cell even allows the legacy UE to select or camp on.

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

We understand that the decision on whether the legacy UE is barred also affect the cell (re)selection mechanism of NES-capable UE since the NES cell needs to distinguish the legacy UE and the NES-capable UE. Thus, the discussion on the necessity of the enhanced cell (re)selection mechanism should be split into two cases.

- Case1: The NES Cell prevents the legacy UEs from accessing.

- Case2: The NES cell is compatible with both the legacy UE and the NES-capable UE.

The analysis is shown below.

- Case1: The NES Cell prevents the legacy UEs from accessing.

As assumed in RAN2, two options can be considered for the legacy UE barring.

      - Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

In legacy, the gNB indicates the intra-/inter frequency excluded cell list via SIB3/4, which implies that such excluded cell list is only applicable for cell reselection. While, the cellBarred or cell reservation field is normally provided via MIB/SIB1, thus this option is applicable for both cell selection and reselection.

- Case 1-1: Cell selection

If the case is cell selection and the candidate camping cell is an NES cell, we understand that only one type UE (i.e. NES-capable UE) can access the NES cell and thus all cell selection parameters are for such UE type (i.e. NES-capable UE). Considering the prioritization or de-prioritization decision on NES-capable UE reflects the appeal of the network itself (e.g. energy saving or load balance), there is no need to have the differentiated handling for NES-capable UEs. Thus, the current cell selection mechanism is sufficient for this case.

- Case 1-2: Cell reselection

If the case is cell reselection and the neighbour cell is an NES cell, we understand that the gNB can use Intra/InterFreqExcludedCellList and/or cellBarred/cell reservation field to block the legacy UE reselecting to the NES cell. Also, there is no need to distinguish different NES-capable UEs for a specific NES cell. Thus, we think the current cell reselection mechanism is sufficient for this case.

In the case that the legacy UEs are prevented from the NES cell, RAN2 considers to reuse the current cell (re)selection mechanism for NES-capable UEs, i.e. no enhancement is needed.

- Case2: The NES cell is compatible with both the legacy UE and the NES-capable UE

As analysed above, we prefer a unified behaviour of the NES-capable UE in performing cell selection or reselection, since we understand that (de)prioritization decision reflects the appeal of the network itself (e.g. energy saving or load balance) other than the UE-specific requirements.

In the cell (re)selection, RAN2 assumes a unified behaviour of the NES-capable UEs in performing cell selection or reselection to an NES cell.

- Case 2-1: Cell selection

Similar to the above, only the way using cellBarred or cell reservation field is applicable for cell selection. In this case, if the candidate camping cell is an NES cell, two types of UEs (i.e. the legacy UE and NES-capable UE) are allowed to access the same cell and thus the NES cell needs to support different handlings for different UE types, otherwise, the intention of introducing the NES cell is unconvinced. Therefore, for the NES-capable UE, we prefer RAN2 to consider that gNB indicates the cell-level NES-specific cell selection parameters, or, the UE dynamic adapts cell selection parameters if the cell is an NES cell.

- Case 2-2: Cell reselection

If the case is cell reselection and the neighbour cell is an NES cell, it is obvious that RAN2 needs to consider the individual handling for the different UE types (i.e. the legacy UE and NES-capable UE). Thus, the issue now turns to how to support the NES-specific handling for the NES-capable UE.

In legacy, both the frequency priority and the cell-specific parameters are used in the cell reselection procedure. The frequency priority is normally considered in the inter-frequency cell reselection and cell-specific parameters are considered in the inter/intra-frequency cell reselection. Since the RRCRelease message can include the frequency level cell reselection configurations for the dedicated UE, we think the simplest way in RAN2 is just to consider cell-level NES-specific cell reselection parameters. In some implementations, the gNB of the NES cell additionally indicates the cell-level NES-specific cell reselection parameters (e.g. cell-level parameters used in the R/S criteria, or, another list of Intra/InterFreqExcludedCellList). In other implementations, the gNB of the NES cell can indicate the NES-capable UE to (de)prioritize the NES cell(s) and then the UE dynamically adapts cell reselection parameters for the indicated NES cell(s).

In the case that both the legacy UE and the NES-capable UE can (re)select the NES cell, RAN2 considers to introduce cell-level NES-specific cell (re)selection parameters to have individual handling for the UEs with/without the NES capability.

As mentioned above, we prefer a unified behaviour of the NES-capable UEs in performing cell selection or reselection to an NES cell. For a specific NES cell, all NES-capable UE would prioritize or deprioritize such a cell and the (de)prioritization decision depends on the gNB strategies/implementation. In some cases, the gNB may let the NES-capable UE prioritize the NES cell to balance the load. In other cases, the gNB may let the NES-capable UE deprioritize the NES cell if the network wants more energy-saving gains. Such a decision reflects more on the appeal of the network itself other than UE-specific requirements.

RAN2 assumes to rely on the network strategies or implementation on whether/when to prioritize or deprioritize the NES cell for the NES-capable UEs.

Conclusion

We have the following proposals:

Proposal 1	In the case that the legacy UEs are prevented from the NES cell, RAN2 considers to reuse the current cell (re)selection mechanism for NES-capable UEs, i.e. no enhancement is needed.

Proposal 2	In the cell (re)selection, RAN2 assumes a unified behaviour of the NES-capable UEs in performing cell selection or reselection to an NES cell.

Proposal 3	In the case that both the legacy UE and the NES-capable UE can (re)select the NES cell, RAN2 considers to introduce cell-level NES-specific cell (re)selection parameters to have individual handling for the UEs with/without the NES capability.

Proposal 4	RAN2 assumes to rely on the network strategies or implementation on whether/when to prioritize or deprioritize the NES cell for the NES-capable UEs.

Reference

[1] RP-213554  New SI: Study on network energy savings for NR

[2] R2-2211428, Report of [POST119bis][304][NES] TP on cell selection／reselection and SSB／SIB-less (Huawei)

Annex – TP for cell selection/reselection

### 6.X.1	Cell selection/reselection

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

- Use IntraFreqExcludedCellList/InterFreqExcludedCellList

- Use the cellBarred or cell reservation fields in MIB/SIB

Editor's note: FFS whether to keep the terminology of “NES cells” and its definition, or change it to“a cell that uses an NES technique”.

Editor's note: FFS the exact mechanism and the spec impacts.

The NW should be able to configure NES-capable UEs to prioritize/de-prioritize a specific NES cell or NES cells on a specific frequency. A unified behaviour is assumed for the NES-capable UEs in performing cell selection or reselection to an NES cell. In the case that both the legacy UE and the NES-capable UE can (re)select the NES cell, cell-level NES-specific cell (re)selection parameters can be considered to have individual handling for the UEs with/without the NES capability.

Editor's note: FFS whether the existing mechanism for cell prioritization/de-prioritization is sufficient.