3GPP TSG-RAN WG2 Meeting #120	R2-2211967

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reselection and Access control for NES

WID/SID:	FS_Netw_Energy_NR - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 is starting to work on study item on NW energy saving (RP-220297) with the scope:

The objectives of the study are the following:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study.

The following example scenarios are listed in no particular order.

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

Note 2: the study of energy savings specifically for IAB is not part of the scope.

The study should coordinate with RAN4 as needed.

RAN2 objectives are to identify possible techniques to improve NW energy efficiency for base station transmission and reception. The work item does not mention separately CONNECTED and IDLE/INACTIVE techniques but targeting different states will likely have quite different techniques. It is also notable that the SI will focus on low load scenarios.

In RAN2#119bis we reached following agreements:

Agreements:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

In this paper we discuss how network that has NES cells deployed could operate without causing too severe impacts to non-NES UEs.

# 2 	Discussion

Whenever the gNB (or one of the network entities it controls, such as a cell, a DU, a CU) operates in a network energy saving (NES) mode such as a dormant state, this entails that some of its hardware components are switched off or kept in a sleep mode to obtain network energy reduction. As a result, the capability of the gNB to provide services to the end users may be temporarily reduced until the energy saving mode is exited. Accordingly, there can be a trade-off between the energy to be saved and performance to be provided. For example, the provided performance during the NES mode (e.g.  in terms of bit rate, latency, number of served UEs, amount of data served, etc.) can be limited or degraded, and data transfer or initial access can be inhibited altogether for some UEs -  especially legacy UEs. To enable increased performance or revert to the normal performance level, the network may want to exit the NES mode, however this comes at the price of an increased energy consumption.

It also seems that the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signalling) without upper layer signaling (e.g. RRC signalling). This may cause issues how the upper layer signaling can control UEs – especially UEs in IDLE/INACTIVE.

Network could choose to control for example that some UEs will not camp at all on NES cells or then they could allow camping but limit the accesses to the system while operating in NES mode.

## 2.1 	Mobility

When a cell is applying NES mode with some of the solutions under discussion, e.g. SIB-less or adaptive SSB/SIB etc, the legacy UEs might not be able to camp on/access the NES cell, while the UEs supporting NES can. Additionally it may be desired by NW that even if legacy UEs could camp they would prefer camping on cells not utilizing NES functionalities.

It seems quite reasonable to assume that NES does not need to be utilized on whole frequency but just on subset of cells – e.g. in locations with more UE density it might be better to keep cells on a frequency not using NES. Thus we think RAN2 should be considering solutions for mobility that handle both intra- and inter-frequency mobility.

As the mobility in RRC_CONNECTED is network controlled it seems possible with already existing signaling to control where UEs will operate in RRC_CONNECTED mode. But even for these scenarios it might be required to be able to quickly move many UEs to different cell when cell starts to operate in NES mode as we discuss in R2-2210185.

In the RRC_IDLE/INACTIVE mobility is controlled by reselections and mostly with broadcast signaling although it is possible to provide dedicated priorities. As the reselection priorities as such only target whole frequency they are not suitable at least as lone solution for mixed deployment frequencies (both NES and non-NES cells on same frequency).

Observation 1: RAN2 should consider in reselection handling scenarios where whole frequency is utilizing NES (e.g. SCell frequency turned off) as well as only subset of cells on a frequency utilize NES

As we can expect that there will be UEs in the system in future which understand NES deployments and UEs which do not (legacy UE) -  any solutions should be able to provide mechanisms that will handle both type of UEs. Thus it seems necessary that solutions (if any) introduced for NES reselection control should be able control legacy UE camping on a NES utilizing cells. One could either

Observation 2: RAN2 should have a mechanism where legacy UEs could be control camping on cells/frequencies utilizing NES

Cell reselection priorities control camping on frequencies in NR and for intra-frequency mobility UEs are controlled via offsets as well as allowed/excluded cell lists. It seems easy to limit legacy UEs to camp on NES cells by listing those cells in the excluded cell lists.

Observation 3: NW could handle legacy UEs not to reselect to NES applying cells by utilizing Intra/InterFreqExcludedCellList

However the UEs supporting NES should still be able to reselect to those NES cells if intended by the NW, even if they are in the excluded cell list to block the legacy UEs. The NW could indicate those NES cells so that the NES capable UEs would consider them as allowed.

If one agrees to be able to derive different exclude cell list for NES capable UEs (compared to legacy UEs) then those cells won’t be used by UEs for cell reselection as defined by 38.304:

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

Thus cell reselection can be controlled effectively with exclude-listed cells to avoid legacy and allow NES UEs to reselect to NES cells. As the signaling of exclude-listed cells is done for both intra and inter-frequency cells it would be possible to steer UE reselection via that handling both intra and inter-frequency cases.

Observation 4: exclude-listed cells can control UE reselection to both intra- and inter-frequency cells

One could also consider using some NES specific cell reselection priorities but that won’t be able to handle intra-frequency scenarios at all

Observation 5: NES specific cell reselection priorities cannot handle intra-frequency scenarios

In RAN2#119bis it was agreed:

The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient

As was discussed above purely exclude-listed cells can handle also this agreement without requiring additional mechanism as the exclude cell list can also indicate whole list as excluded

Observation 6: With exclude-cell list one can prevent UE utilizing all the PCIs of a frequency for reselection

This leads to us to a proposal to start exclude-cell list as baseline for reselection mechanism to prevent legacy UEs to reselect to NES cells as well as allow NES capable UE to reselect to those.

Proposal 1: As a baseline to handle reselection steering to/out of NES cells: the NES capable UEs could derive different excluded list than the legacy UE does based on Intra/InterFreqExcludedCellList.

Additionally one would need to be able to control also cell selection and camping on NES cell for legacy UEs (and still allow NES UEs to camp on those). This can be easily achieved by using NES specific cell barring i.e. legacy UEs follow legacy barring but NES capable UEs would still need to check whether NES specific barring is present or not before making decision that cell is barred for NES capable UE:

Proposal 2: To control cell camping on NES cells: Agree to introduce NES specific cell barring mechanism

## 2.2 	Access control

As we can also have mixed deployments then it becomes more complex to control UEs (NES capable or legacy UE) to camp on some cells on frequency. Especially NES capable UEs should be allowed to camp on NES cells but this could be problematic if large of amount of NES capable cells arrive to the NES utilizing cell simultaneously. Network would need to have control on how those UEs access the cell.

Currently, the network can configure limitations to the initial access using the UAC framework. The UAC is however defined to mitigate network congestion. It seems quite likely that when network is operating in NES mode it cannot handle efficiently many UEs and thus it becomes critical that network can effectively prevent new access in the cell. The UAC information is transmitted in system information (SI), and thus requires SI modification for any changes to be made to the access barring. This implies that currently the network cannot adjust the barring information dynamically, and requires SI re-acquisition. And as the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signalling) without upper layer signaling (e.g. RRC signalling), a slow update of access barring or cell barring can become difficult to manage.

Proposal 3: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.

# 3	Conclusion

Observation 1: RAN2 should consider in reselection handling scenarios where whole frequency is utilizing NES (e.g. SCell frequency turned off) as well as only subset of cells on a frequency utilize NES

Observation 2: RAN2 should have a mechanism where legacy UEs could be control camping on cells/frequencies utilizing NES

Observation 3: NW could handle legacy UEs not to reselect to NES applying cells by utilizing Intra/InterFreqExcludedCellList

Observation 4: exclude-listed cells can control UE reselection to both intra- and inter-frequency cells

Observation 5: NES specific cell reselection priorities cannot handle intra-frequency scenarios

Observation 6: With exclude-cell list one can prevent UE utilizing all the PCIs of a frequency for reselection

Proposal 1: As a baseline to handle reselection steering to/out of NES cells: the NES capable UEs could derive different excluded list than the legacy UE does based on Intra/InterFreqExcludedCellList.

Proposal 2: To control cell camping on NES cells: Agree to introduce NES specific cell barring mechanism

Proposal 3: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.