3GPP TSG-RAN WG2 Meeting #120	 R2-2211984

Toulouse, France, 14 - 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1481 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the unified TCI-state configuration for 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the RAN1 CR R1-2210790 agreed in-principle in RAN1#110bis-e, the Rel-17 unified TCI state and the legacy TCI state cannot be simultanuously configured for a UE in the same band. If the legacy TCI state is not configured when the unified TCI-state is configured, the MAC CE is not able to indicate the TCI-state for PDCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 6.1.3.15 and 6.1.3.44, it is clarified that if the Rel-17 unified TCI state is configured, the TCI State ID field indicates the TCI state configured by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the legacy TCI state is not configured when the unified TCI-state is configured, the MAC CE is not able to indicate the TCI-state for PDCCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.15, 6.1.3.44 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Modificatio

#### 6.1.3.15	TCI State Indication for UE-specific PDCCH MAC CE

The TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-StatesToAddModList and tci-StatesToReleaseList in the PDSCH-Config in the active BWP or by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID or by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. The length of the field is 7 bits.

Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE

Next Modification

#### 6.1.3.44	Enhanced TCI States Indication for UE-specific PDCCH MAC CE

The Enhanced TCI States Indication for UE-specific PDCCH MAC CE is identified by a MAC PDU subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI state IDi: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID or by dl-OrJoint-TCI-State-ToAddModList and dl-OrJoint-TCI-State-ToReleaseList in the PDSCH-Config in the active BWP or the reference BWP. The length of the field is 7 bits.

NOTE 1:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

NOTE 2:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is applied only if sfnSchemePdcch is configured.

Figure 6.1.3.44-1: Enhanced TCI States Indication for UE-specific PDCCH MAC CE

End of  Modificatcion