3GPP TSG-RAN WG2 Meeting #120	                       R2-2211988

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.7.3

Source:	Samsung

Title:	Network Verified UE Location

Document for:	Discussion & Decision

# Introduction

In RAN2#119bis-e (SA2#153E) meeting, SA2 reply LS [1] confirmed that it is possible to leverage the LCS framework for verification of UE location in NTN. More specifically, that the AMF can perform UE location verification and may use assistance information from NWDAF (i.e., analytics containing UE location information) in order to assist in the location verification decision.

In this contribution, we discuss aspects of the SA2 reply LS [1] and provide proposals for corresponding agreements in RAN2.

# Discussion

In RAN2#119-e meeting, RAN2 informed SA2 that it is considering the re-use of LCS framework for network verified UE location information in NTN [2].

RAN2 is considering the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN. RAN2 would like to inform SA2 about this agreement and ask for any related feedback.

In RAN2#119bis-e (SA2#153E) meeting, SA2 reply LS indicated the following [1]:

SA2 has discussed the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN and the solutions to support the network verified UE location in Rel-18 FS_eLCS_Ph3 study in TR 23.700-71. SA2 has concluded that the following aspects are used as basis for normative work: - Verification of UE location provided via satellite access should be performed leveraging the LCS framework at the 5GC. - The AMF is the entity in charge of providing the location verification decision, in line with Rel-17 mechanism of UE location verification. - The AMF may trigger location service procedures as defined in TS 23.273 to determine the UE location verification decision and optional TAI determination. Location information received at AMF is provided by LMF via the NI-LR procedure. The LMF may decide specific positioning methods to be used for verification based on RAN WG decisions. - The AMF may receive assistance information from NWDAF (i.e. analytics containing UE location information) to perform the location verification decision.

Considering that SA2 confirmed the feasibility to leverage LCS framework for location verification in NTN:

Proposal 1: RAN2 to agree the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN.

Moreover, in RAN2#119bis-e, RAN2 discussed proposals from the offline on Rel-18 NR NTN Network verified UE location [3], and agreed the following [4]:

Agreements: - RAN2 assumes that the network is able to compute possible UE locations independently from the GNSS location reported by UE - RAN2 assumes that the UE location verification procedure can be triggered by the CN and it is up to the CN to decide when to trigger the procedure - RAN2 assumes that the verification of the consistency (within 5-10 km) between the actual reported UE location with the UE location(s) computed by the network is up to the 5GC. (this doesn’t mean that RAN2 has nothing to do for this WI objective)

SA2 agreed that the AMF is the entity in charge of the UE location verification decision and the triggering of location service procedures.

Observation 1: SA2 confirm that the AMF may trigger location service procedures to determine the UE location verification decision.

Moreover, in our understanding, the AMF could use NWDAF analytics (i.e. statistics and/or predictions) from the 5GC, as defined in TS 23.288 [5], related to the UE location (e.g. UE mobility analytics and/or UE expected behaviour analytics) in current or previously visited countries, to determine whether the UE reported location information is potentially reliable and trustable or not. Then the AMF could either accept or terminate/reject the UE access attempt to the CN in a given country. This understanding on using NWDAF analytics was also confirmed in the SA2 reply LS [1]:

The AMF may receive assistance information from NWDAF (i.e. analytics containing UE location information) to perform the location verification decision.

Observation 2: The CN-based location verification solution can use assistance information from NWDAF to verify UE reported location information.

One remaining point that is worth discussing in RAN2 is how to handle UEs (Rel-17 and Rel-18 UEs) that do not support this new feature of network verification of their reported location information.

Proposal 2: RAN2 to discuss possible handling of UEs that do not support the new feature of network verification of UE location information.

# Conclusion

In this contribution we discussed aspects of the SA2 reply LS in [4] and provide proposals for corresponding agreements in RAN2. The following are the observation and proposals in this contribution:

Observation 1: SA2 confirm that the AMF may trigger location service procedures to determine the UE location verification decision.

Observation 2: The CN-based location verification solution can use assistance information from NWDAF to verify UE reported location information.

Proposal 1: RAN2 to agree the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN.

Proposal 2: RAN2 to discuss possible handling of UEs that do not support the new feature of network verification of UE location information.

# References

[1] R2-2211129	Response LS on LCS framework for Network verified UE location (NTN)

[2] R2-2208779 LS on LCS framework for NW verified UE location.

[3] R2-2210841	[AT119bis-e][102][R18 NR-NTN] NW verified UE location

[4] RAN2#119bis-E Chairman Notes

[5] TS 23.288 Architecture enhancements for 5G System (5GS) to support network data analytics services