3GPP TSG RAN WG2 Meeting #120                                                        	R2-2211990

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.16.2

Source:	Samsung

Title:	AI/ML Model Management

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#119bis-e meeting, RAN2 discussed the Rel-18 study on AI/ML for NR air interface [1] and concluded the following assumptions and agreements on the organisation aspects and AI/ML methods [2]:

### 8.16.1 Organizational - Assume that RAN2’s work can be somewhat split: A) use-case-centric configuration, signalling and control procedures, B) management of data and AI/ML models (where part of discussion may overlap between use cases). - Assume that e.g. for the management of data and AI/ML models, RAN2 could start by focusing on data collection, model transfer, model update, model monitoring and model selection/(de)activation/switching/fallback (to the extent needed), whether UE capabilities has a role in this. - Chair assumes that we will input on various aspects when the time is right, and e.g. postpone things that obviously need R1 decisions, but there could be some rare exception. ### 8.16.2 AIML methods Assume that R2 will reuse terminology defined by R1 to the extent possible/reasonable Observation: the collaboration levels definitions doesn’t really clarify what is required, more work is needed R2 assumes that for the existing (under discussion) AI/ML use cases, proprietary models may be supported and/or open format may be supported (and maybe RAN2 doesn’t have to further elaborate on this assumption). R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. R2 assumes that a model is identified by a model ID. Its usage is FFS. General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

In this contribution, we discuss model management aspects considering latest related RAN1 agreements.

# 2. Discussion

In RAN2#119bis-e meeting, RAN2 discussed Rel-18 study on AI/ML for NR air interface, and decided to focus on topics of management of data and AI/ML models and capability indication. This outcome is aligned with the SID objectives [1].

The following are RAN2 agreed initial assumptions on the work plan for this study [2]:

- Assume that RAN2’s work can be somewhat split: A) use-case-centric configuration, signalling and control procedures, B) management of data and AI/ML models (where part of discussion may overlap between use cases). - Assume that e.g. for the management of data and AI/ML models, RAN2 could start by focusing on data collection, model transfer, model update, model monitoring and model selection/(de)activation/switching/fallback (to the extent needed), whether UE capabilities has a role in this. - Chair assumes that we will input on various aspects when the time is right, and e.g. postpone things that obviously need R1 decisions, but there could be some rare exception.

Observation 1: RAN2 work on management of data and AI/ML models to be focused on data collection and model transfer, update, monitoring, selection, (de)activation, switching and fallback.

Moreover, RAN2 agreed to define a model ID, e.g. to support handling of the model in LCM procedures [2]:

R2 assumes that a model is identified by a model ID. Its usage is FFS.

In our understanding, the network and UE may use a model ID in the model LCM procedures. For example, model transfer, model update, model monitoring, model selection, model (de)activation, model switching and fallback procedures. Additionally, the model ID can be used during exchange of information related to model operation and functionality. This understanding is aligned with the following agreement in RAN1#110bis [3]:

Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

Proposal 1: The network and UE may use the model ID in model LCM procedures and exchange of information associated with model operation and functionality.

The network and UE may exchange information associated with the model operation and model functionality (or use case). For example, information on models available at the network and UE, including model ID, related use case, model operation (e.g. model training or inference is UE-sided, Network-sided or two-sided).

Proposal 2: The network and UE may exchange information on models available in the network and UE, including model ID, use case and model operation (i.e. UE-sided, Network-sided, or two-sided).

Additionally, RAN1 agreed that the network or UE may trigger model selection, activation, deactivation, switching, and fallback, at least for UE-sided and two-sided models [3]:

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network

This means that the network or UE may perform model selection, (de)activation, switching, and fallback at the UE, for UE-sided and two-sided models.

Observation 2: The network or UE may decide to perform model selection, (de)activation, switching, and fallback at the UE, for UE-sided and two-sided models.

Regarding the case of “Decision by the network”, we see two options:

- Option (a): The NG-RAN may decide the selection, (de)activation, switching and fallback of a UE-sided or two-sided model. In this case, the NG-RAN decision could be based, for example, on information on UE capability to support AI/ML operation and/or models supported/available at the NG-RAN and UE.

- Option (b): The NG-RAN may decide the selection, (de)activation, switching and fallback of a UE-sided or two-sided model based on UE request. This request may include additional information on, e.g. model ID, use case, applicable scenario/configurations, and model operation side (i.e. training or inference UE-sided or two-sided).

In our view, RAN2 could start by studying specification impact related to the case of “Decision by the network”, considering Option (a): Network-initiated, and Option (b) UE-initiated, requested to the network.

Proposal 3: RAN2 to study specification impact associated with the case of “Decision by the network”, for model selection, (de)activation, switching, and fallback at least for UE-sided models and two-sided models.

# 3. Conclusion

In this contribution, we discussed model management aspects considering latest related RAN1 agreements. The following are the observations and proposals in this document:

Observation 1: RAN2 work on management of data and AI/ML models to be focused on data collection and model transfer, update, monitoring, selection, (de)activation, switching and fallback.

Observation 2: The network or UE may decide to perform model selection, (de)activation, switching, and fallback at the UE, for UE-sided and two-sided models.

Proposal 1: The network and UE may use the model ID in model LCM procedures and exchange of information associated with model operation and functionality.

Proposal 2: The network and UE may exchange information on models available in the network and UE, including model ID, use case and model operation (i.e. UE-sided, Network-sided, or two-sided).

Proposal 3: RAN2 to study specification impact associated with the case of “Decision by the network”, for model selection, (de)activation, switching, and fallback at least for UE-sided models and two-sided models.

# 4. Reference

RP-221348 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

RAN2#119bis-e Chairman report

RAN1#110bis-e Chairman report.

R1-2210375	Summary#1 of General Aspects of AI/ML Framework