3GPP TSG-RAN WG2 Meeting#120		       					       R2-2211994

Toulouse, France, November 14th – 18th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on Time Synchronization Status notification towards UE(s)

Agenda item:		6.5.2

Document for: 	Discussion and Decision

- Introduction

In the LS [1], SA2 provides four solutions for time synchronization status notification towards UE(s).

In this paper, we will provide our observations from RAN2 perspectives and give our proposals. Based on the proposals, a reply LS is also provided.

- Discussion

Based on the current specification, gNB obtains 5GS clock (e.g. the reference time information) from GPS or UPF etc, and sends the reference time information to UE via ReferenceTimeinfo (RTI) over SIB without encryption or by RRC dedicated signaling with encryption.

Since PDC (propagation delay compensation) can only be performed for UE in RRC_CONNECTED state, and the reference time information in SIB9 received by UE in RRC_IDLE or RRC_INACTIVE state is not accurate enough for TSN service (e.g. the accuracy may be worse than 1us) and the reference time provision requirement of end stations, only UE in RRC_CONNECTED sate can provide accurate reference time information for DS-TT and/or end stations.

Observation 1: Based on the current RAN2 specification, only UE in RRC_CONNECTED state can perform PDC for accurate time maintenance and provide accurate reference time information for DS-TT and/or end stations.

SA2 has agreed that 5GS shall support informing UE and AF about the 5GS clock quality, including informing the UE and AF about 5GS clock quality degradation/improvement at different levels of degradation and improvement. And in TR 23.700-25 Annex A, the following 4 solutions are provided for time synchronization status notification towards UE(s):

Alternative 1: gNB provides a reference report ID within SIB

In this solution, gNB notifies UE(s) the RAN Time Synchronization Status report by a reference report ID in SIB. Based on the reference report ID, UE can aware the RAN Time Synchronization Status change and (a) re-acquire or (b) automatically determine the RAN Time Synchronization Status.

But based on the observation 1, only UE in RRC_CONNECTED state can provide accurate reference time information for DS-TT and/or end stations. The RAN Time Synchronization Status can be indicated to UE directly with RTI. E.g. if encryption is necessary, RAN Time Synchronization Status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN Time Synchronization Status can be sent to UE by SIB. It is not necessary to provide a reference report ID in SIB and then provide RAN Time Synchronization Status related to the reference report ID in RRC dedicated signaling.

Observation 2: If encryption is necessary, RAN Time Synchronization Status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN Time Synchronization Status can be sent to UE by SIB. It is not necessary to provide a reference report ID in SIB and then provide RAN Time Synchronization Status related to the reference report ID in RRC dedicated signaling.

Alternative 2: gNB provide a flag and a timestamp in SIB9 message so that UEs in RRC_IDLE and RRC_INACTIVE state get information/trigger about a new time synchronization status report available

This alternative covers a case when a time synchronization status needs to be reported to UEs in RRC_IDLE and RRC_INACTIVE states where applications need such information. Upon the UE receives the flag and determines there is a new time synchronization status report available, the UE initiates the relevant legacy procedure to enter the RRC_CONNECTED state to receive the RAN timing synchronization status via dedicated RRC signaling.

But based on the observation 1, applications will not be served by UEs in RRC_IDLE and RRC_INACTIVE states. Similar comments as for Alt1, if encryption is necessary, RAN timing synchronization status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN timing synchronization status can be sent to UE by SIB. It is not necessary to provide a flag and a timestamp in SIB and then provide RAN timing synchronization status in RRC dedicated signaling.

Observation 3: If encryption is necessary, RAN timing synchronization status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN timing synchronization status can be sent to UE by SIB. It is not necessary to provide a flag and a timestamp in SIB and then provide RAN timing synchronization status in RRC dedicated signaling.

Alternative 3: Broadcast the time change status in SIB

The idea of this alternative is to provide RAN Time Synchronization Status in RTI. E.g. The ReferenceTimeinfo (RTI) is extended with:

- Time quality (e.g. quality, clock class)

- Latest time status change time

- Time source

According to the status information, the UE can know whether the time status has been changed.

Considering that RTI only takes effect for UE in RRC-CONNECTED state, the RTI including RAN Time Synchronization Status can be sent to UE by SIB without encryption or by RRC dedicated signaling with encryption.

Observation 4: It is feasible to include RAN Time Synchronization Status in RTI, and sent RTI including RAN Time Synchronization Status to UE by SIB without encryption or by RRC dedicated signaling with encryption.

Alternative 4: Ciphered RAN Time Synchronization Status in SIB

For this solution, the RAN Time Synchronization Status is sent in SIB, and it is ciphered by NAS.

Based on the current RAN2 specification, the reference time information can be sent in SIB or RRC dedicated signaling. If the reference time information is sent in RRC dedicated signaling, it can be ciphered by AS. If the reference time information is sent by SIB, it will not be ciphered.

Considering that RAN Time Synchronization Status is used for reference time information evaluation, and based on observation 1, the reference time information is only used for UE in RRC_CONNECTED state, the RAN Time Synchronization Status is only used for UE in RRC_CONNECTED state. If the RAN Time Synchronization Status need be ciphered, it can also be sent in RRC dedicated signaling. If the RAN Time Synchronization Status need not be ciphered, it can be sent in SIB or RRC dedicated signaling. Thus, from RAN2 perspectives, it is not necessary to cipher the RAN Time Synchronization Status in SIB.

Observation 5: If the RAN Time Synchronization Status need be ciphered, it can also be sent in RRC dedicated signaling; if the RAN Time Synchronization Status need not be ciphered, it can be sent in SIB or RRC dedicated signaling. Thus, from RAN2 perspectives, it is not necessary to cipher the RAN Time Synchronization Status in SIB.

Based on the above observations, the following proposal is given:

Proposal 1: response the SA2 LS to indicate the following information:

- Based on the current RAN2 specification, only UE in RRC_CONNECTED state can perform PDC for accurate time maintenance and provide accurate reference time information for DS-TT and/or end stations.

- Similar as legacy RTI, if the RAN Time Synchronization Status need be ciphered, it can also be sent in RRC dedicated signaling. If the RAN Time Synchronization Status need not be ciphered, it can be sent in SIB and/or RRC dedicated signaling.

- From RAN2 perspectives, it is not necessary to indicate RAN Time Synchronization Status info available in SIB and then send RAN Time Synchronization Status info in RRC dedicated signaling.

- From RAN2 perspectives, it is not necessary to consider encryption issue for RAN Time Synchronization Status in SIB.

Based on the proposal 1, we provide the related reply LS [3]

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: Based on the current RAN2 specification, only UE in RRC_CONNECTED state can perform PDC for accurate time maintenance and provide accurate reference time information for DS-TT and/or end stations.

Observation 2: If encryption is necessary, RAN Time Synchronization Status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN Time Synchronization Status can be sent to UE by SIB. It is not necessary to provide a reference report ID in SIB and then provide RAN Time Synchronization Status related to the reference report ID in RRC dedicated signaling.

Observation 3: If encryption is necessary, RAN timing synchronization status can be sent to UE by RRC dedicated signaling; if encryption is not necessary, RAN timing synchronization status can be sent to UE by SIB. It is not necessary to provide a flag and a timestamp in SIB and then provide RAN timing synchronization status in RRC dedicated signaling.

Observation 4: It is feasible to include RAN Time Synchronization Status in RTI, and sent RTI including RAN Time Synchronization Status to UE by SIB without encryption or by RRC dedicated signaling with encryption.

Observation 5: If the RAN Time Synchronization Status need be ciphered, it can also be sent in RRC dedicated signaling; if the RAN Time Synchronization Status need not be ciphered, it can be sent in SIB or RRC dedicated signaling. Thus, from RAN2 perspectives, it is not necessary to cipher the RAN Time Synchronization Status in SIB.

Proposal 1: response the SA2 LS to indicate the following information:

- Based on the current RAN2 specification, only UE in RRC_CONNECTED state can perform PDC for accurate time maintenance and provide accurate reference time information for DS-TT and/or end stations.

- Similar as legacy RTI, if the RAN Time Synchronization Status need be ciphered, it can also be sent in RRC dedicated signaling. If the RAN Time Synchronization Status need not be ciphered, it can be sent in SIB and/or RRC dedicated signaling.

- From RAN2 perspectives, it is not necessary to indicate RAN Time Synchronization Status info available in SIB and then send RAN Time Synchronization Status info in RRC dedicated signaling.

- From RAN2 perspectives, it is not necessary to consider encryption issue for RAN Time Synchronization Status in SIB.

- References

- R2-2211134, LS on Time Synchronization Status notification towards UE(s) (S2-2209876; contact: Nokia)	SA2, November 2022

- TR 23.700-25, V1.1.0 (2022-10), Study on timing resiliency and TSC and URLLC enhancements

- R2-2211997, Reply LS on Time Synchronization Status notification towards UE(s), November 2022