3GPP TSG-RAN WG2 Meeting #120	R2-2212006

Toulouse, France, November 14-18, 2022

Agenda Item:	6.8

Source:	CATT

Title:	Discussion on the LS from SA2 and CT1 and slice based RACH in RRC state

Document for:	Discussion and Decision

1	Introduction

In the previous RAN2 meeting, the LS [1] [2] from SA2 and CT1 have been received by RAN2. According to the LS and the related CR, some progress on slice list and priority information for cell reselection and random access has been made in SA2 and CT1. In this paper, we will analyse the impact on RAN2 based on the received LS and how to capture applicability of slice-based RACH in RRC states.

2 Discussion

2.1 Discussion on the LS from SA2 and CT1

According to the LS [2] from CT1, the NAS layer provides the received NSAG information from the AMF transparently to AS layer. This means that the NSAG Information not used for slice based cell reselection will also be send the AS layer. In order to distinguish the NSAG(s) used for slice based cell reselection, the S-NSSAIs in Allowed NSSAI or Requested NSSAI are provided to the AS layer for slice-based cell reselection.

CT1 confirms that the below RAN2 assumption is valid: RAN2 also assumes that the NAS layer in the UE is able to provide slice group priorities to AS layer in the UE.. Besides, CT1 has agreed that the NAS layer provides the received NSAG information from the AMF transparently to the AS layer. Additionally, the allowed NSSAI and requested NSSAI are provided to the AS layer for slice-based cell reselection.

Besides, in the latest specification of TS23.501, SA2 have updated the related content based on CT1’s LS as follow:

The UE NAS provides to the UE AS the NSAG Information as received from the AMF and the S-NSSAIs in the Allowed NSSAI as input to cell reselection, except when the UE intends to register with a new set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAG Information as received from the AMF and the S-NSSAIs in the Requested NSSAI, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.

In SA2’s spec, the UE NAS provides the NSAG Information, the S-NSSAIs in the Allowed NSSAI and S-NSSAIs in the Requested NSSAI to UE AS layer to cell reselection. Obviously, this is aligned with CT1’s LS. The AS layer will filter the NSAG Information used for slice based cell reselection based on the received NSAG Information and Allowed NSSAI and Request NSSAI from NAS. Considering that this approach is workable for RAN2 and the specs in SA2 and CT1 have been updated based on the above approach, we think RAN2 should align with the agreements of SA2 and CT1 and updated the RAN2’s spec.

Proposal 1: The AS layer should filter the NSAG Information used for slice based cell reselection based on the received NSAG Information and the S-NSSAIs in Allowed NSSAI or Request NSSAI from NAS.

Proposal 2: RAN 2 should align with the specs of SA2 and CT1 and adopt the CR for TS 38.304 in R2-221007.

2.2 How to capture the slice based RACH in RRC state

In the previous RAN2 meeting, the slice based RACH in RRC_CONNECTED state is down prioritized. In our understanding, in case of RRC re-establishment and DL or UL data arrival when UL synchronisation status is "non-synchronised", it is beneficial to use slice based random access to access the network quickly. For example, when the DL or UL URLLC data arrival but the UL synchronisation status is "non-synchronised", UE supporting slice based random access can use the URLLC specific resource to recovery the UL synchronisation quickly. This is important for time sensitive slicing service. In current specs, there is no limitation to restrict the slice based random access can only be used in RRC_IDLE to RRC_CONNECTED. So we think we can support slice based RACH in RRC_CONNECTED state without spec impact.

Proposal 3: Support the slice based random access in RRC_CONNECTED state and there is no spec impact.

3 Conclusion

According to the above discussion, the following proposals are made:

Proposal 1: The AS layer should filter the NSAG Information used for slice based cell reselection based on the received NSAG Information and the S-NSSAIs in Allowed NSSAI or Request NSSAI from NAS.

Proposal 2: RAN 2 should align with the specs of SA2 and CT1 and adopt the CR for TS 38.304 in R2-221007.

Proposal 3: Support the slice based random access in RRC_CONNECTED state and there is no spec impact.

4 References

- R2-2209358  LS Out on LS on slice list and priority information for cell reselection and Random Access.

- R2-2206909  Reply LS on Slice list and priority information for cell reselection