3GPP TSG RAN WG2 #120	R2-2212013

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.6.3.2

Source:	ZTE Corporation, Sanechips

Title:	Discussion on mobility enhancements for eMTC NTN

Document for:	Discussion and Decision

Introduction

In the new WID of IoT NTN enhancements RP-221806 [1], the following objectives for mobility enhancements are listed:

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

- Define UE RRM core requirements for the above mobility enhancement features [RAN4].

In RAN2#119e meeting, RAN2 had some initial discussion on the CHO enhancements for eMTC NTN and reached the agreement as below:

Agreements: - CHO enhancements for eMTC NTN (i.e., time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution.

In RAN2#119bise meeting, RAN2 had some further discussion and reached the following agreement:

Agreements: - For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events.

In this paper, we will have further discussion on the mobility enhancements for eMTC NTN.

Discussion

Details for time-based and location-based CHO triggering events

As mentioned above, RAN2 has agreed to introduce time/timer based solution and also location based CHO triggering events based on the R17 NR NTN solution for eMTC NTN. Details are FFS.

We copy the related agreements from R17 NR NTN as below:

| Meeting info | Original agreements for R17 NR NTN |
| --- | --- |
| RAN2#114e | Agreements via email (from offline 104): 1. Support CHO location trigger as the distance between UE and a reference location which may be configured as the serving cell reference location or the candidate target cell reference location. FFS if combination can be allowed. 2. The reference location for the event description is defined as cell center. Agreements online: 1. For CHO, joint configuration of location and RSRP as well as time and RSRP triggers are supported. 2. For idle mode reselection, based on configuration NTN UE can prioritise TN over NTN. Configuration details FFS Agreements via email (from offline 104 - second round): 1. CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell. 2. Same CHO trigger conditions and RRM events can be used within NTN and NTN-TN mobility provided these are supported by the UE. NTN-TN means both “from NTN to TN (hand-in)” and “from NTN to TN (hand-in) and from TN to NTN (hand-out)". FFS for enhancements. |
| RAN2#115e | Agreements via email - from offline 103: 1. The following event is supported: condEvent L4: Distance between UE and the PCell’s reference location becomes larger than absolute threshold1 AND the distance between UE and the Conditional reconfiguration candidate becomes shorter than absolute threshold2. FFS other options 2.Specify hysteresis and time to trigger for the location event for RRM and CHO 3.Timing information from RRCReconfiguration message in RRC running CR is removed 4.UE is allowed to perform HO only during T1 to T2 5.Agree to limit to A or B and continue discussion between options A and B - Option A: UTC time + duration/timer, e.g. 00:00:01 + 40s - Option B: Two UTC time to indicate the start (T1) and end time (T2) of the candidate cell, e.g. 00:00:01 + 00:00:41 Agreements via email - from offline 103 second round: 1. RAN2 adopts Option 1: UTC time + duration/timer, e.g. 00:00:01 + 40s for representing T1 and T2 for CHO time event. 2. RAN2 adopts options C: location and RRM and D: time and RRM to be configuration options for CHO |
| RAN2#117e | Agreements via email - from offline 101 - second round: 1. The ellipsoid-Point IE specified in TS 36.331, TS 37.355 (and TS 23.032) is reused for definitions of reference locations in NR NTN. 2.The following for entering and leaving conditions are agreed: Inequality D1-1 (Entering condition 1) Ml1-Hys>Thresh1 Inequality D1-2 (Entering condition 2) Ml2+Hys>Thresh2 - consider the leaving condition for this event to be satisfied when condition D1-3 or D1-4 is fulfilled; Inequality D1-3 (Leaving condition 1) Ml1+Hys<Thresh1 Agreements via email - from offline 101 - third round: 1. remove FFS from field description condExecutionCond and revise the added sentence as: "If network configures condEventD1 or condEventT1 for a candidate cell network shall configure a second triggering event condEventA3, condEventA4 or condEventA5." 2. Z = 3000 km, X = 16 bits if integer with linear granularity 3. Range for OffsetThresholdTA is 0.5ms to 15ms, with spare bits Agreements: 1. Joint time-based and location-based CHO execution triggering for the same candidate cell is not supported in Rel-17 NTN. 2. If the CHO is not executed at T2 (timer associated with this candidate CHO cell) the UE continues to operate in the source cell and evaluates other CHO execution conditions (if configured). Working assumption: 1. T2 timer is defined as an INTEGER (1..6000), where each step represents 100 ms. Its maximum value corresponds to 10 minutes (600 seconds). FFS whether the maximum value needs to be aligned to the cell stop time Inequality D1-4 (Leaving condition 2) Ml2-Hys>Thresh2 Agreements via email - from offline 108: 1. The maximum supported value for timer T2 is 10 minutes (600 seconds). 2. It is up to UE implementation how the UE evaluates the time- or location-based condition jointly with the RRM event Ax, as long as the UE has RRM measurement results within the time window [T1, T2] or when the location condition is met. 3. The maximum number of MeasIDs to be used for CHO execution triggering in NTN is not increased from 2 to 3. |

With reference to the above agreements in NR NTN, we understand the following proposals can be applicable to eMTC NTN:

General aspects:

Proposal 1: Support CHO location trigger as the distance between UE and a reference location which may be configured as the serving cell reference location or the candidate target cell reference location.

Proposal 2: The reference location for the event description is defined as cell center.

Proposal 3: For CHO, joint configuration of location and RSRP as well as time and RSRP triggers are supported.

Proposal 4: Joint time-based and location-based CHO execution triggering for the same candidate cell is not supported in Rel-18 IoT NTN.

Details of new trigger event:

Proposal 5: CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.

Proposal 6: UE is allowed to perform HO only during T1 to T2.

Proposal 7: RAN2 adopts: UTC time + duration/timer, e.g. 00:00:01 + 40s for representing T1 and T2 for CHO time event.

Proposal 8: T2 timer is defined as an INTEGER (1..6000), where each step represents 100 ms.

Proposal 9: The maximum supported value for timer T2 is 10 minutes (600 seconds).

Proposal 10: The following event is supported: condEventD1.

The following for entering and leaving conditions are agreed:

Inequality D1-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D1-2 (Entering condition 2)

Ml2+Hys<Thresh2 (Note: this change is based on the latest stage-3 text)

- consider the leaving condition for this event to be satisfied when condition D1-3 or D1-4 is fulfilled;

Inequality D1-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2-Hys>Thresh2

Necessity to support group based mobility for eMTC NTN

In R18 NR NTN, there is also discussion on supporting group based mobility. The main justification is that, in conventional measurement report triggered mobility procedure, the signaling overhead comes in two aspects: measurement report for HO decision and HO configuration. Different from TN network, in NTN network, the intra-NTN mobility could be triggered due to satellite movement where a larger number UEs will need to be switched to another cell in one time. The signalling overhead for these UEs to report measurement results and receive HO configuration would be very large. Due to huge number of IoT UEs, this issue would be more serious in IoT NTN network than that in NR NTN network.

Observation 1: Different from TN network, in NTN network, the intra-NTN mobility could be triggered due to satellite movement where a larger number UEs will need to be switched to another cell in one time. The signaling overhead for these UEs to report measurement results and receive HO configuration would be very large. Due to huge number of IoT UEs, this issue would be more serious in IoT NTN network than that in NR NTN network.

Moreover, it shall be also noted that after HO to another cell UE need to initiate RACH to target cell to establish the connections. Such a large number of UEs initiating RACH in a short time period might lead to serious collision issue.

Observation 2: A large number of UEs initiating RACH to another cell in a short time period could lead to collision in RACH procedure.

Since the normal HO procedure is still supported for eMTC NTN, we think some solutions need to be discussed to alleviate this issue.

Due to the long RTT between UE and satellites and large coverage of satellite, for UE within a certain location range, they could share similar radio conditions, which means the measurement report receive from such UEs will also be similar. Therefore it is possible for NW to do the HO decision without waiting all measurements report to be received from all UEs, which can help reduce the signaling overhead required for sending the measurement report.

Observation 3: Due to long RTT in NTN and large coverage of satellite, UE within certain location range will face similar radio condition, which makes it possible for NW to do group HO decision with any of report received from such UEs, thus help reduce the signaling overhead required for measurement report.

Another aspect to be considered is how to reduce the signaling overhead when providing UE with the HO configuration. A group manner can be considered. However, if group manner is used for handover a large number of UEs to another cell, the RACH collision issue mentioned in observation 2 may be more serious. Another alternative may be to still use dedicated signaling.

To avoid RACH collision issue, below options can be considered:

- RACH-less HO

- Dedicated RACH resource for group HO

RACH-less HO is discussed in NR NTN but not agreed. This scheme may also have RAN1 impacts. Since the conditions of RACH-less HO might not always be satisfied, the safer way to mitigate the RACH collision issue is to use CFRA resource.

Observation 4: CFRA resource can be used to reduce RACH collision issue in group HO.

If CFRA resource is used, dedicated signaling for HO configuration is more suitable which can also reuse existing HO configuration procedure. It also gives more scheduling flexibility at NW to use dedicate HO configuration, e.g., NW might only want to move some of UEs within the same group considering load balance in different cells or due to change of trajectories of UEs.

Observation 5: Dedicate HO configuration is more suitable to be used as it allows more flexibility of scheduling since NW might only want to move some of some of UEs within the same group considering also load balance in different cells or due to change of trajectories of UEs.

Based on above analysis and observations, one example on how the group HO can be performed is described as below.

Figure 1. An example of group based mobility

As shown in the above Figure 1, UEs can be divided into several groups, e.g. based on its location or some other criteria, and each UE will be assigned with a group id. The group id will be reported together with measurement results in measurement report. Upon reception of measurement report from one or a few of the UEs in this group, network could then decide to handover all the UEs in this group or some of them.

Proposal 11: Group based handover should be supported with the following:

- Divide UE into several groups and assign each UE with a group id.

- UE report the group id together with the measurement results in measurement report.

- Upon reception of measurement report from one or a few of the UEs in this group, Network can decide to handover all the UEs or some of them in this group and send HO command to each UE.

Conclusion

In this contribution, the following observations and proposals were made:

Proposal 1: Support CHO location trigger as the distance between UE and a reference location which may be configured as the serving cell reference location or the candidate target cell reference location.

Proposal 2: The reference location for the event description is defined as cell center.

Proposal 3: For CHO, joint configuration of location and RSRP as well as time and RSRP triggers are supported.

Proposal 4: Joint time-based and location-based CHO execution triggering for the same candidate cell is not supported in Rel-18 IoT NTN.

Proposal 5: CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.

Proposal 6: UE is allowed to perform HO only during T1 to T2.

Proposal 7: RAN2 adopts: UTC time + duration/timer, e.g. 00:00:01 + 40s for representing T1 and T2 for CHO time event.

Proposal 8: T2 timer is defined as an INTEGER (1..6000), where each step represents 100 ms.

Proposal 9: The maximum supported value for timer T2 is 10 minutes (600 seconds).

Proposal 10: The following event is supported: condEventD1.

The following for entering and leaving conditions are agreed:

Inequality D1-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D1-2 (Entering condition 2)

Ml2+Hys<Thresh2 (Note: this change is based on the latest stage-3 text)

- consider the leaving condition for this event to be satisfied when condition D1-3 or D1-4 is fulfilled;

Inequality D1-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2-Hys>Thresh2

Proposal 11: Group based handover should be supported with the following:

- Divide UE into several groups and assign each UE with a group id.

- UE report the group id together with the measurement results in measurement report.

- Upon reception of measurement report from one or a few of the UEs in this group, Network can decide to handover all the UEs or some of them in this group and send HO command to each UE.

References

- RP-221806 Revised WID on IoT NTN enhancements MediaTek Inc.