3GPP TSG-RAN WG2 Meeting #120	R2-2212025

Toulouse, France, November, 2022

Agenda item:		8.9.3

Source:		Lenovo

Title:		Discussion on L2 UE-to-UE relay

Document for:		Discussion and Decision

- Introduction

This document discusses aspects related to relay selection, relay reselection, fallback from relay operation to direct PC5 link and bearer multiplexing in L2 U2U relay case.

- Discussion

Relay Selection:

It was agreed in RAN2#119bis meeting that at least 1) Upper layer trigger; 2) PC5 signal strength conditions can be considered as trigger condition for relay selection. RAN2 further discuss details for trigger 2). Regarding 2), a (pre)configured threshold is provided to the remote UE. Once PC5 measurement towards target remote UE is less than the (pre)configured threshold, the remote UE will be triggered to perform relay selection. If UE releases PC5 link immediately while PC5 measurement towards target remote UE is less than the (pre)configured threshold, it will result in the interruption. Therefore, an exchange between two remote UEs could be needed. For example, when the transmitting remote UE is triggered to perform the relay selection based on the (pre)configured threshold, the transmitting remote UE transmits a message including the indication of relay selection to the peer remote UE. Furthermore, the candidate relay UE(s) can be included in the message. The peer remote UE can respond back to the transmitting remote UE indicating which Relay UE is the best/ suitable among the Relays included by the transmitting remote UE. If this mechanism is not used, RAN2 at least need to ensure that the selected relay must be the one which has suitable radio connection to both transmitter and receiver remote UEs.

Proposal 1: A remote UE performs relay selection when PC5 measurement towards the peer remote UE is less than a (pre)configured threshold.

Proposal 2: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, a PC5 RRC message including the indication of relay selection is sent to receiving (peer) remote UE.

Proposal 3a: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, the transmitting remote UE sends a potential list of candidate Relay UEs to the receiving (peer) remote UE. The list is transmitted directly on PC5 RRC.

Proposal 3b: If P3a is not acceptable, RAN2 at least needs to ensure that the selected relay must be the one which has suitable radio connection to both transmitting remote UE and the receiving (peer) remote UEs.

As usual, the UE may detect the sidelink RLF of the PC5 direct link between two remote UEs. Once the RLF on the direct PC5 link is detected, the remote UE capable of U2U relay operation should perform relay selection.

Proposal 4: RLF on the direct PC5 link between two remote UEs can also be used to trigger relay selection.

Relay reselection:

Fig.1 U2U relay operation

Proposal 7.1b (modified):       Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.  RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

‘PC5 signal strength conditions’ is agreed by RAN2 as a condition to perform relay reselection. In this way, one threshold will be configured to the remote UE e.g UE2 in Fig.1. UE2 determines the PC5 link between the relay UE and UE2 is less than the threshold associated with relay reselection, the remote UE2 indicates to the relay UE. Then, the relay UE transmits the indication such as PC5 unicast link release to the remote UE1. The remote UE1 will perform relay reselection as well.

If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first remote UE once the relay UE detects the PC5 link between the relay UE and the second remote UE is less than the threshold associated with relay reselection.

Proposal 5: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

Proposal 6: If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold associated with relay reselection.

Furthermore, ‘PC5 link release message from relay to remote’ was agreed as a condition to trigger relay reselection of a remote UE. RAN2 needs to discuss what condition will trigger the relay UE to transmit the PC5 unicast link release message. When the relay UE detects the PC5 RLF of the link between the relay UE and the second remote UE, the relay UE needs to indicate to the first remote UE to perform relay reselection e.g via PC5 unicast link release message.

Proposal 7: A U2U relay UE is triggered to transmit PC5 unicast link release message to the first remote UE if the relay UE detects the PC5 RLF on the link with the second remote UE.

Fallback to direct PC5 link

When a remote UE communicates another remote UE via a U2U relay UE, the direct PC5 link may become better. In this case, the remote UE may fall back to the direct PC5 link between two remote UEs. Once the quality of the PC5 direct link is better than the configured threshold, the remote UE can fall back to the direct PC5 link. Namely, the remote UE establishes the PC5 link to the peer remote UE and release the link between the remote UE and the relay UE.

Proposal 8: Remote UE can switch back from the U2U relay operation to direct PC5 link when Remote UE does not need U2U relay anymore.

Proposal 9: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

Bearer Multiplexing

Figure 2 Control plane protocol stack for L2 UE-to-UE Relay

It was agreed in SI stage that the adaptation layer is located in the first hop and second hop between the UE1 and UE2. We don’t need to discuss again whether to add adaption layer into each hop.

Proposal 10: SRAP layer is needed for each hop in U2U relay case.

For the first hop of L2 UE-to-UE Relay, the N:1 mapping between remote UE SL Radio Bearers and first hop PC5 RLC channels should be supported in the first hop PC5 adaptation layer. Furthermore, the adaptation layer over first PC5 hop between Source Remote UE and Relay UE supports to identify traffic destined to different destination remote UEs. It is from resource efficency point of view beneficial to support that packets associated with the different destination UEs served by the same relay UE can be multiplexed in one TB for the first hop.

Proposal 11: In U2U relaying, multiplexing of sidelink data by the transmitter remote UE towards more than one Rx remote UE served by the same relay node into a TB is supported.

Figure 3

It was agreed in SI stage that PC5 adaptation layer of the second hop supports the N:1 bearer mapping between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop and supports the remote UE identification function. If the data from the different transmitting remote UEs towards the same destination UE are received by the relay UE, it is benefitial to allow the data from the different transmitting remote UEs to be multiplexed at the relay UE.

Proposal 12: In U2U relaying, the multiplexing data from the different transmitting remote UEs towards the same destination UE at the relay UE is supported.

Addressing:

Figure 4

Figure 4 shows that Tx-Remote-UE (UE1) is the UE that has some application data to be sent to another Remote UE shown as Rx-Remote-UE (UE3) in Figure 4, via a Relay (UE2). Following addressing mechanism can be used for this purpose:

Table 1: Approach-1

|  | Interface-1 | Interface-2 |
| --- | --- | --- |
| L1L2 SRC ID | SRC of UE1 | SRC of UE2 |
| L1L2 DST ID | DST of UE2 | DST of UE3 |
| HPID | As used by UE1 | As used by UE2 |

Table 2: : Approach-2

|  | Interface-1 | Interface-2 |
| --- | --- | --- |
| L1L2 SRC ID | SRC of UE1 | SRC of UE1 |
| L1L2 DST ID | DST of UE3 | DST of UE3 |
| HPID | As used by UE1 | As used by UE1 |

Table 3: Approach-3

|  | Interface-1 | Interface-2 |
| --- | --- | --- |
| L1L2 SRC ID | SRC of UE1 | SRC of UE2 |
| L1L2 DST ID | DST of UE3 | DST of UE3 |
| HPID | As used by UE1 | As used by UE2 |

While Approach-1 may appear “normal” sidelink approach e.g., since the L2 IDs are from the “current transmitter” to “immediate receiver”, approaches 2 and 3 provide different advantages. Approaches 2 and 3 enable both the U2U relay UE and the UE3 (the receiver remote UE) to receive simultaneously, this is diversity at no extra cost i.e., no extra transmission. When this diversity is not required e.g., the UE1 to UE3 direct link is strong, then the relaying need not be done. The relaying can be switched on and off just using a flag in the SCI. In this way switching between direct and indirect links will be just a matter of toggling the flag in SCI. In both approaches the relay UE needs to “adopt” the remote UE3’s DST Id as its own but this is possible since a relaying will only start when the handshaking between the UE1 and UE2 is complete. Approach-3 has the extra advantage that HARQ process ID can be chosen by the U2U relay on its own, without needing to use the same one as in Approach 2. Approach 2 has however the soft combining advantages. Both approaches 2 and 3 work quite well in future if there will be more than one U2U relays relaying between two remote UEs.

Proposal 13: RAN2 is kindly requested to discuss the addressing mechanisms.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: A remote UE performs relay selection when PC5 measurement towards the peer remote UE is less than a (pre)configured threshold.

Proposal 2: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, a PC5 RRC message including the indication of relay selection is sent to receiving (peer) remote UE.

Proposal 3a: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, the transmitting remote UE sends a potential list of candidate Relay UEs to the receiving (peer) remote UE. The list is transmitted directly on PC5 RRC.

Proposal 3b: If P3a is not acceptable, RAN2 at least needs to ensure that the selected relay must be the one which has suitable radio connection to both transmitting remote UE and the receiving (peer) remote UEs.

Proposal 4: RLF on the direct PC5 link between two remote UEs can also be used to trigger relay selection.

Proposal 5: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

Proposal 6: If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold associated with relay reselection.

Proposal 7: A U2U relay UE is triggered to transmit PC5 unicast link release message to the first remote UE if the relay UE detects the PC5 RLF on the link with the second remote UE.

Proposal 8: Remote UE can switch back from the U2U relay operation to direct PC5 link when Remote UE does not need U2U relay anymore.

Proposal 9: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

Proposal 10: SRAP layer is needed for each hop in U2U relay case.

Proposal 11: In U2U relaying, multiplexing of sidelink data by the transmitter remote UE towards more than one Rx remote UE served by the same relay node into a TB maybe supported.

Proposal 12: In U2U relaying, the multiplexing data from the different transmitting remote UEs towards the same destination UE at the relay UE is supported.

Proposal 13: RAN2 is kindly requested to discuss the addressing mechanisms.

References

Chairman notes for RAN2#119bise meeting.