3GPP TSG-RAN WG2 Meeting #120	R2-2212026

Toulouse, France, November, 2022

Agenda item:		8.9.3

Source:		Lenovo

Title:		Service continuity in L2 U2N relay case

Document for:		Discussion and Decision

- Introduction

RAN2 achieved the following agreement for service continuity in last meeting[1].

Agreements:

Proposal 1 (modified)	For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.  FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE.

Proposal 3	For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching.

Proposal 4 (modified)	For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available).

Proposal 5	For i2d path switch scenario, re-use the existing T304 timer

Proposal 6	For d2i and i2i path switch scenarios, re-use the existing T420 timer.

In this contribution, we further discuss the issues for service continuity in L2 U2N relay case.

- Discussion

It was agreed last meeting that serving L2 U2N Relay UE becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2 can be supported for i2i path switching procedure. It is FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE. According to the discussion last meeting, there is no problem to support event Z2 if same quantity e.g SL-RSRP is used for candidate relay UE and serving relay UE. If the different SL-RSRP/SD-RSRP is used in Z2, whether a separate offset is needed, it should be decided by RAN4.

Proposal 1: The following event can be supported for i2i path switching if same quantity e.g SL-RSRP or SD-RSRP is used for candidate relay UE and serving relay UE.

- event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

Proposal 2: If the different quantity e.g SL-RSRP or SD-RSRP is used for candidate relay UE and serving relay UE in the following event, it should be decided by RAN4. (Sent LS)

- event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

RAN3 assumes that source gNB can signal the serving cell of the relay UE to target gNB during inter-gNB path switching. According to the discussion in last RAN2 meeting, whether to include a single Target Relay L2 ID or a list of Target candidate Relay L2 IDs will be decided by RAN3. If target gNB is responsible for selecting the target relay UE, it is helpful that measurement results for the candidate relay UEs can be provided to target gNB to assist to select the target relay UE. Another point to be discussed is if the target gNB may choose different relay UE than those included in the handover request message by the source gNB?

Proposal 3: Target gNB is responsible for selecting the target relay UE.

Proposal 4: Measurement results for the candidate relay UEs can be provided to target gNB to assist to select the target relay UE.

Proposal 5: RAN2 to discuss if the target gNB may choose different relay UE than those included in the handover request message by the source gNB.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: The following event can be supported for i2i path switching if same quantity e.g SL-RSRP or SD-RSRP is used for candidate relay UE and serving relay UE.

- event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

Proposal 2: If the different quantity e.g SL-RSRP or SD-RSRP is used for candidate relay UE and serving relay UE in the following event, it should be decided by RAN4. (Sent LS)

- event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE

Proposal 3: Target gNB is responsible for selecting the target relay UE.

Proposal 4: Measurement results for the candidate relay UEs can be provided to target gNB to assist to select the target relay UE.

Proposal 5: RAN2 to discuss if the target gNB may choose different relay UE than those included in the handover request message by the source gNB.

References

Chairman notes for RAN2#119bise meeting.