3GPP TSG-RAN WG2 Meeting #120	R2-2212029

Toulouse, France, November, 2022

Agenda item:		8.4.4

Source:		Lenovo

Title:			Analysis on CHO with candidate SCG

Document for:		Discussion and Decision

- Introduction

In RAN#97-e meeting, the WI of Further NR mobility enhancements was revised [1], with the following objective with regards to including target MCG and candidate SCGs for CPC/CPA in NR-DC:

To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2] - CHO including target MCG and target SCG is used as the baseline

This document aims to discuss the potential impacts for the case that CHO includes target MCG and candidate SCGs for CPC/CPA in NR-DC.

- Discussion

In rel-18 mobility, the following cases will be studied in which CHO can further embed another CPA/CPC [1].

Case 1: CHO including target MCG and candidate SCGs for CPA.

- In case1, UE is configured with DC or SA in source side. CHO configuration includes target MCG without target SCG. And the CPA is embedded in CHO configuration.

Case 2: CHO including target MCG, target SCG and candidate SCGs for CPC.

- In case2, CHO configuration includes target MCG, target SCG and the embedded candidate SCG for CPC. Regarding candidate SCG for CPC, the configuration of candidate SCG will be based on the target SCG if delta configuration is used. According to the legacy, target SCG could be same or different from source SCG.

Proposal 1: The following two cases should be supported in Rel-18.

- Case 1: CHO including target MCG and candidate SCGs for CPA.

- Case 2: CHO including target MCG, target SCG and candidate SCGs for CPC.

Proposal 2: The configuration of candidate SCGs for CPA/CPC is embedded in CHO configuration.

In Rel-16 CHO and CPC, the compliance check for an RRCReconfiguration message received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO and CPC execution (when the message is required to be applied). If the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message for CHO and CPC, the UE continues to use the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected. Then, the UE initiates re-establishment procedure or SCG failure information procedure.

In rel-18 mobility, CHO can further embed another candidate SCG for CPA/CPC. For the above three cases, according to the legacy, the compliance check for the embedded CPA can be performed upon the reception of the message or upon CHO execution (when the CHO message is required to be applied). The failure behavior defined also applies in case the UE cannot comply with the embedded SCG configuration for CPA/CPC. But UE will perform reestablishment procedure even the compliance check for CHO configuration is successful and the failure only happens on the embedded CPA/CPC. However, it seems unnecessary to perform reestablishment procedure or SCG failure information if the compliance check on CHO is successful but only the compliance check of the embedded CPA/CPC fails.

Proposal 3: UE considers the compliance check successful if the compliance check for CHO configuration is successful, but the compliance check fails for the embedded CPA/CPC.

In the case of CHO including target MCG and the encapsulated CPA/CPA, the candidate SCG for CPA/CPC is only performed after CHO is executed and the condition for CPA/CPC is met. Therefore, it seems unnecessary for UE to perform the condition evaluation for the embedded CPA/CPC upon the reception of CHO including target MCG and the encapsulated CPA/CPA. It is natural that UE evaluates the conditions for the embedded CPA/CPC after UE successfully executes CHO.

Proposal 4: In the case of CHO including target MCG and candidate SCGs for CPA/CPC UE evaluates the conditions for candidate SCGs for CPA/CPC after UE successfully executes CHO.

- Conclusion

In this contribution, the conclusion and proposal are made based on the discussion:

Proposal 1: The following two cases should be supported in Rel-18.

- Case 1: CHO including target MCG and candidate SCGs for CPA.

- Case 2: CHO including target MCG, target SCG and candidate SCGs for CPC.

Proposal 2: The configuration of candidate SCGs for CPA/CPC is embedded in CHO configuration.

Proposal 3: UE considers the compliance check successful if the compliance check for CHO configuration is successful and the failure only happens on the embedded CPA/CPC.

Proposal 4: In the case of CHO including target MCG and candidate SCGs for CPA/CPC UE evaluates the conditions for candidate SCGs for CPA/CPC after UE successfully executes CHO.

References

RP-222332 R18 MobEnh WID