3GPP TSG-RAN WG2 Meeting #120	R2-2212032

Toulouse, France, 14 – 18 November 2022

Agenda item:	 8.13.4

Source:		Lenovo

Title:		SON enhancements for SPR

Document for:		Discussion and Decision

1	Introduction

In RAN2#119bis-e meeting [1], agreements on SON enhancements for successful PSCell change report were achieved:

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

2	Given that PSCell addition is proposed by all companies, SPR is used as the abbreviations to use for the feature.

3	RAN2 confirm to prioritize NR-DC scenario for SPR.

4	SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. Details of the configuration and report need to be tailored/customized/new message per use case.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

In this paper, details of SON enhancements for successful PSCell change report are further discussed.

2	Discussion

RAN3#117bis-e meeting discussed different aspects related to SPR and sent an LS [2] to RAN2 to ask the following questions:

Q6. Whether the SPCR can be stored at the UE and sent later to the gNB or is sent immediately after the successful PSCell change or addition?

Q7. Which node (MN or SN) retrieves the SPCR from the UE?

Q8. Which node (MN or SN) sends the Successful PSCell Change/Addition configuration to the UE?

This paper would mainly discuss the FFSs left in last RAN2 meeting and the questions asked by RAN3.

Similar as T310/T312/T304 related trigger condition for SHR, for reporting successful PSCell addition/change, RAN2#119bis-e meeting agreed that configuration for generating SPR e.g. T310/T312/T304 trigger threshold associated with SCG can be configured to the UE. The values of the triggering conditions are FFS, similar as T310/T312/T304 trigger condition for SHR, a percentage value rather than an absolute value can be configured as the trigger condition of the SPR.

Proposal 1: A percentage value of T310/T312/T304 can be configured as the trigger condition of SPR.

If SPCR is triggered due to T310/T312, it may indicate that the threshold for triggering PSCell change or CPC procedure is improper or setting of RLM timer T310/T312 in SCG is inappropriate, therefore, the objective of SPCR is to optimize PSCell change related configuration and/or T310/T312 configuration. In R17 MRO for SCG failure, it is the node that initiates the PSCell change performs root cause analysis, this principle can be applied for SPR, e.g.

- For MN initiated PSCell change or CPC, it is MN performs root cause analysis for SPR e.g. analyses whether and how to optimize PSCell change configuration and/or T310/T312 configuration.

- For SN initiated PSCell change or CPC, it is SN performs root cause analysis for SPR e.g. analyses whether and how to optimize PSCell change configuration and/or T310/T312 configuration.

Based on above principle, for MN initiated PSCell change or CPC, it is suitable for the MN to generate/decide T310/T312 trigger threshold for SPR, on the other hand, since T310/T312 trigger threshold of SPR may be a percentage value rather than an absolute value, it is not needed for the MN to achieve the configured timer value of T310/T312 from source SN when MN generates/decides T310/T312 trigger threshold for SPR. Similarly, for SN initiated PSCell change or CPC, it is suitable for the source SN to generate/decide T310/T312 trigger threshold for SPR.

Proposal 2: The node which initiates PSCell change or CPC decides T310/T312 trigger threshold for SPR, i.e.

- For MN initiated PSCell change or CPC, MN decides T310/T312 trigger threshold for SPR;

- For SN initiated PSCell change or CPC, source SN decides T310/T312 trigger threshold for SPR.

For PSCell addition or CPA as well as MN or SN initiated PSCell change or CPC, if SPR is triggered due to T304, it means that T304 configuration is not set properly or RACH configuration towards the target PSCell is not suitable, therefore, RACH configuration and/or T304 configuration need to be optimized by the target SN. Considering it is target SN performs MRO analysis and optimization, target SN is the suitable node to generate/decide T304 trigger threshold for SPR.

Proposal 3: Target SN decides T3104 trigger threshold for SPR.

After the corresponding node decides T310/T312/T304 trigger threshold for SPR, configuration for generating SPR e.g. T310/T312/T304 trigger threshold associated with SCG can be sent to the UE by the MN.

Proposal 4: Configuration of T310/T312/T304 trigger threshold for SPR can be sent to the UE by the MN.

When at least one configured trigger condition for SPR is fulfilled, SPR is stored by the UE including some necessary information for analysis of the near-failure PSCell addition/change, e.g. source PSCell information in case of PSCell change/CPC, target PSCell information, SPCR cause and time elapsed between the CPAC execution and reception of CPAC configuration in case of CPAC. It is better to define a new IE and a new variable for logging and storing SPR in the UE to distinguish and decouple it from SHR, especially for the case of successful PCell change with successful PSCell change, i.e when the UE needs to store both SHR and SPR.

Proposal 5: SPR is logged in a new IE and stored in a new UE variable.

Besides source PSCell information, target PSCell information, neighbour cell information, SPCR cause and time elapsed between the CPAC execution and reception of CPAC configuration, location information can also be reported. And similar as SHR, random access related information can be reported when SPR is triggered due to T304. For the cell which is a CPAC candidate PSCell, it is better to define a new flag to indicate CPAC candidate PSCell which is clearer compared with reusing CHO candidate cell flag.

Proposal 6: The following information can be included in the SPR:

- location information

- random access related information when SPR is triggered due to T304

- a new flag to indicate CPAC candidate PSCell

After the PSCell addition or CPA as well as PSCell change or CPC procedure is successfully completed, simailar as SHR, it is not urgent to sent SPR immediately, i.e. the stored SPR can be sent to the network later. Since MN is the master node and the UE still has connection with it, an indication for availability of SPR may be transmitted to MN by the UE, based on the received indication, MN may request the SPR from the UE via UE Information Request message, and then the UE may transmit the stored SPR to MN via UE Information Response message.

Proposal 7: The UE sends an indication for availability of SPR to MN, MN may request and receive the SPR from the UE via legacy UE Information Request/Response procedure.

3	Conclusion

In this contribution, the details of SON enhancements for SPR are discussed. We have the following proposals:

Proposal 1: A percentage value of T310/T312/T304 can be configured as the trigger condition of SPR.

Proposal 2: The node which initiates PSCell change or CPC decides T310/T312 trigger threshold for SPR, i.e.

- For MN initiated PSCell change or CPC, MN decides T310/T312 trigger threshold for SPR;

- For SN initiated PSCell change or CPC, source SN decides T310/T312 trigger threshold for SPR.

Proposal 3: Target SN decides T3104 trigger threshold for SPR.

Proposal 4: Configuration of T310/T312/T304 trigger threshold for SPR can be sent to the UE by the MN.

Proposal 5: SPR is logged in a new IE and stored in a new UE variable.

Proposal 6: The following information can be included in the SPR:

-	location information

-	random access related information when SPR is triggered due to T304

-	a new flag to indicate CPAC candidate PSCell

Proposal 7: The UE sends an indication for availability of SPR to MN, MN may request and receive the SPR from the UE via legacy UE Information Request/Response procedure.

Reference

[1] Chairman notes for RAN2#119bis-e meeting

[2] R3-226003, LS on inter-RAT SHR and SPCR