3GPP TSG-RAN WG2 Meeting #120	R2-2212033

Toulouse, France, 14 – 18 November 2022

Agenda item:	 8.13.4

Source:		Lenovo

Title:		Successful Handover Report for inter-RAT HO

Document for:		Discussion and Decision

1	Introduction

In R18 [1], SON enhancements for inter-RAT Successful Handover Report would be supported:

Support of SON/MDT enhancements for [RAN3, RAN2]:

•	MR-DC CPAC

•	Successful PScell change report

•	Successful Handover Report (e.g. inter-RAT)

•	NPN

•	RACH report

•	fast MCG recovery

•	NR-U (MRO and UL MLB)

In this contribution, we would discuss details of inter-RAT SHR.

2	Discussion

In previous RAN2#113bis-e meeting [2], the scenarios for SHR were discussed, intra-NR HO and inter-RAT HO from NR to LTE were agreed. Unfortunately, only SHR for intra-NR HO is specifed considering limited time allocation in R17. As list in R18 SON/MDT scope, the SHR for inter-RAT HO should be further discussed in R18. Compared with inter-system inter-RAT HO, intra-system inter-RAT HO has less specification impacts. It is prioritized to discuss SHR for intra-system inter-RAT HO from NR to LTE.

Observation 1: The scenario of inter-RAT HO from NR to LTE was agreed for SHR in RAN2#113bis-e meeting.

In RAN3#117-e meeting [3], it was agreed that SHR for intra-system inter-RAT HO from NR to LTE would be treated first, this is totally aligned with RAN2’s previous agreements. Therefore, in R18 we prioritize SHR for intra-system inter-RAT HO from NR to LTE.

Proposal 1: Prioritize SHR for intra-system inter-RAT HO from NR to LTE in R18.

RAN3#117bis-e meeting discussed inter-RAT SHR from NR to LTE and sent an LS [4] to RAN2 including following information:

RAN3 agreed to support T310 and T312 related triggers for inter-RAT SHR from NR to LTE. But there was no consensus on whether to also support T304 trigger for inter-RAT SHR from NR to LTE. Considering this might impact RAN2 specifications (including LTE specifications), RAN3 has the following questions:

Q1. Is RAN2 planning to impact LTE specifications to support inter-RAT SHR?

Q2. Whether T304 trigger for inter-RAT SHR from NR to LTE is to be supported?

Q3. If yes to Q2, whether the inter-RAT SHR is always encoded in source RAT format or can be encoded based on the RAT format which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR encoded in NR format for T310/T312 triggers and in LTE format for T304 triggers for inter-RAT HO from NR to LTE)?

Further RAN3 is not sure on the retrieval and reporting of inter-RAT SHR and would like to ask RAN2 the following question:

Q4. If yes to Q2, and if inter-RAT SHR is collected due to T304 triggers (configured by target LTE node), what is RAN2’s preference on the following two options?

- Option 1: It is sufficient for UE to report the inter-RAT SHR once UE is back to NR

- Option 2: The LTE node should have the capability to retrieve the inter-RAT SHR

RAN3 further discussed on the potential contents for inter-RAT successful handover and agreed on the following:

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

Considering there might be parallel discussion in RAN2, RAN3 would like to check the following:

Q5: Can RAN2 confirm the support for above parameters for inter-RAT SHR from NR to LTE? Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

Based on the LS [4], we would further discuss the Questions asked by RAN3.

In R17, intra-NR Successful Handover Report (i.e. SHR) is introduced to provide a more robust mobility via reporting failure events observed during successful handovers. For intra-NR SHR, the UE is configured with triggering conditions at least including thresholdPercentageT310, thresholdPercentageT312 and thresholdPercentageT304 to compile the SHR in the handover command. The SHR would be triggered when at least one condition is met.

Principle of intra-NR SHR should be taken as a baseline when cnsidering SHR for intra-system inter-RAT HO from NR to EUTRAN, i.e. at least including T310 related trigger condition andT312 related trigger condition which were agreed in RAN3#117bis-e meeting, besides trigger condition related with RLM timer, T304 related trigger condition which is configured by target RAT should also be considered e.g. in order to optimize RACH configuration towards target cell. Even introducing T304 related trigger condition would impact LTE specifications, we don’t think the impacts are big and it would not introduce complexity. Additionally, supporting T310/T312 trigger condition for inter-RAT SHR may also impact LTE specification, for example, the inter-SHR may be transmitted to the target LTE node. Therefore, besides T310 and T312, T304 trigger condition for inter-RAT SHR from NR to LTE can also be supported.

Proposal 2: Besides T310 and T312, T304 trigger condition for inter-RAT SHR from NR to LTE can also be supported.

After source NR node decides/generates T310/T312 trigger condition or target LTE node decides/generates T304 trigger condition, the configuration of triggering inter-RAT SHR from NR to LTE may be transmitted to the UE via the MobilityFromNRCommand message.

Proposal 3: Configuration of triggering inter-RAT SHR from NR to LTE may be transmitted to the UE via the MobilityFromNRCommand message.

Based on the configurations of triggering  inter-RAT SHR, the UE may store successful inter-RAT HO related information when at least one trigger condition for intra-system inter-RAT SHR is fulfilled, simailar as contents in SHR for intra-NR HO, the source cell ID, target cell ID, cause of SHR, and measurement results can be stored in the intra-system inter-RAT SHR. So, RAN2 can confirm the parameters for inter-RAT SHR from NR to LTE in the LS [2] from RAN3. Besides these items, for intra-NR SHR, target C-RNTI is also included to identify SHR and subsequent RLF report are associated with a same handover procedure, the same issue exists for inter-RAT HO scenario, so it is beneficial to include target C-RNTI in the inter-RAT SHR from NR to LTE. After intra-system inter-RAT HO is successfully completed, the UE may transmit the stored inter-RAT SHR to network.

Proposal 4: The following information can be included in the inter-RAT SHR from NR to LTE:

- source NR cell information

- target LTE cell information

- measurement results of source NR cell, target LTE cell and neighbour cell(s)

- cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

- target C-RNTI

Different from intra-NR SHR, since RAT of source cell and target cell are totally different, the RAT format of the inter-RAT SHR when one or more trigger conditions are fulfilled needs to be considered.  In the LS from RAN3 [2], two options are provided:

- Option 1: inter-RAT SHR is encoded in source RAT format;

- Option 2: inter-RAT SHR can be encoded based on the RAT format which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR encoded in NR format for T310/T312 triggers and in LTE format for T304 triggers for inter-RAT HO from NR to LTE).

For MRO purpose, the node which generates the trigger condition that acutually triggers the inter-RAT SHR should be the one to anzlyze the report for inter-RAT SHR, e.g. inter-RAT SHR should be analyzed by the source RAT if the inter-RAT SHR is triggered due to T310 or T312 trigger threshold is fulfilled, and target RAT is the suitable node to perform root cause analysis if the inter-RAT SHR is triggered due to T304 trigger condition being fulfilled. So, the node which receives inter-RAT SHR from the UE may forward it to corresponding node that generates the trigger condition that triggers the inter-RAT SHR for MRO analysis.

Observation 2: The node which receives inter-RAT SHR from the UE may forward it to corresponding node that generates the trigger condition that triggers the inter-RAT SHR for MRO analysis.

Based on this principle, to enable the source RAT decode and understand the inter-RAT SHR which is triggered due to T310 or T312 trigger threshold being fulfilled, the UE can encode the inter-RAT SHR in NR format, similarly, to enable the target RAT decode and understand the inter-RAT SHR which is triggered due to T304 trigger threshold being fulfilled, the UE can encode the inter-RAT SHR in LTE format. Therefore, inter-RAT SHR can be encoded based on the RAT format of the node which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR can be encoded in NR format if T310/T312 triggers inter-RAT SHR from NR to LTE, or in LTE format if T304 triggers inter-RAT SHR from NR to LTE).

Proposal 5: Inter-RAT SHR can be encoded based on the RAT format of the node which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR can be encoded in NR format if T310/T312 triggers inter-RAT SHR from NR to LTE, or in LTE format if T304 triggers inter-RAT SHR from NR to LTE).

When inter-RAT SHR is triggered due to T310 or T312 trigger threshold being fulfilled, the UE can encode the inter-RAT SHR in NR format. There are two ways for inter-RAT SHR reporting:

- Way 1: UE reports the inter-RAT SHR in NR format to target LTE node or a second LTE node, target LTE node or the second LTE node would not decode the inter-RAT SHR in NR format, but it would forward the inter-RAT SHR in NR format to source NR node for MRO analysis. In this way, to enable the receiving node (i.e. target LTE node or the second LTE node) know which node should perform MRO analysis for the inter-RAT SHR or the received inter-RAT SHR should be transferred to which node for MRO analysis, the UE may also report source cell ID outside of the inter-RAT SHR.

- Way 2: once UE is back to NR, UE reports the inter-RAT SHR in NR format to the NR node, the NR node would forward the inter-RAT SHR in NR format to source NR node for MRO analysis e.g. based on the inter-RAT SHR cause and source cell ID.

Both Way 1 and Way 2 are feasible, they should be supported.

Proposal 6: When T310/T312 triggers inter-RAT SHR from NR to LTE, inter-RAT SHR encoded in NR format can be transmitted to a NR node, or, to the target LTE node or a second LTE node together with source cell ID outside of the inter-RAT SHR.

When inter-RAT SHR is triggered due to T304 trigger threshold being fulfilled, the UE can encode the inter-RAT SHR in LTE format. There are two ways for inter-RAT SHR reporting:

- Way i: UE reports the inter-RAT SHR in LTE format to target LTE node, target LTE node would decode and underatnd the inter-RAT SHR for MRO.

- Way ii: UE reports the inter-RAT SHR in LTE format to a second LTE node, the second LTE node would would forward the inter-RAT SHR in LTE format to target LTE node for MRO analysis e.g. based on the inter-RAT SHR cause and target cell ID.

- Way iii: once UE is back to NR, UE reports the inter-RAT SHR in LTE format to the NR node, if the NR node can decode the inter-RAT SHR in LTE format, the NR node would forward the inter-RAT SHR in LTE format to target LTE node for MRO analysis e.g. based on the inter-RAT SHR cause and target cell ID.

- Way iv: once UE is back to NR, UE reports the inter-RAT SHR in LTE format to the NR node, if the NR node can’t decode the inter-RAT SHR in LTE format, it would forward the inter-RAT SHR in LTE format to target LTE node for MRO analysis. In this way, to enable the receiving node (i.e. the NR node) know which node should perform MRO analysis for the inter-RAT SHR or the received inter-RAT SHR should be transferred to which node for MRO analysis, the UE may also report target cell ID outside of the inter-RAT SHR.

Above Way i/ii/iii/iv are feasible, RAN2 should discuss them.

Proposal 7: When T304 triggers inter-RAT SHR from NR to LTE, inter-RAT SHR encoded in LTE format can be transmitted to the target LTE node or a second LTE node, or, to a NR node optionally with target cell ID outside of the inter-RAT SHR.

3	Conclusion

In this contribution, the details of SHR for intra-system inter-RAT HO are discussed. We have the following observation and proposals:

Observation 1: The scenario of inter-RAT HO from NR to LTE was agreed for SHR in RAN2#113bis-e meeting.

Observation 2: The node which receives inter-RAT SHR from the UE may forward it to corresponding node that generates the trigger condition that triggers the inter-RAT SHR for MRO analysis.

Proposal 1: Prioritize SHR for intra-system inter-RAT HO from NR to LTE in R18.

Proposal 2: Besides T310 and T312, T304 trigger condition for inter-RAT SHR from NR to LTE can also be supported.

Proposal 3: Configuration of triggering inter-RAT SHR from NR to LTE may be transmitted to the UE via the MobilityFromNRCommand message.

Proposal 4: The following information can be included in the inter-RAT SHR from NR to LTE:

- source NR cell information

- target LTE cell information

- measurement results of source NR cell, target LTE cell and neighbour cell(s)

- cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

- target C-RNTI

Proposal 5: Inter-RAT SHR can be encoded based on the RAT format of the node which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR can be encoded in NR format if T310/T312 triggers inter-RAT SHR from NR to LTE, or in LTE format if T304 triggers inter-RAT SHR from NR to LTE).

Proposal 6: When T310/T312 triggers inter-RAT SHR from NR to LTE, inter-RAT SHR encoded in NR format can be transmitted to a NR node, or, to the target LTE node or a second LTE node together with source cell ID outside of the inter-RAT SHR.

Proposal 7: When T304 triggers inter-RAT SHR from NR to LTE, inter-RAT SHR encoded in LTE format can be transmitted to the target LTE node or a second LTE node, or, to a NR node optionally with target cell ID outside of the inter-RAT SHR.

Reference

[1] RP-221825, Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC

[2] Chairman notes of RAN2#113bis-e meeting

[3] Chairman notes for RAN3#117-e meeting

[4] R3-226003, LS on inter-RAT SHR and SPCR