3GPP TSG-RAN WG2 Meeting #120	R2-2212040

Toulouse, France, 14th - 18th Nov 2022

Agenda item:		8.5.3.1

Source:		Lenovo

Title:		Discussion of DRX enhancement

Document for:		Discussion and Decision

- Introduction

- RAN2#119bis has discussed the XR-power saving in RAN and reached the following agreement:

- 1: At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

RAN1#110bis has discussed the DRX enhancement for XR-power saving in UE and reached the following agreement:

Agreement - For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1) - Prioritize semi-static solutions - FFS: Whether dynamic solutions will be also needed

In this contribution we further discuss the power saving of DRX enhancement related to the traffic pattern.

- Discussion

The challenge for these power saving solutions is to achieve a good tradeoff between the power saved at the UE and the additional traffic delay due to not monitoring the PDCCH. In the following, we analyse the issues about the legacy C-DRX for the XR traffic transmion.

- Periodicity Alignment between CDRX and XR Traffic

Figure 1. An example of the mismatch between DRX cycle and non-integer periodicity of XR traffic

One issue of C-DRX operation for XR services is the misalignment between C-DRX cycle and periodicity of XR traffic. XR service typically has a fixed frame refresh rate resulting in non-integer frame periodicity, such as 16.67 ms. However, configuration of C-DRX, e.g. periodicity, ON duration timer, or inactivity timer, is fixed and integer value based on UE-specific RRC signalling. As depicted in figure 1, the mismatch between the DRX cycle length with 16ms and the non-integer video traffic periodicity with 16.67ms can result in either additional power consumption to keep awake in longer time for UE or delay traffic transmission to next DRX on-duration. There may be following candidates to resolve the issue:

- Option 1. Dynamically adjust C-DRX parameters e.g., start offset or periodicity length for the next DRX cycle

For example, NW send a delayed or advanced value against the legacy start offset in Nth DRX period, UE adjust the start offset of the N+1th DRX period. This can flexibly adapt the C-DRX pattern to traffic transmission. In addition to this case, it can be applicable to other cases, e.g., XR application changes the frame rate. Although it has some dynamic signalling overhead, it is flexible and has no impact on the DRX formula.

- Option 2. Several inner DRX cycles within an outer DRX cycle

For example, assume XR frame rate with 60 FPS, configuring an outer DRX cycle length with 50ms, an DRX start offset and 3 inner cycles {17ms,17ms, 16ms} within the outer DRX cycle. UE needs to determines the start occastion of the DRX on-duration based on the sequential inner cycle value within each outer DRX cycle during the DRX procedure for one serving cell. It has no impact on the DRX formula.

- Option 3. New DRX formula similar as R16 CG occasion determination formula with non-integer periodicity

For example, UE determines sequentially the subsequent DRX on-duration occasion according to the shift of the rounded-down value of the non-integer periodicity after determining the first DRX on-duration occasion upon reception of DRX configuration.

UE receive the DRX configuration including the non-integer value of the DRX cycle length configuration: - the MAC entity shall consider sequentially that the Nth (N >= 0) DRX_OnDuration start position occurs in the subframe for which: - [(SFN × 10) + subframe number] = floor (DRX- Start-offset + N × Non-integer DRX cycle) modulo (1024 × 10) - start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

Compared to option 1 and 2, UE selects different DRX formula used for determination drx-onduration occasions with the non-integer DRX cycle and the integer DRX cycle during the DRX procedure. This can avoid the SFN wrap round issue.

- Option 4. Multiple C-DRX configuration in one serving cell similar as R16 multiple CG/SPS

Assume XR frame rate with 60 FPS, UE is awake on all the DRX on-duration occasion determined based on the start offset values 0/17/34ms and cycle lenth value 50ms. This is also applicable for the multiple traffic with different periodicity if multiple C-DRX configuration with separate both start offset and cycle length. Considering the additional C-DRX configuration may be introduced in case of multiple flow with different period, it is preferred to not use multiple C-DRX configuration for this issue to avoid increasing UE complexity for maintain too many sets of C-DRX timers in one serving cell. This should be first excluded.

Proposal 1: RAN2 to discuss the following options to avoid mismatch between DRX cycle with the XR traffic non-integer periodicity:

- Option 1. Dynamically adjust C-DRX parameters e.g., start offset or cycle length for the next DRX cycle

- Option 2. Several inner DRX cycle within an outer DRX cycle

- Option 3. New DRX formula based on the rounding value of the DRX non-integer periodicity

- Multiple DRX configurations for multiple flows with different period

XR services, such as AR, VR, and Cloud Gaming services, over 5G are important vertical applications for 5G commercial deployments. Many of the end user XR and CG devices are expected to be mobile and of small-scale, thus having limited battery power resources. The DRX mechanism is a legacy power saving mechanism to achieve power saving gain for UE in RRC_CONNECTED state. However, some characteristics of XR traffic are not addressed by the legacy DRX mechanism and could result in limited UE power savings.

Current specifications allow the NW to configure only one DRX configuration for a serving cell and, therefore, this single DRX configuration cannot really match the traffic characteristics of all XR flows. Thus, it cannot closely follow the potential short sleep opportunities and result in either low power saving gains, or in an increased traffic delay.

There have been several contributions in last RAN1 meeting proposing to study solutions with multiple active DRX configurations each of them aligned with a different XR flow/stream. The motivation is that a single DRX configuration is not efficient from power saving perspective since traffic characteristics are too different and can’t be “tracked/covered” by a single DRX configuration.

With a single data flow for the UE, if the DRX periodicity matches the traffic arrival of that data flow, then dynamic grant can be used as the main means to support XR traffic. However, no XR use case is of a single data flow: for VR, there is video data flow in the DL and pose/control data flow in the UL with different periodicities. For AR, with for example two data flows in the DL (video, audio/data) and three data flows in the UL (video, pose/control, audio/data) with different periodicities, it can be difficult to determine a periodicity and Onduration which satisfies requirements of all these data flows thereby enabling a good power saving gain.

When there are multiple data flows, especially when at least integer relationship does not exist among the periodicities of the different data flows, there might be the risk that packet arrival of some data flows arrival occurs in the DRX-Off duration, e.g., UE not monitoring PDCCH which would in turn lead to some additional delay.

Having multiple simultaneous DRX configurations could help schedule transmissions corresponding to different traffic characteristics in a timely manner, e.g., by providing on-durations on several occasions according to expected traffic arrivals. For instance, video traffic can be a pseudo periodic traffic expected with inter-arrival times of about 1/fps (e.g., 16.67 ms corresponding to 60 ms); and a voice traffic may be a periodic traffic with 10 or 20 ms periodicity.

It has been proposed to have the UE monitor the PDCCH while the drx-onDurationTimer (or drx-InactivityTimer) is running in any of the DRX configurations, i.e., the overall active time is a logical ‘OR’ of the active times given by each DRX configuration.

We also see multiple active DRX configurations or the DCI-based switching between DRX configurations as potential enhancements for reducing PDCCH monitoring. DCI-based switching between DRX configuration refers to the case that NW dynamically activates/re-activates/deactivates DRX configurations, which allows for a dynamic adaptation of already activated DRX configurations or activation of additional DRX configuration.

Proposal 2: RAN2 to study the usage of multiple active DRX configuration and the DCI-based switching/activation of DRX configurations.

- Reference SFN for determining the DRX start offset in case of SFN wrap round

Figure 2. An example of the DRX cycle mismatch in case of SFN wrap round

The repeated DRX pattern based on the DRX formula in every hyper frame period does not match the DL traffic arrival pattern in case of SFN wrap round as depicted in figure 2. The DRX mismatch issue in case of SFN wrap round causes additional power consumption to keep UE longer awake.

In R16 IIOT WI, similar mismatch issue between CG occasion and traffic periodicity in case of SFN wrap round has been discussed and solved in R16 CG enhancements. For simplicity and similar principle, RAN2 should discuss whether to reuse a similar timeReferenceSFN IE with value 0 or 512 for DRX configuration for determining the closest SFN preceding the reception of DRX configuration related to DRX Offset for determining the first DRX on-duration occasion to avoid the DRX cycle mismatch between UE and NW in case of SFN wrap round, and then determine the subsequent DRX-On duration based on a time of periodicity shift.

Proposal 3: Support to introduce drx-timeReferenceSFN IE having {0,512} for C-DRX to determine the SFN related to C-DRX start offset and always determines the subsequent DRX-On duration by shifting the integer value of the DRX periodicity.

- Non-uniform monitor occasions in the DRX-OnDuration for DL arrival jitter handling

Figure 3. An example of the arrival jitter of DL XR traffic

In Release 17, RAN1 agreed that video traffic suffers from random jitter around its mean period (typically ±4 ms), covering a long range of 8 ms due to the jitter caused by the encoding delay and network transfer time. To avoid the transmission delay, a simple solution using existing DRX functionality, is to configure the drx-onDurationTimer to run during the entire jitter range. However, given the long jitter range, the UE would monitor the PDCCH for a long time, which would lead to limited power saving gains, especially for video periods shorter than 16.67 ms.

We thus propose to introduce a two-stage DRX solution as follows. For example, drx-onDurationTimer is configured by NW considering the jitter range for UE, UE first determining the start time of DRX on-duration based on DRX formula to match the traffic arrival period without jitter based on proposal 1. And then UE monitors PDCCH occasions with uniform gap during the DRX_OnDurationTimer is running. For example, UE monitors the PDCCH occasions configured by a bitmap, e.g. bitmap is superimposed on the drx-onduration. Compared to legacy SSSG switching, this has no additional switch delay and switch signalling.

Proposal 4: Support to configure non-uniform PDCCH monitoring occasions during the DRX_OnDuration.

- Conclusion

In this contribution, we discuss power saving enhancements for XR traffic. We have the following proposals:

Proposal 1: RAN2 to discuss the following options to avoid mismatch between DRX cycle with the XR traffic non-integer periodicity:

- Option 1. Dynamically adjust C-DRX parameters e.g., start offset or cycle length for the next DRX cycle

- Option 2. Several inner DRX cycle within an outer DRX cycle

- Option 3. New DRX formula based on the rounding value of the DRX non-integer periodicity

Proposal 2: RAN2 to study the usage of multiple active DRX configuration and the DCI-based switching/activation of DRX configurations.

Proposal 3: Support to introduce drx-timeReferenceSFN IE having {0,512} for C-DRX to determine the SFN related to C-DRX start offset and always determines the subsequent DRX-On duration by shifting the integer value of the DRX periodicity.

Proposal 4: Support to configure non-uniform PDCCH monitoring occasions during the DRX_OnDuration.

References

RAN1#110bis-e chairman note.

RAN2#119bis-e chairman note.

R2-2209938, Discussion of DRX enhancement, Lenovo