3GPP TSG-RAN WG2 Meeting #120	R2-2212053

Toulouse, 14th November – 18th November, 2022

Agenda item:		8.3.4

Source:		Lenovo

Title:		Cell selection/re-selection in NES

Document for:		Discussion and Decision

# Introduction

The following progresses on Cell Selection/Re-selection for NES techniques are achieved in RAN2 #119bis-e [1].

Agreements:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

In this contribution, we further discuss the related issues.

# Discussion

Considering the network deployment and progress, there could be a possible scenario as shown in Fig. 1 that a NES neighbours to the legacy cell and the NES UE attempts to camp on one of the cells which provide overlapped coverage to the UE. It was agreed that the network should be able to configure NES capable UEs to (de)prioritize NES cells. The (de)prioritization of NES cells cannot be ensured only by the existing S criterion for cell selection and frequency priority for cell reselection. In order to support NES capable UEs to (de)prioritize NES cells, the NES capable UEs shall at least be able to know which cell is the NES cell. For example, the NES cell broadcasts the related information.

Figure 1 NES UE at the edge of NES cell and legacy cell

Proposal 1: In order to enable the NES capable UEs to (de)prioritize NES cells, the NES identification will be supported.

Referring to the SID, A NES cell can be defined as a cell which supports NES technologies including supports different levels of energy savings by different levels of sleep and supports dynamic/semi-static transition among different levels of energy savings.

Proposal 2: A NES cell can be defined as a cell which supports any of the NES means including supports different levels of energy savings by different levels of sleep and supports dynamic/semi-static transition among different levels of energy savings.

Further, as shown in Fig.1, there could be multiple cells including NES cell and legacy cell that satisfy the S criterion. Referring to the existing mechanism, the UE shall perform ranking of all cells that fulfil the cell selection criterion S. Then, the strongest NES cell of the highest frequency priority in order to enable the NES capable UEs to (de)prioritize NES cells.

Proposal 3: The NES UE may select/reselect the strongest NES cell on the highest priority frequency.

Another issue was discussed and it was agreed that “Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell”. That is to say the network shall be able to bar legacy UE and allow the access attempt of NES UEs only. The legacy UE could be unknown of the NES cell, for example, it is not necessary for the legacy UE to identify the type of the cell (e.g. NES cell) by which its access attempt is barred.

Observation 1: It could be unknown of the NES cell to the legacy UE, for example, it is not necessary for the legacy UE to identify the type of the cell (e.g. NES cell) by which its access attempt is barred.

And from the perspective of UE power saving, it’ll be better for the legacy UEs to be barred. Therefore, the legacy “cellBarred” can be used to bar the access attempt of legacy UEs.

Proposal 4: The legacy UE is barred by the legacy “cellBarred” when the legacy UE attempts to access to a NES cell in a level of energy savings.

# Conclusion

In this contribution, we discuss the Cell Selection/Re-selection in Rel-18 NES and provide the proposals:

Proposal 1: In order to enable the NES capable UEs to (de)prioritize NES cells, the NES identification will be supported.

Proposal 2: A NES cell can be defined as a cell which supports any of the NES means including supports different levels of energy savings by different levels of sleep and supports dynamic/semi-static transition among different levels of energy savings.

Proposal 3: The NES UE may select/reselect the strongest NES cell on the highest priority frequency.

Observation 1: It could be unknown of the NES cell to the legacy UE, for example, it is not necessary for the legacy UE to identify the type of the cell (e.g. NES cell) by which its access attempt is barred.

Proposal 4: The legacy UE is barred by the legacy “cellBarred” when the legacy UE attempts to access to a NES cell in a level of energy savings.

# References

RAN2#119bis-e chairman notes.