3GPP TSG-RAN WG2 Meeting #120	R2-2212054

Toulouse, 14th November – 18th November, 2022

Agenda item:		8.3.5

Source:		Lenovo

Title:		NES impact to UE mobility

Document for:		Discussion and Decision

# Introduction

The following progresses for NES techniques are achieved in RAN2 #119bis-e [1] after the issue on connected mode mobility was discussed.

=>	Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study.  FFS whether any enhancements is needed.

=>	FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation

=>	As a first priority, discussion on RAN2 group handover are confined to the CHO framework

In this contribution, we further discuss the related issues.

# Discussion

There could be the following cases from a network deployment perspective:

Case a) a cell implementing Rel-18 network energy savings techniques (NES cell) neighbors to the legacy cells;

Case b) a NES cell neighbors to the NES cells;

Case c) a NES cell neighbors to the legacy cells and NES cells as shown in Figure 1;

Considering the mobility of the UE that supporting the NES technique (NES UE), the UE could handover from the NES cell to the NES cell or legacy cell. Correspondingly, it could be possible that a NES UE connected to a legacy cell, and the UE handovers from the legacy cell to a NES cell. The above cases are all within the NES scope of considerations.

Observation 1: A NES UE can handover between an NES cell and a non-NES cell (legacy or otherwise).

Figure 1 NES cell neighboring to legacy cell and NES cell

For case c), a NES UE is at the edge of NES cell #1, for example the NES cell #1 is a single carrier or the anchor carrier of the NES UE. Both the NES cell #2 and the legacy cell #3 could be the target cell of the UE according to the measurement report. Shall the source cell prioritize the NES cell when selecting of the target cell for the NES UE to better supporting NES?

Proposal 1: RAN2 to discuss whether the NES cell(s) shall be prioritized for the NES UE among the target cells.

On the other side, if a legacy UE is at the edge of a cell. Both the NES cell and the legacy cell could be the target cell of the UE according to the measurement report. It is possible for the NES cell to apply the energy saving technology, for example, Longer SSB/SIB1 periodicity, state transition to sleep mode, etc. The cell could change to an unsuitable cell for a legacy UE after the legacy UE handovers to NES cell. Although the network will hand over the legacy UE to another cell, it introduces signaling overhead. Shall the NES cell be de-prioritized for the legacy UE among the candidate target cells?

Proposal 2: RAN2 to discuss whether the NES cell(s) shall be de-prioritized for the legacy UE among the target cells.

The corresponding aspects can be considered in the case of CHO, for example, the cell supports NES can be configured to the NES UE as one of the conditions in CHO case. Also, it was summarized as the Solution 2: Enhanced CHO by prioritizing NES cells in CHO condition evaluation [2] after the RAN2#119bis email discussion. However, it seems not easy to configure the corresponding condition to the legacy UE.

Proposal 3: RAN2 to discuss how to configure the conditions to prioritize NES cells in CHO condition evaluation.

# Conclusion

In this contribution, we discuss the handover aspects in Rel-18 NES and provide the proposals:

Observation 1: A NES UE can handover between an NES cell and a non-NES cell (legacy or otherwise).

Proposal 1: RAN2 to discuss whether the NES cell(s) shall be prioritized for the NES UE among the target cells.

Proposal 2: RAN2 to discuss whether the NES cell(s) shall be de-prioritized for the legacy UE among the target cells.

Proposal 3: RAN2 to discuss how to configure the conditions to prioritize NES cells in CHO condition evaluation.

# References

RAN2#119bis-e chairman notes.

R2-2209758, Discussion on Network energy saving for CONNECTED UE - group CHO and BWP adaptation,	Apple