3GPP TSG-RAN WG2 Meeting #120                             	R2-2212060

Toulouse, France, November 14 –18, 2022

Agenda item:	8.3.4

Source:	Samsung

Title:	Discussion on Cell Selection and Reselection for NES

Document for:	Discussion & Decision

# Introduction

In RAN2#119bis-e meeting, RAN2 made the following agreements:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

Also, RAN2 email discussion [POST119bis-e][304] provided an open issue list [1]. This document discusses those open issues.

# Discussion

In RAN2#119bis-e, RAN2 had a consensus on necessity of de-prioritizing NES cell for NES-capable UEs. A remaining issue is whether prioritizing NES cell is needed. We see the load-balancing is a valid scenario. For instance, the NES cell is using NW DTX/DRX mechanism where gNB periodically or temporarily switches off its TX/RX functionalities. Since legacy UEs do not understand the NW DTX/DRX, the legacy UEs are most likely to be barred or strongly de-prioritize the NES cell. There could be only small number of NES-capable UEs in the area. In this case, for NES-capable UE, it would be much better to select or reselect the NES cell. The NES cell has enough capability to serve more UEs and the UEs can access the NES cell without performance degradation.

Whether it is beneficial highly depends on ratio or presence of NES-capable UEs among total UEs in the coverage, which is related to operator’s deployment scenario. Different operators have different situations. Hence, the flexibility of prioritization of NES cell is beneficial for operators.

Observation 1. Different operators have different situations. Hence, the flexibility of prioritization of NES cell is beneficial for operators.

Proposal 1. gNB configures whether NES-capable UE prioritizes NES cell over non-NES cell.

The next issue is mechanism to incentivize NES-capable UEs camping on cells. The latest TP [2] captures “The NW should be able to configure NES-capable UEs to prioritize/de-prioritize a specific NES cell or NES cells on a specific frequency.” We see frequency or cell prioritization mechanism during cell selection and reselection is sufficient. No additional mechanism is needed.

Proposal 2. Additional mechanism to incentivize NES-capable UEs camping on cells is not necessary.

# Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. gNB configures whether NES-capable UE prioritizes NES cell over non-NES cell.

Proposal 2. Additional mechanism to incentivize NES-capable UEs camping on cells is not necessary.

# Reference

[1] R2-2211428, Report of [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less (Huawei), Huawei, HiSilicon

[2] R2-2211427, TP on cell selection / reselection and SSB / SIB-less, Huawei, HiSilicon