3GPP TSG-RAN WG2 #120	Tdoc R2-2212093

France, Nov. 14th – 18th 2022

Agenda Item:	8.13.7

Source:	Ericsson

Title:	SON support for NPN

Document for:	Discussion, Decision

# 1	Introduction

During RAN2#119bis, the following were agreed:

- SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

- Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case.

- RAN2 prioritizes the use cases of RLF report and logged MDT enhancement for NPN.

This contribution aims to discuss the SON and MDT enhancements for private networks optimization including both SNPN and PNI-NPN scenarios.

# Discussion

## 2.1	Co-existence of SON/MDT reports for SNPN and PN

Collecting data as part of SON/MDT reports for SNPN and PN, may become significantly inefficient in particular when the UE moves back and forth between SNPN and PN.

Considering the scenario that SNPN is deployed/managed by the enterprises independently from the PN, the data collection as part of SON/MDT features lend itself special considerations in particular when users connected to the SNPN moves frequently between SNPN and PN. Therefore, we think a user (holding a UE) moving frequently between SNPN and PN should not ruin the SON reports collected in one network (PN) by collecting the SON reports in the other network (SNPN) and vice versa.

A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected data (e.g., RLF) for one network in the other networks, or revealing sensitive data (e.g., MHI) of one network to the other networks.

In the following we discuss different SON reports and MDT related data collection and the need for co-existence of the reports at the UE to avoid revealing the data of one network to the other network while collecting the data for both network avoiding one network damaging the other network’s data collection.

#### Co-existence of SNPN and PN SON reports

In the following, with some examples, we discuss different aspects of SON reports collection and reporting in particular when PN and NPN exist together. Although RA report and MHI are not the highest priority for NPN as of now, we would like to bring up the attention on the issues we may face later. Hence it seems necessary to consider different SON reports when designing the solution as otherwise we may end up with a solution that is not future proof i.e., may become problematic for some of the SON functions.

RA report: A UE in PN network may collect up to 8 RA report in PN (not being fetched by the network) and moving from PN to SNPN leads to additional RA reports to be collected for the SNPN. On the basis of the current TS 38.331 the UE would not be able to log RA report for the SNPN since it already logged 8 RA reports collected in PN. In addition, the UE would not be able to report the RA reports collected from the PN to the SNPN since the PLMNs may not be equivalent. Hence an SNPN as an independent network would not be able to collect SON reports without having impact form PN network. The same situation can be foreseen if the UE collect the SON reports with the same approach as is for the PN network i.e., the SON reports collection in PN network can be affected/damaged by the SON reports collected by the NPN.

UE cannot collect RA reports for SNPN if it collects 8 RA reports in PN that are not fetched by the PN and vice versa, unless deleting the PN SON reports that leads to wastage and under-utilized SON features.

RLF Report: Considering the RLF report the UE may log an RLF report in PN and then move to an NPN and experience another RLF leading to yet another RLF report. As of now the UE can store only one RLF-Report, hence if this situation occurs, the UE may delete the RLF report and other SON information collected for the PN. We believe that this situation should be avoided, so that SON reports collected for operations performed in independent networks do not affect each other.

A UE having an RLF report in PN, may risk deleting its PN RLF if it experiences another RLF in SNPN. This (upon frequent move between PN and SNPN) may ruin SON reports of an independent network by another independent network co-existing together.

This accordingly leads to a situation that the mobility issues e.g., RLFs occurring at the boundaries of the SNPN and PN remains undetected to the network, as most of the RLF reports of one network may be ruined by the RLF occurring in the other network.

Deleting the RLF reports may lead to a situation that the mobility issues at the boundaries of PN and SNPN remains hidden to the networks.

Mobility History Information: In yet another critical example, the UE may continue to log the MHI information when mobbing from a PN to NPN. According to the current MHI solution in TS38.331, the UE logs visited cells as part of MHI in a PN networks and upon mobility to a SNPN, it continues adding new visited SNPN cell. This implies that the UE logs and reports mix of PN and NPN visited cells for MHI which means the information concerning the UEs connected to one NPN may be reported to another NPN or a PN. The same issue may exist in various way for other SON reports e.g., SHR, or CEF report.

Given the examples above, we propose that a UE served by PN that enters SNPN, logs SON reports associated to the SNPN in separate variable not damaging/ruining the data collected for the PN network and not revealing sensitive data (e.g., MHI) to the other network. Given that, with separate variables, the UE can collect and report the SON related information and measurements to the corresponding network without ruining the other networks SON report and without revealing sensitive information between networks Therefore, we propose the following.

UE logs SON reports for SNPNs in separate variables.

#### 2.2.2  Co-existence of MDT reports for SNPN and PN

In the following we discuss different aspects of MDT reports collection and reporting to the network.

As of now, and basing the current TS38.331, a UE in PN may collect logged MDT measurements and move to NPN and may continue to log the MDT related information (and vice versa). This leads a UE to collect both PN and NPN related logs in the logged MDT report even if SNPN and PN are totally independent networks and controlled by separate entities.

A UE in PN may collect logged MDT measurements and move to NPN, will continue logging the MDT related information (and vice versa). This leads a UE to collect both PN and NPN related logs in the logged MDT report even if SNPN and PN are totally independent networks and controlled by separate entities.

On the other hand, given that the TCE ID is known PLMN-wide (i.e., not known in SNPNs) if the MDT reports logged in the PN node is reported to SNPN node, since the Trace ID in the logged MDT report is not known for the SNPN gNBs, the logged MDT report cannot be forwarded to the TCE. In our opinion, a UE with subscription to both public and private networks i.e., SNPN, can log MDT reports associated to the SNPN in a new variable instead. Therefore, the MDT reports associated to PN is not overridden by the other network types i.e., SNPN. With this, the UE can report the associated logged MDT related information to a network wherein the event that triggered the report occurred.

UE logs MDT report for SNPNs in a separate variable.

If the above proposals are agreeable RAN2 needs to discuss how to assure co-existence of the PN and SNPN SON reports in particular whether the UE needs to consider additional memory or  it is possible to share the existing memories between PN and SNPN SON/MDT reports e.g., sharing 68KB of memory allocated to the logged MDT report or doubling the memory size i.e., 64 KB memory for PN MDT and 64 KB memory for NPN MDT.

RAN2 discuss whether to consider additional memory e.g., additional 64KB memory for the SNPN MDT report or share the existing memory between SNPN and PN reports.

In addition, some companies proposed to discuss out of coverage (OoC) in MDT report when UE is detecting coverage holes e.g., an area that no sufficient signal level is detected by UE for accessing specific services. In our opinion, when there is no suitable cell for a UE to camp on while the UE is requested to access a specific service, the UE logs an OoC indication in either RA report or CEF or new report. With this information, the network can identify the OoC for NPN. It is worthwhile to mention that particularly in scenarios that the upper layers (e.g., an SNPN service/application) request the UE to join an SNPN network and UE find no suitable cell for the requested SNPN, there might not be an appropriate logged MDT configuration from the same network at the UE to log the OoC event in the corresponding MDT report. Hence for such critical scenarios (in which the SNPN applications request the UE to register to an SNPN network) we can not rely on the MDR reports. Hence, we propose:

UE logs an out of coverage (OoC) indication either in RA report or CEF report or a new report.

## 2.3	Building coverage map for NPN and PN

When both PN and NPN are deployed in an area and MDT reports are used to build coverage map of the area, it is important to the network knowing from the MDT report which measurements cells are providing coverage to NPN and which cells are providing coverage to PN. More specifically, as we know one use case of the MDT report is to build coverage map for a given network. As of now the UE includes the measurements results of the neighboring cells associated with the PCI of the neighboring cells in the MDT report. Based on the current solution a OAM is not capable to differentiate the network type based on the provided measurements (i.e., PCI and the RSRP measurements of the cells in an area). Hence it is not possible to build a coverage map for a specific network.

Furthermore, UE access to a NPN cell is restricted to CAG ID or SNPN ID on cell level and a UE does not necessarily have any network selection information i.e., CAG ID or SNPN ID on frequency layers.

In the following, we discuss the required enhancements in the MDT report as well as MDT configuration, to enable the network to build the coverage map for a specific network e.g., PN or NPN.

    - Enhancement on the MDT report

In our understanding, since a UE only includes PCI and carrier frequency information for the neighbor cells in the logged MDT report (on frequency level), it is not possible for the network to identify whether the cell provides coverage to NPN or not (on cell level).

As of now, a UE reports a radio measurement (e.g., neighboring cells measurements) in MDT report without any indication of the neighbor cells type (e.g., PN or NPN type), when both PN and NPN are deployed in an area.

By collecting logged measurements from the UE, where the network type is also included, the network can derive an accurate coverage map of each of the network types. This allows for optimization of network performance and better fulfilment of the service level agreements for each of the networks. Thus, we propose the following.

RAN2 enhance the logged MDT report with cell type indication (e.g., SNPN cell) as part of the measurement results.

    - Enhancement on the MDT Configuration

Similarly, a PN UE may collect MDT measurements on configured target frequencies for neighbor cells belonging to NPN when the cells operate on the same target carrier frequency. In other words, the UE collects both PN and NPN related MDT measurements in logged MDT report wherein two neighbor NPN and PN cells are sharing the target carrier frequency (e.g., configured as part of InterFreqTargetInfo). Therefore, building a coverage map for network types e.g., PN, or NPN for the area may not be enough accurate.

As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

We request RAN2 to discuss UE behavior for collecting logged MDT measurements concerning the target carrier frequency:

RAN2 enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A SNPN user (holding an SNPN allowed UE) may move frequently between SNPN and PN which may ruin collected data (e.g., RLF) for one network in the other networks, or revealing sensitive data (e.g., MHI) of one network to the other networks.

Observation 2	UE cannot collect RA reports for SNPN if it collects 8 RA reports in PN that are not fetched by the PN and vice versa, unless deleting the PN SON reports that leads to wastage and under-utilized SON features.

Observation 3	A UE having an RLF report in PN, may risk deleting its PN RLF if it experiences another RLF in SNPN. This (upon frequent move between PN and SNPN) may ruin SON reports of an independent network by another independent network co-existing together.

Observation 4	Deleting the RLF reports may lead to a situation that the mobility issues at the boundaries of PN and SNPN remains hidden to the networks.

Observation 5	A UE in PN may collect logged MDT measurements and move to NPN, will continue logging the MDT related information (and vice versa). This leads a UE to collect both PN and NPN related logs in the logged MDT report even if SNPN and PN are totally independent networks and controlled by separate entities.

Observation 6	As of now, a UE reports a radio measurement (e.g., neighboring cells measurements) in MDT report without any indication of the neighbor cells type (e.g., PN or NPN type), when both PN and NPN are deployed in an area.

Observation 7	As of now, building coverage map for an area (including both PN and NPN coverage) can be misleading for coverage analysis purpose, as MDT reports does not reflect the cell type (e.g., PN or NPN type), and a coverage of NPN maybe counted as coverage of PN and vice versa. Note that neighbouring cells measurement in MDT reports includes only PCI info of the cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE logs SON reports for SNPNs in separate variables.

Proposal 2	UE logs MDT report for SNPNs in a separate variable.

Proposal 3	RAN2 discuss whether to consider additional memory e.g., additional 64KB memory for the SNPN MDT report or share the existing memory between SNPN and PN reports.

Proposal 4	UE logs an out of coverage (OoC) indication either in RA report or CEF report or a new report.

Proposal 5	RAN2 enhance the logged MDT report with cell type indication (e.g., SNPN cell) as part of the measurement results.

Proposal 6	RAN2 enhance the MDT configuration (interFreqTargetInfo) to enable logging only NPN or PN cells per frequency.

# References

[RP-221825], Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC, 3GPP TSG RAN meeting #96, Budapest, Hungary, June 6-9, 2022.