3GPP TSG RAN WG2#120	           R2-2212096

Toulouse, France, 14th – 18th November 2022

Agenda item:	8.2.2

Source: 	Lenovo

Title: 	On SL Positioning Protocol and Architecture Aspects

Document for:	Discussion

# Introduction

The Rel-18 NR positioning evolution SID was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included the study of potential solutions to enable SL Positioning. The following agreements were made during RAN2#119-e [2] and RAN2#119bis-e [3] relating to the SL Positioning study:

RAN2#119-e: Agreements: Proposal 1 (modified): Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. FFS if partial coverage case assumes anything about which UEs are in coverage. Proposal 2: Study the architecture and signaling procedures to enable at least the following two operation scenarios: -Operation Scenario 1: PC5-only-based positioning. -Operation Scenario 2: Combination of Uu- and PC5-based positioning. Agreement: RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role. Agreement: Proposal 4 (modified): Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion: -Target UE: UE to be positioned -Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. FFS: clarification of the knowledge of the anchor UE. Additional roles can be considered. Agreements: Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP). FFS where it is specified. The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355). Agreement: Study the potential impact to LPP for support of sidelink positioning procedures between UE and LMF. FFS how much impact (if any), e.g., only to carry the new protocol, and if the PC5-only and hybrid PC5+Uu cases are the same or different. Agreement: RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers. Agreement: RAN2 will study the question of cast type for positioning signalling. For SL-PRS, follow RAN1 decision and consider cast type if something arises in RAN2 scope.

RAN2#119-bis-e: Agreement: Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work. 1. Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF 2. Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP 3. Use of SLPP/RSPP between the UE and the LMF Agreement: Proposal 3 (modified): In order to enable sidelink positioning, SLPP/RSPP shall support at least the following functionalities: 1. SL Positioning Capability Transfer 2. SL Positioning Assistance Data exchange 3. SL Location Information Transfer 4. Error handling 5. Abort This agreement does not imply any specific signalling structure. Agreement: Proposal 5: Unicast/one-to-one operation is assumed as baseline for exchange of sidelink positioning signaling. Proposal 6 (modified): RAN2 shall study applicability of at least the following positioning signaling for groupcast/broadcast (in addition to unicast), including addressing any security aspects (involving SA3 where needed). FFS the specific use case: • SL positioning capability transfer • SL positioning assistance data • FFS SL location information transfer Agreement: Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation. FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless. Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs. RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

This contribution provides a detailed discussion into the potential areas to develop in order to support the necessary functionality and procedures to perform SL positioning.

# SL Positioning Protocol and Architecture

## Location of new SL Positioning Protocol Stack (SLPP/RSPP)

LPP serves as point-to-point positioning protocol or UE-terminated protocol between a location server (E-SMLC, LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources, which may include GNSS (RAT-independent methods) or NG-RAN node for RAT-dependent methods. LPP covers the initiation or maintenance or termination of one or more LPP sessions, management of different transactions and other aspects to enable different location procedures. Normally, the NAS layer (control plane) or OMA SUPL (secure user-plane) may be utilised to transfer such LPP messages.

Referring to SA2’s latest LS [4], SA2 has concluded that the RSPP/SLPP protocol is to be located under the application layer but was unable to reach consensus as to whether the new positioning protocol is to be located over the V2X/ProSe layer or over the AS Layer. Since SL positioning requires tight coordination between the RAN/AS layers and the higher-layers, and most the enabling features require a good degree of RAN interaction such as selection of positioning technique, selection of anchors, etc. where such information may be transparent to the higher-layers. However, it is recommended that RAN2 continue to study pros and cons of the location of the new positioning protocol layer based on the different options provided by SA2.

Proposal 1: RAN2 to further study the location of SLPP/RSPP layer based on the required SL Positioning functionality.

Another issue is whether the SLPP is carried over SL control plane or user plane, each with its own merits and benefits. The decision on which interface to transmit SLPP/RSPP will have an effect on the subsequent design including the need for QoS provisioning, SL positioning message size constraints, etc. In addition, the choice of utilising PC5-S or PC5-U will have the following trade-offs in terms of additional specification work as also highlighted by SA2:

- PC5-S is not supported for groupcast and broadcast in terms of link management. At least link management may be necessary for support certain positioning methods such as SL-TDoA.

- PC5-U on the other hand supports all cast types but the current operation is session less for groupcast and broadcast.

Another aspect worth further investigation in terms of the CP-solution is if the new SLPP/RSPP protocol is based atop of the PC5 RRC layer, however this is currently only supported for unicast transmissions and dependent of initial PC5-S establishment. The QoS design would also be dependent on whether UP-based or CP-based signalling is considered for the new positioning protocol.

SA2 also proposed the use of PC5-D (Discovery interface) for carrying the SLPP/RSPP messages/payload, which may require the need for a physical discovery channel originally supported for Rel-12/Rel-13 D2D communications. In any case, in order to discover the UEs involved in a SL positioning session including Anchor UEs and target-UE, a discovery mechanism may be required. However, it should be noted that the discovery interface should carry SLPP/RSPP messages related to the discovery of anchors and not all SLPP/RSPP messages as could be interpreted by the SA2 LS [4].

Proposal 2: RAN2 to further study the support for groupcast and broadcast in order to assess the merits between PC5-S and PC5-U in terms of the transport of SLPP/RSPP messages.

Proposal 3: RAN2 to support discovery procedures for at least anchor UE(s) and target-UE. FFS the AS layer impacts of discovery procedures.

## SL Positioning ID Management

SA2 has also requested the RAN WGs input for several RAN parameters, including the type of identifiers required to support SL positioning. SL communications makes self-generated source-IDs and higher-layer generated destination IDs in order to support different communication cast types. Similarly, in order to identify the different types of positioning nodes/UEs involved, RAN2 is recommended to study whether existing UE ID management is sufficient or whether a dedicated SL positioning ID management needs to be supported in the new positioning layer.

Proposal 4: RAN2 to further study whether existing/new SL ID management needs to be supported for SL positioning.

## Architecture

The Uu positioning architecture is primarily based on the involvement of the LMF and as such can only operate in in-coverage scenarios with a network deployment supporting an LMF. In the case of SL positioning, the use cases span across different vertical applications (e.g., V2X, Public Safety, IIoT and Commercial ranging), which have different requirements in terms of coverage scenario operation. The proposed SL positioning architecture should also support partial and out-of-coverage scenarios. This aspect was extensively studied during the RAN study in TR 38.845 [6], where the SL positioning framework should cater to all coverage scenarios.

Furthermore, due to the distributed nature of SL positioning there are multiple entities which may act as configuration and positioning calculation entities, e.g., RSU, gNB, target-UE, anchor-UE, other assistant UEs, positioning reference units, etc., which is different from the Uu operation where the LMF was responsible for both positioning configuration and positioning calculation. It is, therefore, important to consider the involvement of such entities within the SL positioning architecture.

SA2 has also further concluded that the LMF may be involved in the SL positioning procedures between a Target UE and Anchor UE, which may be a standalone extension of the LPP, a new protocol or both. RAN2 has further agreed such aspects including whether or not to extend the existing protocols may be best considered during the normative work. Based on the LMF involvement, the UE may need to support both LPP and RSPP/SLPP. This may be inherently limiting for UEs wishing to only perform ranging with respect to other UEs in the network, which only support SLPP/RSPP.

Observation 1: For UEs wishing to only perform ranging in-coverage and without LPP capabilities, LMF involvement may be unnecessary.

Proposal 5: RAN2 to support both LMF-dependent (e.g., UEs supporting both LPP and SLPP/RSPP) for in-coverage scenarios and LMF-independent (e.g., UEs only supporting SLPP/RSPP) SL positioning architectures for all coverage scenarios including in-coverage, partial coverage and out-of-coverage.

### SL Positioning Server UE

SA2 has clarified that the role of a SL Positioning Server UE is applicable for partial coverage and out-of-coverage scenarios and only in cases that a constrained UE is not able to support all the SL Positioning/Ranging features. In addition, such UEs may be discoverable and further selected for result calculation purposes only.

Based on the SA2 LS, the need for a SL Positioning Server UE is not so clear from RAN perspective, since it caters to only certain limited scenarios in SL positioning based on coverage situations and UE capabilities. It is assumed that a target UE and anchor UEs have sufficient capabilities to perform SL Positioning including, configuration, measurements and result calculation across all coverage scenarios. It is not clear why certain UEs may not perform result calculation as this was supported in previous releases, i.e., UE-based positioning. In addition, the result calculation is usually up to implementation. Based on the SA2’s latest LS [4], the scope of introducing additional discovery and selection procedures may increase the complexity of the SL positioning framework. However, the aforementioned issues may be resolved if the SL Positioning Server UE may be integrated as part of the Anchor UE or Target-UE.

Observation 2: A SL Positioning Server UE as a separate UE is only limited to partial and out-of-coverage scenarios and applicable to SL positioning scenarios involving limited/constrained capability UEs at least without result calculation capabilities.

Proposal 6: RAN2 to consider support of “SL Positioning Server UE” as a functionality which is part of an Anchor UE or Target-UE.

Proposal 7: Down-prioritize the support for a separate SL Positioning Server UE from RAN2 perspective since it only serves very few limited scenarios and increases the complexity of SL positioning procedures, e.g., additional discovery and selection procedures.

# SL Positioning Techniques

## SL RAT-dependent Positioning techniques

The potential SL positioning/ranging techniques should be able to support different scenarios, especially given that anchor nodes (whose location is usually known) and non-anchor nodes (whose location is usually unknown or only coarsely known) can be considered given the distributed nature of SL communications. The SL RAT-dependent positioning techniques to be supported may be summarized as follows:

- SL timing-based positioning methods

  - SL-TDoA

  - SL-RTT (including one-way and two-way RTT methods)

- SL angular-based positioning methods

  - SL-AoD

  - SL-AoA

- SL Location fingerprinting methods

  - Exploiting RRM measurements such as SL RS RSRP/RSRQ measurements

RAN1 has already agreed on a few supported SL positioning methods including SL-RTT, SL-AoA and SL-TDoA. As such RAN2 should coordinate with RAN1 on the supported positioning techniques by awaiting further feedback on the feasibility of the different timing-based and angle-based SL positioning techniques.

Proposal 8: RAN2 to support the signalling design for at least SL-RTT type methods, SL-TDoA, SL-AoA.

### Anchor UEs/RSUs

The positioning requirements agreed in RAN1#109-e [7], target both absolute and relative positioning requirements. For each of these cases, it is expected that anchor UEs with relative stable mobility patterns and known UE locations, may be configured to participate in SL positioning/ranging session. This is especially required for positioning techniques requiring 2 or more anchor nodes for enhanced positioning performance.

Observation 3: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques and in other cases improve positioning performance.

One of the key responsibilities of anchor nodes, would be to transmit/receive SL-PRS, perform SL positioning measurements and reporting, as well as any other key positioning-related tasks. In the case of anchor UEs, there are additional issues worth studying including the selection, triggering and configuration of anchor nodes to perform the aforementioned tasks. And the solutions for anchor nodes issues should be studied for both LMF-dependent SL positioning architecture and LMF-independent SL positioning architecture as discussed in Section 4, to support in coverage, partial coverage and out of coverage scenario. For LMF-dependent SL positioning architecture, LMF may need to perform management to the anchor nodes in addition to legacy target UE, while for LMF-independent SL positioning architecture, the anchor nodes management (e.g., including (re)selection, triggering and configuration) may be controlled by gNB if in-coverage or determined by an UE that starts/initiates the SL positioning session, and therefore the corresponding coordination procedures need to be studied.

According to SA2’s LS [4], further clarity is required on whether Assistant UE selection/reselection is required from RAN perspective. From RAN perspective, since the Anchor UE may fulfil the role of the assistant UE, the same triggering, (re-)selection, and configuration procedures may be applicable to an assistant UE (if need of such a UE is determined).

Proposal 9: RAN2 is suggested to investigate the procedures related to anchor UE(s) including triggering, (re)selection, and configuration when performing sidelink positioning procedures. The same procedures may be applicable to an Assistant UE (if need for such a UE is determined).

# SL Positioning Configuration and Resource Allocation

## General

In the context of the Uu positioning, the LMF is a key central entity coordinating the PRS/SRS resource configuration with the serving and neighbouring gNBs via the NRPPa interface, while in SL the gNB is responsible for the configuration and resource allocation of all SL UEs, while in-coverage. In order to avoid additional specification efforts, it is recommended that SL positioning resource allocation mechanism reuses many of the principles that already exist to enable SL communications.

Observation 4: For Uu positioning, the LMF coordinated the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

## Configuration

The SL positioning configuration may comprise of all necessary parameters required to support a SL positioning/ranging session. This may include not only time-frequency resources or other physical-layer parameters, but also the type of configured positioning techniques, selected and configured anchor nodes, QoS information, and additional assistance information that may assist the configuration node with the SL positioning configuration Figure 3 is a general conceptual overview of the SL positioning configuration, measurement and reporting flow for SL positioning.

Figure 3: SL positioning configuration, measurement and reporting flow for SL positioning

Proposal 10: RAN2 to further study the SLPP/RSPP transmission of the SL positioning configuration depending on the type of configuration node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, LMF, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

## Resource Allocation Mode 1 and Mode 2

It should be possible to extend the current Mode 1 and Mode 2 resource allocation modes in SL to configure SL PRS in all coverage scenarios, i.e., in-coverage, partial coverage and out-of-coverage as already agreed by RAN1.

RAN1#110 [5]: Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging: - Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS - Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS

Mode 1 offers a centralized approach to configure SL PRS for in-coverage and partial coverage scenarios, while Mode 2 enables SL PRS configuration by UEs in out-of-coverage scenarios.

According to this resource allocation mode, the gNB may configure the SL PRS resources in a centralised manner depending on the configured positioning technique. For techniques requiring relative position estimation amongst multiple pair of UEs (e.g., between an anchor node UE and target-UE), gNB-based resource coordination should be sufficient. The gNB may also coordinate SL PRS for multiple anchor node UEs when performing triangulation using timing-based methods such as SL-TDoA or angular-based position estimation, for the purposes of absolute location estimation.

In addition, depending on the type of location request the gNB may configure one of the following:

- Dynamic grants for one shot SL positioning estimates (based on one shot SL PRS transmissions) (e.g., in the case of immediately triggered requests)

- Configured grant schemes in the case that periodic location requests are required, e.g., Type 1 and 2 CG grants (based on periodic SL PRS transmissions).

Proposal 11: RAN2 to further study the impacts of Mode 1 coordination & signalling of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE) in line with RAN1’s agreement.

Mode 2 is beneficial for scenarios not requiring network infrastructure, where the Initiator UE (UE starting/initiating the SL positioning session) may autonomously select and reserve resource for the purposes of SL positioning. This relies on a pre-configuration of resource pools within a UE. Given the distributed nature of this scheme, Mode 2 can ensure stability of SL PRS for a given period by selecting and reserving resources for SL PRS transmission. However, this may depend on the type of SL PRS configuration (e.g., bandwidth, number of symbols, etc.).

The resource selection triggers may be different in the case of SL PRS when compared to the existing schemes for SL data (TB) transmission, where the selection is dependent on the generation of a new TB or a pre-emption mechanism depending on the SL traffic priority. Similarly, in the case of SL Positioning, the QoS may also have an impact on the resource selection mechanism in Mode 2. The sensing window should also be able to accommodate a SL PRS transmission based on the accuracy requirements of the SL positioning service request.

Proposal 12: RAN2 to further study Mode 2 coordination of SL PRS resources in line with RAN1’s agreement for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Positioning (e.g., one or more anchor UEs) over sidelink.

# SL Measurement and Reporting

SL positioning measurements can be performed by the target-UE in a similar fashion to the current Uu positioning framework. The target-UE is usually configured separately to report certain measurements based on the positioning method and can also be extended to the SL positioning framework. In the context of relative positioning, it can be expected that the SL positioning measurements may be performed either by the Initiating or Responding UEs, depending on the type of configured positioning methods, e.g., the two-way SL-RTT requires that both the Initiator and Responder UE perform the timestamp measurements for enhanced accuracy.

It can be noted that depending on the scenario, one or multiple entities may have different responsibilities in terms of transmitting the SL positioning configuration, performing measurements, reporting (if required, e.g., UE-based methods would require optional reporting) and finally the position calculation. It can be further observed that the measurement node/entity may receive the SL-PRS transmissions and then performs the associated SL positioning measurements.

Proposal 13: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based positioning), e.g., RSU/Anchor-UE, Target-UE.

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE.

NOTE: This has a dependency on the SL Positioning architecture as well as positioning modes (e.g., UE-assisted, UE-based).

The SL Positioning framework should also be capable of supporting different types of reporting of both SL positioning measurements and absolute/relative location estimate. Table 3 shows the various reporting types depending on the reporting configuration.

Table 3: Types of SL Positioning Reporting

| SL Positioning Reporting Types | Overview |
| --- | --- |
| One-shot | - The UE/device performing measurements may report to the positioning calculation entity in a one-shot manner to support immediate reporting. - The UE (positioning calculation) entity may report the absolute/relative location estimate to the requesting node/network entity, e.g., UE, LMF, etc. |
| Triggered | - The UE/device performing measurements may report to the positioning calculation entity based on defined event-based criteria, e.g., based on a location change/cell change. - The UE/device performing positioning calculation may report the location estimate to the requesting entity upon trigger of a configured event. |
| Periodic | - The UE/device may provide periodic positioning measurement or location estimate reports with the same time interval between two consecutive reports. |

Proposal 14: Support different SL Positioning reporting types including one-shot, triggered and periodic reports.

RAN1#110 has made an agreement to support measurement quality metrics. It is also important to assess the quality of the SL PRS measurements, which were performed, e.g., in the form of confidence intervals relating to a specific measurement, predefined margins of error, or other range reliability metrics. This can be applied to timing, angle, distance/ranging, direction/orientation estimates. This serves as additional assistance information for the positioning calculation entity.

Proposal 15: From RAN2 perspective, measurement quality metric reporting should also be supported to assess the quality of SL positioning measurements.

# Conclusion

The following observations regarding potential SL positioning solutions have been noted as part of the discussion:

Observation 1: For UEs wishing to only perform ranging in-coverage and without LPP capabilities, LMF involvement may be unnecessary.

Observation 2: A SL Positioning Server UE as a separate UE is only limited to partial and out-of-coverage scenarios and applicable to SL positioning scenarios involving limited/constrained capability UEs at least without result calculation capabilities.

Observation 3: Anchor nodes with known locations such as UEs and RSUs can enable certain SL positioning techniques and in other cases improve positioning performance.

Observation 4: For Uu positioning, the LMF coordinated the transmission configuration of PRS/SRS, while for in-coverage SL UEs, the gNB is the key configuration entity.

The proposals relating to the SL Positioning framework are summarized as follows:

Proposal 1: RAN2 to further study the location of SLPP/RSPP layer based on the required SL Positioning functionality.

Proposal 2: RAN2 to further study the support for groupcast and broadcast in order to assess the merits between PC5-S and PC5-U in terms of the transport of SLPP/RSPP messages.

Proposal 3: RAN2 to support discovery procedures for at least anchor UE(s) and target-UE. FFS the AS layer impacts of discovery procedures.

Proposal 4: RAN2 to further study whether existing/new SL ID management needs to be supported for SL positioning.

Proposal 5: RAN2 to support both LMF-dependent (e.g., UEs supporting both LPP and SLPP/RSPP) for in-coverage scenarios and LMF-independent (e.g., UEs only supporting SLPP/RSPP) SL positioning architectures for all coverage scenarios including in-coverage, partial coverage and out-of-coverage.

Proposal 6: RAN2 to consider support of “SL Positioning Server UE” as a functionality which is part of an Anchor UE or Target-UE.

Proposal 7: Down-prioritize the support for a separate SL Positioning Server UE from RAN2 perspective since it only serves very few limited scenarios and increases the complexity of SL positioning procedures, e.g., additional discovery and selection procedures.

Proposal 8: RAN2 to support the signalling design for at least SL-RTT type methods, SL-TDoA, SL-AoA.

Proposal 9: RAN2 is suggested to investigate the procedures related to anchor UE(s) including triggering, (re)selection, and configuration when performing sidelink positioning procedures. The same procedures may be applicable to an Assistant UE (if need for such a UE is determined).

Proposal 10: RAN2 to further study the SLPP/RSPP transmission of the SL positioning configuration depending on the type of configuration node and SL positioning mode (e.g., UE-based or UE-assisted), e.g., gNB, LMF, RSU/Anchor-UE, Target-UE by considering at least:

- UE-specific (via dedicated signalling) or common (via broadcast/groupcast signalling, e.g., SIB) SL positioning configurations.

- UE assistance information to assist the configuration node, e.g., gNB, LMF, UE in supporting SL positioning procedures.

Proposal 11: RAN2 to further study the impacts of Mode 1 coordination & signalling of SL PRS resources for one or more UEs participating in a SL positioning session (e.g., one or more anchor UEs and a target-UE) in line with RAN1’s agreement.

Proposal 12: RAN2 to further study Mode 2 coordination of SL PRS resources in line with RAN1’s agreement for one or more UEs participating in a SL positioning session e.g., one or more anchor UEs and a target-UE) including at least:

- The use of pre-configured resources in out-of-coverage scenarios.

- Initiator UE (e.g., target UE) indicates the SL-PRS resource to other UEs involved in SL Positioning (e.g., one or more anchor UEs) over sidelink.

Proposal 13: RAN2 to consider the supported signalling and procedures depending on which node/entity:

- Performs the SL positioning measurements and associated reporting (e.g., if needed for UE-based positioning), e.g., RSU/Anchor-UE, Target-UE.

- Performs the SL positioning calculation e.g., gNB, LMF, RSU/Anchor-UE, Target-UE.

NOTE: This has a dependency on the SL Positioning architecture as well as positioning modes (e.g., UE-assisted, UE-based).

Proposal 14: Support different SL Positioning reporting types including one-shot, triggered and periodic reports.

Proposal 15: From RAN2 perspective, measurement quality metric reporting should also be supported to assess the quality of SL positioning measurements.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-e, Dec. 2021.

- R2-2208704, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), RAN2#119-e, Aug. 2022.

- R2-2210804, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), RAN2#119-bis-e, Oct. 2022.

- R2-2211139, “LS on RAN dependency for Ranging/Sidelink Positioning”, RAN2#120, Nov. 2022

- R1-2208147, “Session notes for 9.5 (Study on expanded and improved NR positioning)”, Ad-Hoc Chair (Huawei), RAN1#110, Aug. 2022.

- 3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17), V2.0.0, September 2021.

- R1-2205574, “Session notes for 9.5 (Study on expanded and improved NR positioning)”, Ad-Hoc Chair (Huawei), RAN1#109-e, May 2022.