3GPP TSG-RAN WG2 Meeting #120	R2-2212109

Toulouse, France, 14–18. November 2022

Agenda item:	8.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of session-based and session-less sidelink positioning

WID/SID:	FS_NR_pos_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This document addresses protocol aspects of sidelink positioning that are within the scope of the Rel-18 sidelink positioning objective [1]. In particular, we discuss how to support session-based and session-less sidelink positioning and present our views.

# 2	Session Definition

In view of past contributions and the subsequent discussion on session-based and session-less sidelink positioning, we observe the need to clarify the terminology.

Proposal 1: RAN2 to agree on the definition of session-based and session-less positioning.

In our understanding, a session is a time-delimited two-way link enabling interactive expression and information exchange between two or more communication devices.

In other words, a session is present if a given pair of session members is engaged in bi-directional interaction and commits to a pre-defined behavior, determined by the employed communication protocol. Bi-directional exchange includes request-response communication model but an established communication session may involve more than one message in each direction.

A session is typically stateful, meaning that at least one of the communicating parties needs to hold current state information and save information about the session history to be able to communicate, as opposed to stateless communication, where the communication consists of independent requests with responses. For example, timers and counters are part of state-full sessions.

Proposal 2: RAN2 to confirm that a positioning session is characterized by a time-limited two-way link enabling interactive expression and information exchange between two or more communication devices, typically in presence of state (ie, information about session history). FFS other characteristics.

In the positioning context, the following examples fall under the above definition of session-based positioning:

- target UE requests a specific roadside unit (RSU) to (a) measure its PRS, and (b) report the measurement results back to it within certain time frame

- target UE requests another UE to be localized (eg, based on PRS measurement results provided, or based on a new measurement campaign orchestrated by the requested UE)

- presence of timers and dependency of response content on (request) input

On the other hand, the following behavior does NOT satisfy the definition of a session-based positioning and will be therefore referred to as session-less positioning:

- roadside RSUs unilaterally broadcast their PRS (possibly including assistance data) and target UE measures them for OTDOA purposes having obtained assistance data via cell SIB broadcast or pre-configuration

- target UE collects all PRS measurements and computes its own location without actively engaging in any data exchange with near-by (anchor) UEs

From these examples, one could argue that session-less positioning could be characterized by independent unidirectional transmissions as a complement to bi-directional interactivity. However, if such a criterion is to be used, the cast type of such transmissions must be reflected as well. Groupcast reception is configured based on the concept of a pre-defined group, ie such group must be identified and configured prior to any groupcast transmission. Similarly, sidelink unicast relies on PC5 RRC session establishment.

Proposal 3: Session-less positioning is based on independent unidirectional broadcast transmissions. FFS other characteristics of session-less positioning including other cast types.

Clearly, multiple UEs can engage in session-based positioning to which end a multitude of positioning protocols can be used between any given pair of network nodes. For example, when localizing a given target UE, LPP-based (ie session-based) interaction between the target UE and the LMF in combination with (S)LPP-communications between the LMF and the engaged anchor UEs can still be seen as part of a single positioning session facilitated by the LMF. In other words, a multi-user session does not necessarily need to involve a two-way link between every pair of participating UEs (full-mesh communications).

Observation 1: Sessions can be centrally coordinated based on server-client relationship, or be of peer-to-peer distributed nature whereby multitude of communication protocols can be used.

It is further observed that the choice of positioning method has implications on whether session-based or session-less positioning is to be used. For example, if a target UE only passively measures pre-activated PRS from neighboring anchors and uses these measurements for OTDOA purposes. The target UE can perform all actions unilaterally so there is theoretically no need to establish a sidelink positioning session. The anchor UEs are not actively engaged with the given target UE.

However, if the target UE is to be localized based on RTT methods, the anchors must actively collaborate with the target UE and employ a predefined communications protocol to this end. Clearly, the criterion for session usage is satisfied in this case. One could even extend this UE interactivity and mutual dependence argument to mandatory usage of session-based positioning in case of RTT methods to ensure seamless functionality.

Observation 2: Some positioning methods or their configurations allow avoiding session establishment and eliminate overhead.

The presence of LMF as positioning decision-maker can be seen as another satisfactory condition for the adoption of session-based positioning.

Proposal 4: Study the scenarios where session-based and session-less SL positioning are applicable.

# 3	Performance and Design Issues

A session has multiple components:

- setup – this primarily includes directives or handshakes regarding the identification of session member / capability / protocol / purpose,

- activation – each session must be started and terminated as well as its resources allocated and activated,

- execution – in this phase, data is exchanged and session state managed,

- error-handling – key characteristics of the session responsible for retransmission / recovery / abort measures

Observation 3: A session consists of

setup (handshakes for member/capability/protocol/purpose identification)

activation (start/stop boundaries, resource activation)

execution (data exchange, state management)

error-handling (retransmissions, recovery/abort measures)

Clearly, broadcast and groupcast can be applied to each session part and help minimizing the overhead associated with 1-to-1 interactions. To this end, one can use also implicit standardization convention or explicit pre-configuration, including self-configuration.

Proposal 5: Study methods for reducing signaling overhead in reliable session-based positioning, including at least group/broadcast, pre-configuration including self-configuration, pre-activation including self-activation.

While broadcast is generally a useful tool for overhead reduction, its unconstraint ad hoc usage can lead to channel congestion and interference, and consequently to poorer performance. To avoid this risk, broadcast must be employed as tool for overhead reduction while paying attention to the minimization of channel congestion risk.

Proposal 6: Study method for minimizing impact on channel congestion in session-less positioning.

Whenever SLPP and LPP are used simultaneously, there may be need to use session-based sidelink positioning as LPP is a session-based protocol. It is therefore desirable to understand the implications of introducing a session-less SLPP.

Proposal 7: Study co-existence between session-based & session-less sidelink positioning.

# 4	Conclusion

This document has made the following observations and proposals:

Proposal 1: RAN2 to agree on the definition of session-based and session-less positioning.

Proposal 2: RAN2 to confirm that a positioning session is characterized by a time-limited two-way link enabling interactive expression and information exchange between two or more communication devices, typically in presence of state (ie, information about session history). FFS other characteristics.

Proposal 3: Session-less positioning is based on independent unidirectional broadcast transmissions. FFS other characteristics of session-less positioning including other cast types.

Observation 1: Sessions can be centrally coordinated based on server-client relationship, or be of peer-to-peer distributed nature whereby multitude of communication protocols can be used.

Observation 2: Some positioning methods or their configurations allow avoiding session establishment and eliminate overhead.

Proposal 4: Study the scenarios where session-based and session-less SL positioning are applicable.

Observation 3: A session consists of

setup (handshakes for member/capability/protocol/purpose identification)

activation (start/stop boundaries, resource activation)

execution (data exchange, state management)

error-handling (retransmissions, recovery/abort measures)

Proposal 5: Study methods for reducing signaling overhead in reliable session-based positioning, including at least group/broadcast, pre-configuration including self-configuration, pre-activation including self-activation.

Proposal 6: Study method for minimizing impact on channel congestion in session-less positioning.

Proposal 7: Study co-existence between session-based & session-less sidelink positioning.

# References

[1]  "RP-221814, Revised SID on Study on expanded and improved NR positioning".