3GPP TSG-RAN WG2 Meeting #120	R2-2212124

Toulouse, France, 14-18 November 2022

Source: 			Qualcomm Incorporated

Title: 	Maximum aggregated bandwidth for FBG5

Document for: 	Decision

Agenda Item: 	6.24.1.1

# Introduction

RAN2 in their #119bis meeting discussed the UE capability signalling for maximum aggregated bandwidth for FBG5, based on RAN4 LS [1]. In this document we further try to analyse the signalling overhead reduction gain of the new signalling mechanism.

NOTE: This document builds on top of the understanding described in [2]. In particular it is assumed that the existing UE capability signalling mechanism supports signalling of “explicit” fallback combinations by means of Feature Sets for a single BandCombination entry. Such signalling is considered as the baseline for signalling overhead analysis.

# Discussion

  - Signaling of supported maximum aggregated BW for FBG5

RAN4’s table for FBG5 is reproduced below for reference.

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| (unchanged legacy FBG2,3,4) |  |  |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3, 4 and 5 are 400 MHz, 200 MHz, 100 MHz, 100 MHz and 200 MHz respectively except for CA bandwidth class A. For CA bandwidth classes of fallback group 5, requirements apply for non-interlaced 100 MHz and 200 MHz channel bandwidths (each CA bandwidth class consisting of up to two contiguous sub-blocks each with component carriers of a single channel bandwidth). NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. NOTE 3: In this release of the specification, the minimum requirements for intra-band contiguous CA configurations apply for aggregated channel bandwidths up to 1600 MHz (this note is not relevant for UE capability parsing by the network). |  |  |  |

RAN4 explains the intention of the new maximum aggregated bandwidth as follows.

- “RAN4 have also determined that some UEs have enhanced aggregated bandwidth capability for fallback BW classes compared to the ‘dropping CCs’ interpretation of the BW class fallback rule. Specifically, some UEs have independent maximum limits on number of carriers and aggregated bandwidth.”

It is our understanding that the “dropping CCs” case corresponds to the fallback band combination as defined in TS38.306 (“implicit” fallback band combination in [2]), where the total aggregated CA bandwidth is naturally reduced by “dropping CCs”. The “enhanced” case RAN4 is referring to is the case where the total aggregated CA bandwidth for fallback band combinations is maintained as much as possible to the maximum aggregated bandwidth supported by the UE.

We try to show these two cases with the example where the UE supports bandwidth class R6 (600MHz ≤ BWChannel_CA ≤ 1200MHz, 6CCs), and the maximum aggregated bandwidth of 600MHz. With the existing UE capability signalling, the UE can signal the following superset band combination.

  - Max.100MHz BW x 6CCs (Aggregated bandwidth: 600MHz) >> Bandwidth class R6

Now with the “dropping CCs” case, the UE capability for the fallback band combinations can be indicated as follows without the UE explicitly signalling those capabilities.

  - Max.100MHz BW x 5CCs (Aggregated bandwidth: 500MHz) >> R5

  - Max.100MHz BW x 4CCs (Aggregated bandwidth: 400MHz) >> R4

  - Max.100MHz BW x 3CCs (Aggregated bandwidth: 300MHz) >> R3

  - Max.100MHz BW x 2CCs (Aggregated bandwidth: 200MHz) >> R2

For the “enhanced” case on the other hand, the UE can explicitly signal the fallback band combinations as follows using the Feature Sets. It should be noted that the fourth band combination corresponding to bandwidth class R2 is a fallback band combination of the third band combination corresponding to bandwidth class R3. Therefore, the UE shall not signal the fourth one explicitly, unless additional functionality, e.g. more MIMO layers, is supported.

  - Max.100MHz BW x 4CCs + Max.200MHz BW x 1CC (Aggregated bandwidth: 600MHz) >> R5

  - Max.100MHz BW x 2CCs + Max.200MHz BW x 2CCs (Aggregated bandwidth: 600MHz) >> R4

  - Max.200MHz BW x 3CCs (Aggregated bandwidth: 600MHz) >> R3

  - Max.200MHz BW x 2CCs (Aggregated bandwidth: 400MHz) >> R2

The key idea of RAN4 solution is reduce the number of Feature Sets that are signalled compared to the “enhanced” case above. When other UE capabilities except for CC bandwidth within the Feature Sets are the same for all CCs, it is possible to signal the UE capability with a single Feature Set by additionally indicating the supported maximum aggregated bandwidth, as follows.

  - Max.200MHz BW x 6CCs / Maximum aggregated bandwidth=600MHz

The network can configure any CC bandwidth as long as it satisfies both the maximum CC bandwidth and the maximum aggregated bandwidth limitations as signalled by the UE. It can be seen that the new signalling covers all fallback band combinations of the “enhanced” case (bandwidth classes R6, R5, R4, R2). Note that this signalling is not backward compatible. The legacy network would misunderstand that the UE indeed supports Max.200MHz BW x 6CCs and aggregated bandwidth of 1200MHz. It should be applicable to only new bandwidth classes of FBG5 and future bandwidth classes.

  - Signaling overhead reduction

We try to explain the expected signalling reduction gain in terms of number of UE capability parameter entries, using the examples described in the previous section.

Table-1: Number of UE capability parameter entries

|  | FeatureSetDownlink / FeatureSetUplink | FeatureSetDownlinkPerCC / FeatureSetUplinkPerCC | FeatureSetDownlinkId / FeatureSetUplinkId in FeatureSetCombination |
| --- | --- | --- | --- |
| “Enhanced” case with exiting UE capability signalling | 4 | 2 (100MHz BW, 200MHz BW) | 4 |
| New signalling with maximum aggregated BW | 1 | 1 (200MHz BW) | 1 |

It should be noted that here we are looking at the signalling overhead reduction for one intra-band contiguous CA band combination which consists of 6CCs. Overall reduction gain is higher for band combinations with higher number of band entries and CCs. We should also not forget the fact that today’s UEs supports very large number of CA band combinations, and the size of UE Capability Information message can easily exceed the maximum PDCP SDU size which requires the UE to optimize the content of band combination list and associated feature set combinations, e.g. choose not to report certain band combinations and/or feature sets.

# Conclusion

In this document we looked at the signalling overhead reduction gain of the new UE capability signalling mechanism described in RAN4 LS [1]. Based on the analysis, it is proposed to introduce the new UE capability signalling for “maximum aggregated bandwidth” applicable to the new bandwidth classes of FBG5.

Proposal:	Introduce the new UE capability signalling for “maximum aggregated bandwidth” applicable to the new bandwidth classes of FBG5.

CRs are submitted in [3] and [4].

# References

[1]		R2-2209347		LS on new contiguous BW classes for legacy networks (R4-2215160)		RAN4

[2]		R2-2212123		UE capability signalling for fallback band combinations and fallback bandwidth classes		Qualcomm Incorporated

[3]		R2-2212125		CR to 38.331 on Introduction of maximum aggregated BW for FBG5		Qualcomm Incorporated

[4]		R2-2212126		CR to 38.306 on Introduction of maximum aggregated BW for FBG5		Qualcomm Incorporated