3GPP TSG-RAN2 Meeting # 120	R2-2212132

# Toulouse, France, Nov 14th - Nov 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3674 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RLC mode reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#108 meeting ,RAN2 reached the below agreement on RLC mode reporting. 1: When the peer UE in RRC_CONNECTED receives an SLRB configuration with RLC AM/UM from the initiating UE via PC5 RRC and if the LCH has not been configured in the peer UE, it reports at least RLC mode by the initiating UE via PC5 RRC to its gNB. PC5 QoS profile is optional to be reported. But in the current RRC spec, it recorded the condition of triggering SUI for Rx UE is as below:“upon RLC mode information updated from the associated peer UE”. The current spec in TS38.331 is not consistent with the RAN2 agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the condition of triggering SUI for Rx UE from “upon RLC mode information updated from the associated peer UE” to “upon a new logical channel is configured in the UE upon receiving RRCReconfigurationSidelink from the associated peer UE” to align the RAN2 agreement. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality sidelink UE information for NR sidelink communication Inter-operability: If network implements the change but not the UE, there is no inter-operability issue. If the UE implements the change but not the network, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect agreement captured in the current RRC spec, and if both RLC mode and QoS profile are sent, the current spec will provide less information to gNB compared with the RAN2 agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

5.8.3.2	Initiation

A UE capable of NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation that is in RRC_CONNECTED may initiate the procedure to indicate it is (interested in) receiving or transmitting NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation in several cases including upon successful connection establishment or resuming, upon change of interest, upon changing QoS profiles, upon receiving UECapabilityInformationSidelink from the associated peer UE, upon a new logical channel is configured in the UE upon receiving RRCReconfigurationSidelink from the associated peer UE or upon change to a PCell providing SIB12 including sl-ConfigCommonNR. A UE capable of NR sidelink communication may initiate the procedure to request assignment of dedicated sidelink DRB configuration and transmission resources for NR sidelink communication transmission. A UE capable of NR sidelink communication may initiate the procedure to report to the network that a sidelink radio link failure or sidelink RRC reconfiguration failure has been declared. A UE capable of NR sidelink discovery may initiate the procedure to request assignment of dedicated resources for NR sidelink discovery transmission or NR sidelink discovery reception. A UE capable of U2N relay operation may initiate the procedure to report/update parameters for acting as U2N Relay UE or U2N Remote UE (including L2 Remote UE's source L2 ID).

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception, upon accepting the sidelink DRX configuration from the associated peer UE. A UE capable of NR sidelink communication that is in RRC_CONNECTED and is performing sidelink unicast transmission with resource allocation mode 1 may initiate the procedure to report the sidelink DRX assistance information or the sidelink DRX configuration reject information received from the associated peer UE, upon receiving either of them from the associated peer UE. A UE capable of NR sidelink communication that is performing sidelink groupcast transmission may initiate the procedure to report the sidelink DRX on/off indication for the associated Destination Layer-2 ID.

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the Destination Layer-2 ID and QoS profile associated with its interested services that sidelink DRX is applied, for NR sidelink groupcast or broadcast reception.

End of Change